ASAM SOVD

Service-Oriented Vehicle Diagnostics

API Specification

Version 1.0.0

Date: 2022-06-30

Base Standard

© by ASAM e.V., 2022

Disclaimer

This document is the copyrighted property of ASAM e.V.

Any use is limited to the scope described in the license terms. The license terms can be viewed at www.asam.net/license

Table of Contents

Foreword	8

- Introduction	9

  - Overview	9

  - Motivation	9

- Relations to Other Standards	11

  - References to Other Standards	11

- SOVD Architecture	12

  - Usage Scenarios	12

    - Remote	12

    - Proximity	12

    - In-Vehicle	13

- SOVD API General Aspects	14

  - HTTP REST Based Approach	14

  - Entities	15

    - Entity Types	15

    - Multi-SOVD Server Environments	16

      - Public SOVD Server	17

      - Private SOVD Server	17

    - Entity Hierarchy and Relations	18

    - Entity Identifier and Path	18

  - Resources	19

    - Resource Collections	19

    - Standardized Resources	20

  - SOVD API Versioning	21

  - Internationalization and Localization	22

    - Internationalization	22

    - Unit Definitions	23

    - Other Localization Topics (Numbers and Dates)	24

  - Response Status Codes	24

    - Generic Definition of Status Codes	24

    - Definition of Error Details	25

    - SOVD Error Codes for Common Cases	26

  - Common Primitive Data Types	27

  - Definition of JSON Schema and Objects for Vehicle Responses	28

  - Discovery of the SOVD Server for the Proximity Use Case	30

    - General Concept	30

    - Network Connection	30

    - Address an SOVD Server Using mDNS	30

    - Discovery of the SOVD Server Using DNS-SD	30

    - Example	31

  - Discovery of the SOVD Server for the Remote Use Case	31

- Capability Description	32

  - Introduction	32

  - Capability Description Structure	32

    - Info Object	32

    - Path Item Object	33

    - Operation Object	33

    - Schema Object	34

    - Header Object	34

      - Authorization Request Header	34

      - Location Response Header	35

      - X-SOVD-Mode Request header	35

    - Parameter Object	35

      - include-schema	35

  - Online and Offline Capability Descriptions	35

    - Introduction	35

    - Offline Capability Description	36

    - Online Capability Description	36

  - Variant Handling in Offline Capability Description	36

    - Introduction	36

    - Extension: x-sovd-applicability	37

    - Usage of x-sovd-applicability by the SOVD Client	37

- SOVD REST API	39

  - Introduction	39

  - Definition of Conventions for API Methods	39

    - Query Parameters	39

    - Request Body	39

  - Timeout Definitions	39

  - API Methods for Access to Capability Description Content	40

    - Introduction	40

    - Query an Online Capability Description	40

  - API Methods for Discovering of Entities and Resources	41

    - Introduction	41

    - Query of Available Entities Under This SOVD Server	41

      - Discover Contained Entities	41

      - Query Sub-Entities of an Entity	44

      - Query Related Entities of an Entity	45

    - Query Entity Capabilities	46

  - API Methods for Fault Handling	49

    - Introduction	49

    - Read Faults from an Entity	49

    - Read Details for a Fault	53

    - Delete All Faults of an Entity	56

    - Delete Single Fault of an Entity	57

  - API Methods for Data Resource Read / Write Access	58

    - Introduction	58

    - Query of Data Resource Categories and Groups	59

      - Retrieve Categories Supported by a Data Resource

Collection	59

      - Retrieve Groups Supported by a Data Resource Collection	60

    - Retrieve List of All Data Provided by the Entity	61

    - Read Single Data Value from an Entity	63

    - Read Multiple Data Values from an Entity	66

      - Retrieve List of All Data-Lists Provided by the Entity	66

      - Creating a Data List for Reading Multiple Data Values at

Once from an Entity	68

      - Read Multiple Data Values at Once from an Entity Using a

Data List	70

      - Delete an Existing Data List	72

    - Write a Data Value to an Entity	73

  - API Methods for Configuration	74

    - Introduction	74

    - Retrieve List of All Configurations Provided by the Entity	74

    - Read Configuration	77

      - Introduction	77

      - Read Configuration as Bulk Data	77

      - Read Configuration as Parameters	78

    - Write Configuration	80

      - Introduction	80

      - Write Configuration as Bulk Data	81

      - Write Configuration as Parameters	82

  - API Methods for Control of Operations	83

    - Introduction	83

    - Ensuring Proximity Diagnostics	84

    - Retrieve List of All Available Operations from an Entity	84

    - Get Details of a Single Operation	86

    - Start Execution of an Operation	89

    - Get Executions of an Operation	92

    - Get the Status of an Operation Execution	93

    - Stop the Execution of an Operation	95

    - Support for Execute / Freeze / Reset and OEM-Specific Capabilities	96

  - API Methods for Support of Target Modes	98

    - Introduction	98

    - Retrieve List of All Supported Modes of an Entity	99

    - Get Details of a Single Mode of an Entity	100

    - Explicit Control of Entity States via Their Defined Modes	102

    - Hint-Based Control of Entity States via Their Defined Modes	104

  - API Methods for Locking	104

    - Introduction	104

    - Acquire a Lock on an Entity	105

    - Get All Acquired Locks of an Entity	107

    - Get a Single Active Lock of an Entity	108

    - Modify the Expiration Time of an Acquired Lock on an Entity	109

    - Release an Acquired Lock on an Entity	111

  - API Methods for Software Update	112

    - Introduction	112

      - Update Origins	112

      - Stepwise Software Update	113

      - Automated Software Update	113

      - Autonomous Update Package	113

      - Example	113

    - Retrieve List of All Updates	114

    - Get Details of Update	116

    - Automated Installation of an Update	119

    - Prepare Installation of an Update	120

    - Execute Installation of an Update	122

    - Get Status of an Update	123

    - Delete Update Package from an SOVD Server (Optional)	126

    - Register an Update at the SOVD Server	127

  - API Methods for Handling of Bulk Data	128

    - Introduction	128

      - Resource Organization	129

      - Encoding of Bulk Data	129

    - Retrieve List of all Bulk Data Categories	130

    - Read Bulk Data Meta Data	131

    - Download and Upload Bulk Data	133

      - Download Bulk Data	133

      - Upload Bulk Data	135

    - Delete Bulk Data	137

      - Delete All Bulk Data Defined by Category	137

      - Delete Specific Bulk Data Resource	139

  - API Methods for Logging	140

    - Introduction	140

    - Retrieve List of All log Information	140

    - Configure SOVD Logging	144

    - Retrieve the Current SOVD Logging Configuration	145

    - Reset SOVD Logging Configuration to Default	147

  - Authentication of SOVD Clients (Informative)	148

    - Introduction	148

      - Online Authentication and Authorization	149

      - Offline Authentication and Authorization	150

      - Implementation Hints	151

    - Secure Connection using TLS	151

    - Verifying SOVD Client Credentials and Requesting a Token at the Backend	152

    - Verifying SOVD Client Credentials at the Vehicle	152

    - Requesting a Token	153

    - Request Header for Access-Restricted Resources	154

    - Validating a Token	155

- Classic Diagnostic Adapter	156

  - Introduction	156

  - Access to UDS Based Entities	156

  - Specific Mapping of UDS Services to SOVD Modes	157

    - Mapping of SessionControl ($10) Subfunctions to SOVD Modes	157

    - Mapping of SecurityAccess ($27)	157

    - Mapping of $29 Authentication to SOVD Modes (Informative)	157

    - Mapping of Communication Control ($28) Subfunctions to SOVD

Modes	158

    - Mapping of Control DTC Settings ($85) Subfunctions to SOVD Modes158

  - Mapping of UDS Services to Data Resources	158

  - Mapping of UDS Services to Fault Resources	158

  - Mapping of UDS Services to Operation Resources	158

    - Mapping of Routine and IOControl Services to Operation Resources 158

    - Mapping of EcuReset Service ($11) to Operation Resources	159

- Terms and Definitions	160

- Symbols and Abbreviated Terms	162

- Bibliography	164

Appendix: A.	SOVD API Common Example	166

  - Introduction	166

  - Entity Hierarchy	166

  - Entities and Their Resources	167

    - App AdvancedLaneKeeping	167

    - App WindowControl	168

    - App Navi	168

    - Component Camera	169

    - Component PowerSteering	169

  - Complex Sequence Examples	170

    - Creation and Usage of a Temporary Data-List Resource	170

    - SteeringAngleControl Operation incl. Modes and Locks	170

  - Function VehicleHealth	173

    - Introduction	173

    - Retrieval of VehicleHealth	174

Appendix: B.	Specific Variant Applicability	177

  - Introduction	177

  - Example	177

Figure Directory	179

Table Directory	180

Foreword

The SOVD Standard provides an API for diagnosing software-based vehicles. It provides uniform access to the diagnostic content of HPCs and their related applications as well as classical ECUs.

SOVD follows an HTTP REST based approach. Thereby no automotive specific stack is needed on client side. Due to the flexible type-system used, it provides access to a broad variety of content required for HPC diagnostics.

SOVD supports the following scope:

- Capability discovery

- Reading and deletion of fault entries

- Reading and writing of data resources

- Reading and writing of configurations

- Control of operations (including control of entity states via defined modes and locking of entities)

- Software update

- Handling of bulk data

- Logging data access

For the convenience of the user, a machine-readable OpenAPI definition of the methods is published alongside this document.

- Introduction

  - Overview

The ASAM SOVD standard defines an API which standardizes the methods for diagnosing HPCs and classical ECUs, the retrieval of the diagnostic capabilities, and the discovery of the SOVD methods in a vehicle. The SOVD API provides a unified access to classic ECUs and HPCs. This access can be performed remotely (e.g., backend or cloud), in the workshop (e.g., workshop tester), or in the vehicle (e.g., onboard tester).

The SOVD API leverages existing technologies as described in chapter 2.1. The SOVD API provides the following functions:

- Clients can access the faults, including reading the fault entries, reading environment data, and deleting fault entries.

- Measurements and identifications from all entities in the vehicle can be read. In addition, identifications may be written as well.

- SOVD supports the execution of routines, I/O controls, and software functions. Their execution can only be performed in certain modes or states. Thus, an SOVD client can set the Component into a specific mode.

- The configuration of a vehicle (e.g., equipment, country, customer demand, variant coding etc.) can be read and written using the SOVD API.

- SOVD encapsulates the OEM specific software update strategy (incl. FOTA) through a generic API.

- Access to logging information of an HPC.

With these features SOVD can cover all areas of the vehicle life cycle: Engineering (Development), Manufacturing (Production), After Sales (Maintenance and Repair), and Vehicle operation (Use).

There are several aspects which are not covered by the SOVD standard, as they are specific to the implementation of an SOVD server, for example:

- Prevention and quick reaction to attack vectors like denial of service, zero-day exploits of vulnerabilities, …

- Recognition of security incidents

- Maintaining the operational safety based on data monitoring

- Management of security incidents

- Load balancing of concurrent requests

  - Motivation

Vehicle electronics topologies are changing rapidly. New functions such as autonomous driving require application-level software of high complexity to be executed within the vehicle.

The introduction of HPCs in vehicles is associated with changes in the classical E/E vehicle architectures. While classical distributed embedded ECU architectures were used previously, domain- or zone-based E/E architectures dominate today.

On HPCs, there will be an agile exchange of applications to enable new functionalities for the user. Since the HPCs can be compared to classic computers running numerous applications,

this means that diagnostics are no longer limited to classic fault codes (DTCs with environment data). An analysis of the running programs is required in real operation. The diagnostic of these programs is comparable to the error and runtime analysis of software running on PCs. That means, it has the memory usage, processor load and number of active services that is required to be recorded, and therefore also log and trace files might be needed.

This shifts the focus from ECU diagnostics in today's vehicles from checking hardware to checking the software functionality of applications, which corresponds to a paradigm shift.

All these topics result in new challenges regarding the management of the vehicle life cycle, which includes the diagnostic, (re-)configuration and re-programming of vehicle electronics.

New operating systems also no longer follow the classic rules for embedded systems, therefore, the diagnostic changes and adapts due to several factors, these include:

- Faster release and update cycles

- Increased requirements such as data protection or security

- State-of-the-Art diagnostic API using current information technologies.

The SOVD standard takes these points into account and provides a diagnostic API based on current IT technologies for the purpose of diagnosing vehicles with modern E/E architectures.

- Relations to Other Standards

  - References to Other Standards

The SOVD API uses the following standards and technologies:

- The API follows the REST principles [16] and uses JSON [28] for encoding the transmitted data.

- SOVD is designed to work with HTTP/1.1 [23] but for achieving the best communication performance HTTP/2 [22] is recommended. No HTTP/2 specific features are used.

- The SOVD API utilizes the OpenAPI [10] specification to define the API as well as the diagnostic capabilities of the vehicle.

- The authentication and authorization of clients builds upon OpenID Connect and OAuth 2.0 [12], [13], [14], but an OEM may use other authentication mechanism like certificates if required.

- ASAM data types [35] are mapped to JSON types.

- SOVD Architecture

  - Usage Scenarios

As wireless connectivity technologies enter the automotive marketplace there will be new methods of accessing and using vehicle diagnostics. Historically, vehicle diagnostics have been focused on proximity-based use cases, where a technician or worker is located close to the vehicle and connects an external device to perform diagnostic tasks. Wi-Fi and mobile broadband network access (4G, 5G) allow for remote and OTA use cases such as remote diagnostics, configuration, and software updates. With the introduction of HPCs into new vehicle architectures this also gives the ability to perform in-vehicle diagnostic tasks without any permanent external connectivity.

SOVD offers solutions for in-vehicle, proximity and remote diagnostics use cases. This standard intends to meet the requirements for those use cases, which are not covered by current diagnostic stacks, based on UDS (ISO 14229 [1]), D-PDU API (ISO 22900-2 [18]), ODX (also known as ISO 22901-1 [9] or ASAM MCD-2D) and ASAM MCD-3D (also known as ISO 22900-3 [29]).

    - Remote

For remote use cases it is assumed that the operator is not close to the vehicle but accesses the vehicle remotely via (mobile) broadband network (over-the-air). Remote use cases do not necessarily require a human end-user but can also include cloud-based services such as fleet management, data collection and analysis that are only indirectly feeding information to a human user. Remote use cases may rely on onboard use cases, e.g., to buffer data in cases where the network is not available.

Remote diagnostics use cases include but are not limited to the following:

- Information retrieval (e.g., fuel level, battery health check)

- Checking operational status

- Configuration

- Emissions check

- Workshop repair preparation

- Remote troubleshooting

- Remote activation of vehicle functions

- Software update

- Firmware update

- Fleet management

- Configuration of the software

    - Proximity

For proximity use cases it is assumed a technician or worker is close to the vehicle and has a test device connected to a particular vehicle (by cable or wireless).

Proximity diagnostics use cases include but are not limited to the following:

- Troubleshooting

- Performing actuations or stimulations for functional checks

- Periodical technical inspection

- Emissions check

- Checking operational status

- Software update

- Firmware update

- Configuration of the software

    - In-Vehicle

For in-vehicle use cases it is assumed that these use cases can run autonomously within the vehicle without a permanent connection to a remote server or a proximity tester. However, results of onboard use cases may also be accessed by proximity or remote use cases.

In-vehicle diagnostics use cases include but are not limited to the following:

- In-vehicle monitors

- Preventive/Predictive Maintenance

- Fleet monitoring scenarios (e.g., periodically collecting vehicle status)

- SOVD API General Aspects

  - HTTP REST Based Approach

The SOVD API follows the REST principles [16]. This means that diagnostic content is provided in form of resources.

A resource is accessible via its specific resource path. In SOVD the resource path is composed of the path to the individual entity (see 4.2) and the standardized resources and resource collections provided for that entity (see 4.3).

The operations available for the individual diagnostic content are represented using HTTP methods. SOVD makes use of the following HTTP methods:

Table 1 HTTP methods used by SOVD

| HTTP method | Purpose |
| --- | --- |
| GET | Reading content from a resource |
| PUT | Update content of a resource (e.g., by writing a new value) |
| POST | Creation of new (temporary) resources |
| DELETE | Deletion of created resources, resetting content to default |

The following example shall briefly illustrate the REST based approach used by SOVD. Details on how the resource structure is built are given in the following chapters.

The example shows a software application in the vehicle providing functionality to control the windows, for example it offers information about the “RearWindows” positions.

Figure 1	WindowControl example

In terms of SOVD this is implemented by providing an SOVD entity “WindowControl” containing the leaf resource “RearWindows”, which can be read using a GET operation.

The path elements “apps” and “data” of the resource path in the following example will be explained in the upcoming chapters 4.2 and 4.3.

Example:

Request:

GET {base_uri}/apps/WindowControl/data/RearWindows HTTP/1.1

  - Entities

The SOVD API defines entities holding diagnostic content. To access diagnostic content, an SOVD client needs to specify the entity it wants to interact with. This subchapter covers the entity types, the hierarchy, and how to reference the entity in a method request.

    - Entity Types

The SOVD standard defines the entity types as shown in Figure 2. The entity types are defined flexibly enough to support various topologies and diagnostic strategies. SOVD entities hold the diagnostic content as resources. Resource collections available for an entity type are defined in 4.3.1.

Figure 2	Entity types overview

Table 2 describes the different entity types in more detail.

Table 2 Details of entity types

| EntityType | EntityCollectionType | Description |
| --- | --- | --- |
| SOVDServer |  | An SOVDServer represents the entry point of the API structure. Resources on that level are only related to the SOVD server itself and not aggregated from other entities. |
| Area | areas | An Area represents a logical view and can be used to describe various vehicle topologies, such as domain architectures (body control, Infotainment, powertrain), zone architectures (front, rear), or logical architectures (software defined vehicle). |
| Component | components | A Component is a representation of hardware or software. A Component can also represent a combination of both (e.g., ECU with basic software). Components representing hardware are characterized by the availability of a CPU and execute software. Examples are HPCs, ECUs, and Smart sensors. Software as part of Components is characterized as basic software required to execute Apps. Examples for basic software are operating systems, or Hypervisor. Only software Components or Components containing software parts shall link to Apps. |
| App | apps | An App represents an application executed on a Component, e.g., Advanced-Lane-Keeping or Window Control. |
| Function | functions | A Function represents a functional view and allows an OEM to define access to diagnostic information, which may be spread across several Components, e.g., Vehicle Identification. Note: This standard defines no further details for the handling of Functions. |

From an API perspective, there is no differentiation between hardware and software Components. However, an OEM may choose to do so and represent this within a component hierarchy.

    - Multi-SOVD Server Environments

This standard neither defines nor demand a specific architecture within a vehicle. It is intended to support any architecture defined by an OEM, including multiple SOVD servers in a single vehicle.

It is strongly recommended, that each entity in a vehicle is bound to and can only be accessed through a dedicated SOVD server (see 4.2.1).

      - Public SOVD Server

Each SOVD server can be directly accessible from the outside through connectivity measures (remote or proximity). It is within the responsibility of the OEM to provide identification of a Public SOVD server for remote SOVD clients. For proximity clients, the identification method as described in 4.9 shall be used.

Figure 3	Public SOVD server

      - Private SOVD Server

An SOVD server can also be restricted to be accessible from the in-vehicle network. Direct access to a Private SOVD server is then only possible for an On-Board SOVD client. Diagnostic Services as described in this document, which are provided by a Private SOVD server have to be forwarded by a Public SOVD server to an external SOVD client, in case these Diagnostic Services should be externally available.

Figure 4	Private SOVD server

    - Entity Hierarchy and Relations

The entity types described above can be used to build a hierarchy for the SOVD API. Figure 5 depicts the standardized entity hierarchy. The entry point into this structure is the SOVD server. OEMs are free to select the needed entity types. For simplicity of this figure, the {Entity-collection}s are not shown as separate classes but can be seen from the name of the respective association ends.

Figure 5	Entity hierarchy

An Area can have multiple subareas, which themselves are Areas and thus can have subareas. Areas on all levels can have links to Components. This information can be used by an SOVD client to present diagnostic information according to the topology.

A Component can have multiple subcomponents, which themselves are Components and thus can have subcomponents. Components on all levels can have links to Apps. This information can be used by an SOVD client to present all Apps running on a Component.

    - Entity Identifier and Path

The standard only defines the names of the entity collection resources and the entity types. The entity identifiers and names are specified by the vehicle manufacturer when the API is realized.

Every entity is accessible via its entity-path. The entity path is built from the element’s entity collection name and the entity identifier.

The path elements are separated by a forward-slash (‘/’). In case of a sub level entity all path elements are concatenated. For resource on entity level SOVDServer the entity collection name is omitted.

Example:

{base_uri}/components/DrivingComputer/subcomponents/Linux

  - Resources

This subchapter describes the standardized resource types, how they are organized into collections and which are available for a specific entity type.

An OEM may specify additional resource types, collections, and top-level-resources for an entity. To avoid naming conflicts with resources, collections thereof, and resource types standardized in a later version of the SOVD API, the OEM shall use the prefix “x-<oem>-“.

Constraints:

- A resource shall only be accessible via a single path.

- Access shall only be provided to an entire resource. It shall not be possible to access parts of a resource by means of filters, e.g., a single parameter of a data or configuration resource.

- A resource shall only be available on the entity that represents the diagnostic content. An entity shall not aggregate resources which are held by e.g., subcomponents.

    - Resource Collections

Each entity provides its diagnostic content in the form of resources at the SOVD API. These resources are accessible via their respective resource collections depending on the underlying type of diagnostic content they provide.

Table 3 shows the standardized resource collections for SOVD. This resource collection classification helps a generic tester to present the resources of different types in a specific way. Each resource collection can be queried for its contained resources.

Table 3 Standardized resource collections

| Resource Collection | Description |
| --- | --- |
| configurations | Collection of configuration resources |
| bulk-data | Collection of bulk data resources for the respective BulkDataCategory |
| data | Collection of static and dynamic data resources |
| data-lists | Collection of combined data resources of an entity |
| faults | Collection of fault resources of an entity |
| operations | Collection of operation resources |
| updates | Collection of update package resources |
| modes | Collection of mode resources |
| locks | Collection of lock resources |

Table 4 gives an overview on which resource collections are allowed on which entity type.

Table 4 Relation between entities and resource collections

| Entity level | configurations | bulk-data | data | data-lists | faults | operations | updates | modes | locks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOVDServer | X | X | X | X | X | X | X | X | X |
| Area | not supported by this standard |  |  |  |  |  |  |  |  |
| Component | X | X | X | X | X | X | 1 | X | X |
| App | X | X | X | X | X | X | 1 | X | X |
| Function | not supported by this standard |  |  |  |  |  |  |  |  |

    - Standardized Resources

The resource names shown in Table 5 are defined by this standard for an entity.

Table 5 Standardized resource names

| Name | Description |
| --- | --- |
| docs | Provides the online capability description. |
| version-info | Provides supported SOVD API versions. |
| logs | Access to logging information |
| related-apps | Apps hosted on a Component |
| related-components | Components related to an Area |
| data-categories | Categories (currentData, identData, ...) used within data resource collections. |
| data-groups | Groups used to structure data in a specific way (e.g., engine performance related, ...) |
| authorize | Initiate authentication and authorization with the vehicle |
| token | Request token from the vehicle for authorization |

Table 6 gives an overview, which resource is allowed to be used on which entity type.

1 Not allowed

Table 6 Relation between entities and resources

| Entity level | docs | version-info | logs | related-apps | related-components | data-categories | data-groups | authorize | token |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOVDServer | X | X | X | 2 | 2 | X | X | X | X |
| Area | X | 1 | 2 | 2 | X | 2 | 2 | 2 | 2 |
| Component | X | 1 | X | X | 2 | X | X | 2 | 2 |
| App | X | 1 | X | 2 | 2 | X | X | 2 | 2 |
| Function | X | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |

Resources, which are created by an SOVD client by usage of the SOVD API are considered “temporary”. Consequently, they are not available in a capability description.

Note: An example of resources and resource collections is shown in Appendix: A.

  - SOVD API Versioning

The SOVD API uses URI based versioning. Thus, the version of the SOVD API is included in the URI.

For this standard version base_uri is defined as https://<SOVD-Server-Host>/<OEM specific>/v1/  The optional path element <OEM specific> can be used to avoid clashes

in the URIs provided by in-vehicle HTTP servers.

The SOVD API provides the URI https://<SOVD-Server-Host>/<OEM specific>/version-info to allow SOVD clients to determine which versions are supported by an SOVD server. The path to the version-info resource shall remain the same for this and all future versions of the SOVD API.

If an SOVD client uses a version not supported by the SOVD server, the server shall respond with the status code 404 (not found). An SOVD server may support multiple versions or may redirect an SOVD client to the same URI of the supported version as long as this URI provides syntactically the same information.

Table 7 shows the schema of the version-info resource.

2 not supported by this standard

Table 7	VersionInfo type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| sovd_info | SOVDInfo[] | M | SOVD specific information |
| schema | OpenAPI Schema | M | Schema of the version-info resource |

Table 8	SOVDInfo type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| version | string | M | Version of the SOVD standard supported by the SOVD server as a semantic version string (see https://semver.org) |
| base_uri | string:uri-reference | M | Version specific base URI for interacting with the SOVD server. |
| vendor_info | VendorInfo | O | Vendor specific information |

Table 9	VendorInfo type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| version | string | M | Version of the SOVD server (vendor specific string). |
| name | string | M | Name of the vendor of the SOVD server. |

The VendorInfo type may include additional vendor specific information describing the SOVD server implementation in more detail.

  - Internationalization and Localization

    - Internationalization

Most resources accessible through the SOVD API provide a human-readable name using the attribute name. This name can be translated using the identifier defined by the attribute translation_id which is provided as a sibling of the attribute name.

If the response contains multiple internationalizable texts (as in Table 42), the translation identifier is built up by using the name of the internationalizable attribute and appending

_translation_id. For example, in Table 42 the attribute symptom can be internationalized and thus the internationalization identifier is provided by the attribute symptom_translation_id.

    - Unit Definitions

The SOVD API provides information on the unit of a value (e.g., odometer distance in kilometers) so that the data can be interpreted correctly. For converting the value into different unit systems (e.g., imperial and metric), the unit information provided by the SOVD API contains the physical dimensions as well.

The unit information is provided as part of the schema in the attribute x-sovd-unit with the following attributes:

Table 10	x-sovd-unit attributes

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| display_name | string | M | The string for displaying the unit in an SOVD client (e.g., “km/h” or “min-1”. |
| reference | string | O | Identifier of the unit for localization, e.g., as provided by the ODX_RS_UNIT_LIB.odx-d [26]. |
| factor_si_to_ unit | number | M | Factor to calculate the value in the defined unit. |
| offset_si_to_ unit | number | O | Offset to add to the value. |
| physical_ dimension | PhysicalDimen sion | C | Definition of the physical dimension based on SI units Condition: Unit has physical dimension |

Table 11	PhysicalDimension type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| length | integer | C1 | Exponent for length |
| mass | integer | C1 | Exponent for mass |
| time | integer | C1 | Exponent for time |
| current | integer | C1 | Exponent for current |
| temperature | integer | C1 | Exponent for temperature |
| molar_amount | integer | C1 | Exponent for molar-amount |
| luminous_ intensity | integer | C1 | Exponent for luminous intensity |

- C1: Only set if dimension is not 0.

Example:

x-sovd-unit: display_name: 'km/h' factor_si_to_unit: 3.6

offset_si_to_unit: 0 physical_dimension:

length: 1

time: -1

    - Other Localization Topics (Numbers and Dates)

Numbers are always transmitted using JSON numbers. Thus, there is no need to specify a localization for numbers.

Dates provided by the SOVD server (e.g., expiration of locks) are always provided according to RFC 3339 [19] and require no localization on the SOVD server side.

If an entity provides dates (e.g., the programming date of an entity provided as a data resource), the SOVD server shall keep the existing format and not perform any localization.

  - Response Status Codes

The SOVD API aims to follow the standard HTTP status codes as defined in RFC 7231 ([2], chapter 6), to indicate the success or failure of a request. The following status codes are defined:

- 2xx success status codes confirm the successfull execution of the request.

- 4xx error status codes indicate an error on the SOVD client side.

- 5xx error status codes indicate an error on the SOVD server side.

    - Generic Definition of Status Codes

In the following tables the commonly used response status codes which are applicable for the SOVD API are described. The description of a specific method may provide additional details or further response status codes if necessary.

Table 12:	Common SOVD server-side supported status codes on success

| Code | Text | Description |
| --- | --- | --- |
| 200 | OK | The request has been successful. This response code is also used if empty collections are returned. |
| 201 | Created | A new resource, e.g., a data list, has been created. |
| 202 | Accepted | The request has been accepted, but a response is not yet available. More details on the response are defined in the methods which return this status code. |
| 204 | No Content | The request has been successful, but the response provides no processable content. |

Table 13:	Common SOVD server-side supported status codes on SOVD client error

| Code | Text | Description |
| --- | --- | --- |
| 400 | Bad request | The request from the SOVD client was invalid. |
| 401 | Unauthorized | The access to the requested resource requires prior authentication. No or insufficient authentication information has been provided (expired, malformed, incorrect credentials, …). |
| 404 | Not found | The requested resource could not be found. |
| 406 | Not acceptable | The requested resource representation is not supported. |
| 409 | Conflicted | The request conflicted with the current state of the entity or resource. |
| 415 | Unsupported Media Type | The request could not be serviced because the payload is in a format not supported by the method of the target resource. |

Table 14:	Common SOVD server-side supported status codes on SOVD server error

| Code | Text | Description |
| --- | --- | --- |
| 500 | Internal Error | An internal error occurred. |
| 501 | Not implemented | The SOVD server does not support the required functionality. |
| 503 | Method Unavailable | The method is temporarily unavailable. |
| 504 | Gateway Timeout | No response received from the underlying entity (e.g., no UDS response from an ECU) in a given amount of time. |

    - Definition of Error Details

It is recommended that the response body of an error response includes a more detailed description of the error which helps in diagnosing the issue. The method descriptions in chapter 6 use the GenericError defined here to provide this additional information.

Table 15	GenericError type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| error_code | string (see ErrorCode) | M | SOVD standardized error code |
| vendor_code | string | C | Vendor specific error code Condition: If the value of error_code is vendor- specific this attribute has to be filled. |
| message | string | M | Message describing the problem in more detail. |

| translation_id | string | O | Identifier for translating the message |
| --- | --- | --- | --- |
| parameters | Map of string values (defined as additionalProperties) | O | Key-value-list which provides additional information on the error. |

On some occasions an error applies only to parts of a response, e.g., when reading a data resource. In this case the type DataError is used which includes a path to the erroneous attribute of the response:

Table 16	DataError type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| path | string:json-pointer | M | A JSON Pointer describing which element of the response is erroneous. |
| error | GenericError | C | Describes the error more precisely. Condition: The attribute referenced by path contains an error. |

    - SOVD Error Codes for Common Cases

Table 17	SOVD error codes for common cases

| ErrorCode | Description |
| --- | --- |
| vendor-specific | Details are specified in the vendor_code |
| not-responding | The Component which handles the request (e.g., an ECU) has been queried by the SOVD server but did not respond. |
| error-response | The Component receiving the request has answered with an error. For UDS, the message should include the service identifier (Key: ‘service’ and Value of type number) and the negative response code (Key: ‘nrc’ and Value of type number). |
| invalid-signature | The signature of the data in the payload is invalid. |
| incomplete-request | The request does not provide all information (e.g., parameter values for an operation) required to complete the method. The message should include references to the missing information. |
| invalid-response-content | The response provided by the Component contains information which could not be processed. E.g., the response of an ECU does not match the conversion information known to the SOVD server. |

|  | The message should include references to the parts of the invalid response attribute as well as a reason why the attribute is invalid. |
| --- | --- |
| sovd-server-misconfigured | The SOVD server is not configured correctly, e.g., required configuration files or other data is missing. The message should include further information about the error. A client shall assume that this error is fatal and a regular operation of the SOVD server cannot be expected. |
| sovd-server-failure | The SOVD server is able to answer requests, but an internal error occurred. The message should include further information about the error. |
| insufficient-access-rights | The SOVD client does not have the right to access the resource. |
| precondition-not-fulfilled | The preconditions to execute the method are not fulfilled. |
| update-process-in-progress | An update is already in progress and not yet done or aborted. |
| update-automated-not-supported | Automatic installation of update is not supported. |
| update-preparation-in-progress | An update is already in preparation and not yet done or aborted. |
| update-execution-in-progress | Another update is currently executed and not yet done or aborted. |

  - Common Primitive Data Types

Table 18 presents the list of common JSON data types used within the SOVD API in addition to those defined in the OpenAPI specification (see [10] which itself is defined on top of the JSON value types introduced in RFC 8259 (see [28])).

Note: In this specification the format x:y is used in the definition for an attribute's type to denote that x refers to the JSON type and y provides a basic semantic identification of the value following the JSON Schema validation.

Table 18	SOVD primitive data types

| JSON Type | JSON Format | Comment |
| --- | --- | --- |
| string | byte | Base64 encoded characters |
| string | binary | A sequence of octets, e.g., binary/bulk data |
| string | password | A hint to UIs to obscure input. |
| string | email | An email address according to RFC 5322 (see [30]) |
| string | uri | An absolute URI according to RFC 3986 (see [31]), e.g., {base_uri}/components/ecu-1 |
| string | uri-reference | A URI reference according to RFC 3986 (see [31]), e.g., /components/ecu-1 |
| string | uri-template | An URI template string according to RFC 6570 (see [32]), e.g., /components/{ecu-id}/data |

| string | json-pointer | A string pointing to a specific value within a JSON document according to RFC 6901 (see [33]), e.g., #/data/batteryVoltage |
| --- | --- | --- |
| string | uuid | A Universally Unique Identifier (UUID) according to RFC 4122 (see [34]) |
| string | date | A full date according to RFC 3339 (see [19]) |
| string | date-time | A date-time according to RFC 3339 (see [19]) |
| string | time | A time according to RFC 3339 (see [19]) |
| string | duration | A duration according to RFC 3339 (see [19]) |
| string | period | A period according to RFC 3339 (see [19]) |
| string | regex | Regular expressions as defined in ECMA 262 (see [17]) |
| string | hex | A string representing a hexadecimal number, e.g., “fe001a34” |
| AnyValue |  | The type of the attribute is either object, array, number, integer, boolean or string. |

Table 19	Mapping of ASAM data types

| ASAM Data Type | JSON Type | JSON Format | Comment |
| --- | --- | --- | --- |
| A_ASCIISTRING | string | - |  |
| A_BITFIELD | string | bit-field | The bitfield is represented as a string of 0 and 1s and not as a number. |
| A_BOOLEAN | boolean | - |  |
| A_BYTEFIELD | string | byte |  |
| A_FLOAT32 | number | float |  |
| A_FLOAT64 | number | double |  |
| A_(U)INT8 / 16 / 32 | integer | int32 |  |
| A_(U)INT64 | integer | int64 |  |
| A_UNICODE2STRING | string | - |  |

  - Definition of JSON Schema and Objects for Vehicle Responses

The SOVD API returns any information retrieved from the vehicle as JSON objects and utilizes the JSON schema for representing the schema (structure, attributes, types etc.) of the object. A common example is that the vehicle returns the programming and coding date of an ECU as a response structure like the following:

Figure 6	Example response structure

For this response structure the following example JSON schema and object is generated:

Table 20	Mapping of example response structure to JSON

| Schema | Object |
| --- | --- |
| { "type": "object", "properties": { "PA_Programming_Date": { "type": "object", "properties": { "PA_Year": { "type": "integer" }, "PA_Month": { "type": "integer" }, "PA_Day": { "type": "integer" } } }, "PA_Coding_Date": { "type": "object", "properties": { "PA_Year": { "type": "integer" }, "PA_Month": { "type": "integer" }, "PA_Day": { "type": "integer" } } } } } | { "PA_Programming_Date": { "PA_Year": 2021, "PA_Month": 10, "PA_Day": 10 }, "PA_Coding_Date": { "PA_Year": 2021, "PA_Month": 10, "PA_Day": 10 } } |

The rules for generating the JSON schema and object are as follows:

- Simple attributes are mapped to JSON attributes using a type as defined in Common Primitive Data Types.

- Structures are represented as a JSON object where each of the attributes of the structure is an attribute in the JSON object. For the enclosed attributes the mapping described here is applied again.

- Arrays are mapped to JSON arrays. The element’s type in the array is determined by the type of the JSON element in the JSON array. For example, an array of strings would be mapped to a JSON array of JSON strings, and an array of complex structures would be mapped to a JSON array of JSON objects.

- Multiplexers are complex structures where the relevant attributes are defined by a value. The attribute which defines the type to be used is specified using the Discriminator object as defined by the OpenAPI specification together with oneOf definition in the JSON schema to define the JSON schema for the possible values.

  - Discovery of the SOVD Server for the Proximity Use Case

    - General Concept

Use Multicast DNS (mDNS, RFC6762 [20]) to obtain the IP address and DNS Service Discovery (DNS-SD, RFC 6763 [24]) to identify SOVD servers. It is recommended to define a suitable Time To Live (TTL) for mDNS to ensure that Services which have disappeared from the network will not appear to be valid for a longer period.

    - Network Connection

The SOVD server shall connect to the local network using an existing DHCP server or if not available use an AutoIP address.

The SOVD server shall listen on the port 7690.

    - Address an SOVD Server Using mDNS

The public SOVD server shall implement mDNS host according to RFC6762 ([20]). It shall respond to its local hostname which consists of the hostname (could be the VIN or user defined) (e.g., “ABC123456789.local”).

In case of multiple public SOVD servers, each public SOVD server has to be assigned a unique hostname (e.g., by extending the VIN with (1), (2) and so on).

Note: This is already done by the mDNS protocol: In case a foreseen hostname already exists in the subnet, e.g., “(1)” is added to the hostname.

Note: It is recommended for each SOVD server to provide its own unique hostname in order not to have non-deterministic name assignment (with (1), (2) etc.).

    - Discovery of the SOVD Server Using DNS-SD

The public SOVD server shall implement DNS based service discovery as described in RFC6763 ([24]) with the following parameters:

- Protocol description: ASAM SOVD

- Primary Transport Protocol: TCP

- TXT Record key/value pairs

  - identification=ABC122345789

  - accessurl=https://ABC122345789.local:9999/vehicle

    - Example

Client asks via DNS-SD for available SOVD Services (broadcast):

$ dns-sd -B _sovd._tcp local.

Answer:

| Timestamp | A/R Flags | if Domain | Service Type | Instance Name |
| --- | --- | --- | --- | --- |
| 23:54:47.783 | Add 3 | 4 local. | _sovd._tcp. | ABC123456789 |

Then the client asks the vehicle for infos (Lookup):

$ dns-sd -L "ABC123456789" _sovd._tcp. local.

Answer:

DATE: ---Thu 02 Dec 2021---

16:44:24.438 …STARTING…

16:44:24.816 ABC123456789._sovd._tcp.local. can be reached at ABC123456789.local.:9999 (interface 11)

Then the IP address is resolved:

$ dns-sd -Gv4v6 ABC123456789.local

Answer:

DATE: ---Thu 02 Dec 2021---

16:48:50.274 …STARTING…

Timestamp	A/R Flags	if Hostname	Address	TTL 16:48:50.257 Add 40000003 11 ABC123456789.local. 2A02:8070:A1C2:…8EFF:FE35 120

16:48:50.257 Add 40000002 11 ABC123456789.local. 192.168.178.28	120

  - Discovery of the SOVD Server for the Remote Use Case

In case the SOVD API is also provided for the remote use case, there are several ways how to discover and how to address individual vehicles. This part is not specified in the SOVD standard.

- Capability Description

  - Introduction

This chapter introduces the structure and definitions underlying the SOVD capability descriptions. The documentation of an SOVD API exposed by an SOVD Server, in form of an SOVD capability description, builds on top of the OpenAPI Specification [10] (with version >= 3.1.0). The following subchapters will provide an overview of SOVD-specific definitions as well as conventions and recommendations for documenting an SOVD API.

For the convenience of the user, a machine-readable definition of the API methods (YAML) is published alongside this document. It is recommended to use the structure and generic types described in this file when building an offline capability description.

  - Capability Description Structure

An SOVD capability description is written in the form of the OpenAPI Specification. Table 21 lists the mandatory fields of an OpenAPI Specification which shall be used for documenting an SOVD API, if the respective feature is implemented by the underlying SOVD Server.

Table 21	SOVD API required fields

| Attribute | Description |
| --- | --- |
| servers | Documentation of the single SOVD Server the SOVD capability description is defined for. |
| paths | Documentation of SOVD resources implemented by the SOVD server. |
| components | Documentation of the SOVD-specific definitions as well as OEM-specific reusable definitions, e.g., parameters, responses, schemas, etc. |
| security | Documentation of the API security mechanism implemented by the SOVD Server. |

The following list of fixed fields introduced by OpenAPI Specification are not used as part of an SOVD capability description: jsonSchemaDialect, webhooks, and externalDocs.

Further SOVD-specific extension properties are introduced in the following subchapters using the underlying extension mechanism of the OpenAPI Specification. These extension properties allow to document further SOVD specific information and are therefore grouped based on the OpenAPI Specification objects they can be specified for. For example, the SOVD Standard version underlying to an SOVD capability description can be documented as part of the Info object of an OpenAPI document as presented in subchapter 5.2.1.

    - Info Object

Table 22 provides an overview of all SOVD extension properties for an OpenAPI Info object.

Table 22	Extension Property Info

| Extension Identifier | Usage in SOVD | Description |
| --- | --- | --- |
| x-sovd-version | Mandatory | Documents the SOVD standard version underling to the capability description. |

    - Path Item Object

Table 23 provides an overview of all SOVD extension properties for an OpenAPI Path Item

Object.

Table 23	Extension Property Path Item

| Extension Identifier | Usage in SOVD | Description |
| --- | --- | --- |
| x-sovd-lock-required | Conditional | Documents if a lock is required for the interaction with a particular resource. If specified as part of a Path Item Object, any action (HTTP method) on the resource would require a lock. |
| x-sovd-required-modes | Conditional | Documents the list of required modes which have to be set to an entity in order to invoke the method defined by this Path Item Object, e.g., writing an ECU configuration. |
| x-sovd-data-category | Conditional | Documents the category of a data resource exposed via the SOVD API. |
| x-sovd-name | Mandatory | Documents the name of a resource. |
| x-sovd-data-groups | Conditional | Documents the groups an individual data resource belongs to. The type of the value is an array of ValueGroup. |
| x-sovd-fault-scope | Conditional | Documents the scope for which a fault is defined. See attribute scope in Fault for more details. |
| x-sovd-fault-display-code | Conditional | Documents the display code of a fault. See attribute display_code in Fault for more details. |
| x-sovd-fault-severity | Conditional | Documents the severity of a fault. See attribute severity in Fault for more details. |

    - Operation Object

Table 24 provides an overview of all SOVD extension properties for an OpenAPI Operation

object.

Table 24	Extension Property Operation

| Extension Identifier | Usage in SOVD | Description |
| --- | --- | --- |
| x-sovd-lock-required | Conditional | Documents if a lock is required for the interaction with a particular resource. If specified as part of an Operation object, the lock is required only for performing this particular action (HTTP method, e.g., POST) on the resource. |
| x-sovd-required-modes | Conditional | Documents the list of required modes which have to be set to an entity in order to invoke the method defined by this Operation object, e.g., writing an ECU configuration. |
| x-sovd-proximity-proof-required | Conditional | Documents that the execution of an entity operation requires a proof of proximity. |
| x-sovd-asynchronous-execution | Conditional | Documents whether the execution of the operation is asynchronous or not. |
| x-sovd-capabilities | Conditional | Documents the list of SOVD capabilities a specific operation of an entity supports at the respective methods available at the SOVD API. |

    - Schema Object

Table 25 provides an overview of all SOVD extension properties for an OpenAPI Schema

object.

Table 25	Extension Property Schema

| Extension Identifier | Usage in SOVD | Description |
| --- | --- | --- |
| x-sovd-unit | Conditional | Documents the unit of a property within a JSON schema. |
| x-sovd-translation-id | Conditional | Documents a translation identifier for a property within a JSON schema, e.g., the name of a resource. |

    - Header Object

The following subchapter provides an overview of defined SOVD headers that are used across the SOVD API methods.

      - Authorization Request Header

Whenever a client interacts with resources exposed by an SOVD server by sending HTTP requests to its SOVD API, a valid token shall be provided within the HTTP Authorization

header of the request, if access to the resource requires authorization. If the authorization schema requires, other headers may be used as well.

      - Location Response Header

Whenever an SOVD client has to be informed about the resource location of a newly created resource or the client should be re-directed, the resulting response provides a respective HTTP Location header where the value is an absolute URI pointing to the related resource.

      - X-SOVD-Mode Request header

If a required ECU mode is not explicitly set before, a Hint-based mode control on the target mode and its value can be provided through the X-SOVD-Mode header.

    - Parameter Object

The following subchapter provides an overview of defined SOVD query parameters which are used across the SOVD API methods to detail resource requests or specify filter criteria for the returned responses. For serializing query parameter values, SOVD follows the default definitions of OpenAPI Specification. Therefore, query parameters shall be specified using the “form” style with “explode=true”. Based on that, query parameter values of type array or object are represented by repeating the corresponding query parameter for each value of the array or property of the object. For example, a request for retrieving the set of data of two specific categories and requesting the related schemas in the response will look like the following:

GET	{base_uri}/{entity-path}/data?data-categories=identData&data-categories=currentData&include-schema=true HTTP/1.1

      - include-schema

The query parameter include-schema allows to specify whether the response should include schema information in form of a schema property value or not. The schema attribute will include the definition of all properties in the response. This allows to add OEM-specific extensions. A property may include its own schema property to be more precise on the type definition for the object in which it is contained. If the query parameter include-schema is set to true, the schema properties as defined by the specification is set on all objects in the response.

  - Online and Offline Capability Descriptions

    - Introduction

The following chapter describes how an SOVD client learns about the diagnostic capabilities of a vehicle. Technically, the capabilities are described in an OpenAPI specification. SOVD distinguishes between two different scenarios.

In the first scenario, a diagnostic tool developer wants to develop an SOVD client for a complete series including all possible variants. Thus, the developer needs upfront information about the diagnostic capabilities which might be available on a vehicle. For this scenario, the SOVD standard defines an offline capability description (see 5.3.2), which contains complete information about all diagnostic capabilities and for which vehicle variants they are applicable (see 5.4).

In the second scenario, a diagnostic tool developer develops a fully generic diagnostic SOVD client without any prior knowledge about the vehicle. For this case, the SOVD standard defines an online capability description (see 5.3.3), which can be read from the vehicle by the SOVD client. The returned OpenAPI specification can be used by the SOVD client to identify the available entities, resources and which methods can be applied to them.

An SOVD capability description is a well-formed and fully specified OpenAPI specification. This allows SOVD clients to handle online capability descriptions and offline capability descriptions in the same way as well as use the same tooling and mechanisms for parsing and utilizing their content. The main difference is the scoped context on the specified parts of the API (see 6.4.2).

    - Offline Capability Description

An offline capability description provides the documentation for all SOVD methods of a vehicle and therefore specifies how to access and interact with all available entities, e.g., Components or Apps.

    - Online Capability Description

The SOVD online capability description is a mechanism which allows clients to query how to interact with a specific resource provided by the SOVD server. It is fully compatible to an offline capability description, limited to a single resource. An SOVD server may not provide an online capability description for standardized entities, resources, or resource collections.

  - Variant Handling in Offline Capability Description

    - Introduction

Traditionally, vehicle projects are developed with multiple variants of ECUs which contain changes to the functional specification based on vehicle model, features opted by the customer, generational evolution of ECUs, etc. These variations in ECU software specification in-turn result in changes in the diagnostic specifications. For instance, the resource for reading the SteeringWheelRotationSpeed can be excluded if the model variant does not support it. The process of handling variants, in classical diagnostics, is achieved through ECU-VARIANTs in ODX data (see [9]).

In SOVD, the two types of entities – Components, which represents the classical ECUs and HPCs -, and Apps, which represents applications running on HPCs, require the possibility to define variants. The process of handling variants in SOVD can be achieved through an OpenAPI custom extension (see [10]) detailed out in this chapter.

Note: The process to identify and manage variants is proprietary and depends heavily on tools, use cases, engineering processes and rules, etc. of the OEM. Hence, the mechanisms described below are one way of handling variants. There could be alternative mechanisms of managing variants. Such alternatives can be accommodated within the implementations if the OpenAPI and SOVD standard boundaries are not violated.

    - Extension: x-sovd-applicability

SOVD introduces the custom extension x-sovd-applicability that can be defined in the offline capability description. The extension contains the list of variants which support a specific resource (path), response structure (schema), etc. The specification author can introduce it at various fields of the document as needed complying with the OpenAPI standard requirements.

Note: Feature options opted by the customer while purchasing the vehicle also result in numerous variants of the vehicle. They are typically represented in vehicle manufactures using “option codes” which is an expression representing all possible options of the specific vehicle. A support for a resource could also be determined based on that expression. OEMs can add such option code strings to the extension as a bespoke requirement. The processing of such expressions is usually proprietary to the vehicle manufactures and hence not part of this standard.

    - Usage of x-sovd-applicability by the SOVD Client

A client shall use the information in the offline capability description to drive the behavior of the application based on the variant mounted in the vehicle. The variant information is present as part of the response of the method GET /{entity-path}/{entity-Id}. Once the variant information is known, the client can use this information to determine, from the x-sovd-applicability extension in offline capability description, which resources or schemas match for the vehicle to be diagnosed.

In   the   example,   the   resource   /components/PowerSteering/data

/SteeringWheelRotationSpeed is applicable for the variant “Pow_Str_Variant_High” only.

"/components/PowerSteering/data/SteeringWheelRotationSpeed": { "get": {

"summary": "get the steering wheel rotation speed for diagnostics",

"description": "get the steering wheel rotation speed for diagnostics",

"responses": {

"200": {

"description": "positive response schema for steering wheel rotation speed",

"content": { "application/json": { "schema":{

"$ref":"#/components/schemas/SteeringWheelRotationSpeed"

}

}

}

}

}

},

"x-sovd-applicability": {

"description": "Applicability Extension to specify which variant supports this schema",

"variant_id_list": { "type": "array", "items": [

"Pow_Str_Variant_High"

]

}

}

}

- SOVD REST API

  - Introduction

In the following chapters, the standardized SOVD API methods are introduced. For each API method, the underlying path and query parameters, request, and response headers as well as request and response bodies and the resulting response status codes of the API methods are specified. Standardized HTTP request and response headers are only specified if they are explicitly required from an SOVD viewpoint, like for instance authorization. In addition, request headers are defined as required on top of standard HTTP headers.

In case an SOVD server provides additional request parameters, the functionality shall be guaranteed also in case they are not used by an SOVD Client.

  - Definition of Conventions for API Methods

    - Query Parameters

The convention optional means, that the parameter may be set by the SOVD client. In case the parameter is not used, the default is valid in the SOVD server, if described. The evaluation of the parameter is mandatory for the SOVD server.

    - Request Body

The convention optional means, that the parameter is supported by the SOVD server, when defined in the capability description. In this case the SOVD client shall set the parameter.

  - Timeout Definitions

The SOVD API definition does not define any timeouts. However, there are topics which are not covered by the regular HTTP protocol timeouts and thus these timeouts are implementation specific. An SOVD server implementation has to provide the timeout in seconds then as part of the response which initiated the action.

Table 26	Timeout Definition Header

| Header | Operation | Description |
| --- | --- | --- |
| X-SOVD-Status-Retention-Timeout | POST /{entity-path}/operatio ns/{operation-id}/executions PUT /updates/{upda te-package-id}/[automated ] ¦¦[prepare]¦¦ [execute] | Time after which the status information (e.g., execution of an operation or update) will not be available anymore after the action completed. |

  - API Methods for Access to Capability Description Content

    - Introduction

The purpose of an online capability description is to get all required information to use a specific SOVD method, in a self-contained manner and without requiring an overall offline capability description. Therefore, the SOVD API allows to request such online capability descriptions at individual resources of the SOVD API to provide corresponding online documentation to SOVD clients. For example, if an online capability description is requested at a specific operation resource of an ECU, the SOVD API returns an OpenAPI specification, which describes all relevant information needed for interacting with this operation resource. This comprises, for example,

- the supported HTTP methods of the operation

- related request and response structures

- defined HTTP response status codes

as well as SOVD-specific extensions such as the supported capabilities of the operation or

translation_ids for parameters within requests or responses.

    - Query an Online Capability Description

Method:

GET /{Any path}/docs

Method Description:

For entities, resources, and resource collections, a corresponding online capability description can be requested at the SOVD API. The resulting online capability description is a self-contained, valid OpenAPI specification. The description contains information which refers to the creation, reading, updating, or deleting of the respective element and its direct child elements as defined by the SOVD standard.

An online capability description is requested for a data resource /{entity-path}/data/{data-id} via GET /{entity-path}/data/{data-id}/docs. For example, to enrich the responses with additional details such as the underlying schema of the response using the “include-schema” query parameter on the request. Moreover, for each supported HTTP method all defined HTTP response status codes are documented together with the data model of the resulting response payloads.

Path Parameters:

The path parameters depend on the resource to interact with.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Body:

The online capability description of the underlying resource in form of an OpenAPI specification in JSON document format.

Example:

Request:

GET {base_uri}/apps/WindowControl/data/RearWindows/docs HTTP/1.1

Response:

HTTP/1.1 200 OK

Content-Type: application/json [OpenAPI Spec]

  - API Methods for Discovering of Entities and Resources

    - Introduction

This chapter describes methods to

- Discover entities

- Discover subordinate entities of an entity

- Discover related entities of an entity (e.g., Components of an Area, Apps of a

Component)

This allows the client to traverse the topology to find out, which methods can be used for which resources. Using these methods, an SOVD client can traverse the topology to identify entities providing resources.

    - Query of Available Entities Under This SOVD Server

GET /{Entity-Collection}

Discover Contained Entities Method:

Method Description:

This method provides the list of contained entities for each requested entity collection (see Table 2).

Note: The collections contain the list of entities the SOVD server knows. It does not reflect the communication status with each entity.

Path Parameters:

Table 27	Path Parameters - Discover contained entities

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| Entity-Collection | EntityCollect ionType | M | Entity collection type |

Query Parameters:

Table 28	Query Parameters - Discover contained entities

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 29	Response Status Codes - Discover contained entities

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 30 | The request was successful. |

Response Body:

Table 30	Response Body - Discover contained entities of entity collections

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | EntityReference[] | M | Array of entity references. |
| schema | OpenAPI Schema | C | Schema of the contained entities. Condition: Only provided if the query parameter include- schema is true. |

Table 31	EntityReference type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Identifier of the entity. |
| name | string | M | Name of the entity. |
| translation_id | string | O | Identifier for translating the name |
| href | string:uri-reference | M | URI of the subordinate entity including {base_uri}. |

Example 1: Request:

GET {base_uri}/components HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id": "DrivingComputer", "name": "Driving computer",

"href": "{base_uri}/components/DrivingComputer"

},

{

"id":	"Camera", "name": "Camera Unit",

"href": "{base_uri}/components/Camera"

},

{

"id":	"PowerSteering", "name": "Power steering Unit",

"href": "{base_uri}/components/PowerSteering"

}

]

}

Example 2: Request:

GET {base_uri}/areas HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"name": "Autonomous Driving", "href": "{base_uri}/areas/Driving"

}

]

}

GET /areas/{area-id}/subareas

GET /components/{component-id}/subcomponents

Query Sub-Entities of an Entity Method:

Method Description:

This method provides the list of sub-entities for each Area or Component. In case of subcomponents or subareas these methods can be used to discover the resource tree.

Path Parameters:

This method accepts either the area-id or the component-id as a path parameter depending on the requested resource type (Area or Component), which is part of the URI.

Query Parameters:

Table 32	Query Parameters - Query sub-entities of an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 33	Response Status Codes - Query sub-entities of an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 34 | The request was successful. |

Response Body:

Table 34	Response Body - Query sub-entities of an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | EntityReference[] | M | Array of entity references. |
| schema | OpenAPI Schema | C | Schema of the contained entities. Condition: Only provided if the query parameter include-schema is true. |

Example 1: Request:

GET {base_uri}/components/DrivingComputer/subcomponents HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id":	"Linux",

"name": "Linux-based ADAS",

"href": "{base_uri}/components/DrivingComputer/subcomponents

/Linux"

},

{

"id":	"AdaptiveAUTOSAR", "name": "Adaptive AUTOSAR",

"href": "{base_uri}/components/DrivingComputer/subcomponents

/AdaptiveAUTOSAR"

}

]

}

Example 2: Request:

GET {base_uri}/areas/Driving/subareas HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id":	"Perception", "name": "Perception system",

"href": "{base_uri}/areas/Driving/subareas/Perception"

},

{

"id":	"Controlling", "name": "Controlling system",

"href": "{base_uri}/areas/Driving/subareas/Controlling"

}

]

}

GET /areas/{area-id}/related-components

GET /components/{component-id}/related-apps

Query Related Entities of an Entity Method:

Method Description:

This method provides the list of related entities (i.e., Components of an Area or Apps present on a Component) for each Area or Component.

Path Parameters:

This method accepts either the area-id or the component-id as a path parameter depending on the requested resource type (Area or Component), which is part of the URI.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 35	Response Status Codes - Query related entities of an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 36 | The request was successful. |

Response Body:

Table 36	Response Body - Query related entities of an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | EntityReference[] | M | Array of entity references. |

Example:

Request:

GET {base_uri}/areas/Driving/related-components HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id":	"DrivingComputer", "name": "Driving computer",

"href": "{base_uri}/components/DrivingComputer"

}

]

}

    - Query Entity Capabilities

Method:

GET /{entity-path}/{entity-id}

Method Description:

This method returns the capabilities of an entity.

Path Parameters:

This method accepts the entity-id as a path parameter.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 37	Response Status Codes - Query of entity capabilities

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 38 | The request was successful. |

Response Body:

The response body contains a property for each supported resource and related collection (see 4.3).

Note: This method returns the resource and related collections. Querying the resources of a specific resource-collection is described in the individual method subchapters.

Table 38	Response Body - Query of entity capabilities

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Entity-id |
| name | string | M | Name of the entity |
| translation_id | string | O | Identifier for translating the name |
| variant | AnyValue | C3 | Identification of the variant |
| configurations | string:uri-reference | C1 | A reference to the configurations collection |
| bulk-data | string:uri-reference | C1 | A reference to the bulk data collection |
| data | string:uri-reference | C1 | A reference to the data collection |
| data-lists | string:uri-reference | C1 | A reference to the data-lists collection |
| faults | string:uri-reference | C1 | A reference to the faults collection |

| operations | string:uri-reference | C1 | A reference to the operations collection |
| --- | --- | --- | --- |
| updates | string:uri-reference | C1 | A reference to the updates collection |
| modes | string:uri-reference | C1 | A reference to the modes collection |
| relatedapps | string:uri-reference | C2 | A reference to the reference-collection providing the Apps (only available for Components) |
| related components | string:uri-reference | C2 | A reference to the reference-collection providing the Components (only available for Areas) |
| subareas | string:uri-reference | C2 | A reference to the reference-collection providing the subareas (only available for Areas) |
| subcomponents | string:uri-reference | C2 | A reference to the reference-collection providing the subcomponents (only available for Components) |
| locks | string:uri-reference | C1 | A reference to the reference-collection locks |
| logs | string:uri-reference | C1 | A reference to the resource logs |

- C1: Reference is only present if the entity supports this resource collection or resource.

- C2: Reference is only present if the collections are not empty.

- C3: The attribute is provided in case the SOVD server supports variant identification as described in 5.4.

Example

Request:

GET {base_uri}/apps/AdvancedLaneKeeping HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"id":	"AdvancedLaneKeeping",

"name":	"Advanced Lane Keeping", "configurations": "{base_uri}/apps/AdvancedLaneKeeping/

configurations",

"bulk-data":	"{base_uri}/apps/AdvancedLaneKeeping/bulk-data", "data":	"{base_uri}/apps/AdvancedLaneKeeping/data",

"faults":	"{base_uri}/apps/AdvancedLaneKeeping/faults", "operations":	"{base_uri}/apps/AdvancedLaneKeeping/operations", "logs":	"{base_uri}/apps/AdvancedLaneKeeping/logs"

}

  - API Methods for Fault Handling

    - Introduction

The SOVD API provides methods for reading and deleting faults from an entity. A fault resource represents any kind of identified failure. In addition, for a single fault more detailed information (e.g., environment data from UDS) can be retrieved. If supported by the entity, single fault entries may be deleted as well. The method for reading faults provides sophisticated filtering mechanisms like providing only fault entries which fulfill certain criteria such as being an active fault.

The provided mechanisms are flexible enough to support reading fault entries from HPCs and ECUs. The provided mechanisms allow the transfer of additional contextual information for analyzing the root cause of the fault.

    - Read Faults from an Entity

Method:

GET /{entity-path}/faults

Method Description:

This method provides the fault entries which are detected for an entity. The returned fault entries may be filtered by their status, severity or scope using query parameters.

For the faults resource collection (/{entity-path}/faults) the capability description contains the schema definition of this collection and therefore not a detailed description of each fault.

The fault entries are contained in /{entity-path}/faults/{fault-code} and the capability description of each fault contains the details like the schema of the environment_data.

Path Parameters:

This method does not support path parameters.

Query Parameters:

If multiple filter parameters are included they are combined by logically AND.

Table 39	Query Parameters - Read faults from an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| status[key] | string | O | Filters the available elements based on a key from the status. The |

|  |  |  | provided value is a full match against the key and value of the attribute status in the response. For filtering by multiple status, the parameter is repeated (0 .. *). If multiple status are provided, they are logically OR combined. The keys supported for faults are defined in the capability description of the resource collection faults. |
| --- | --- | --- | --- |
| severity | integer | O | Filter fault entries by their severity. Any fault with a severity equal to or below the given number will be included in the response. |
| scope | string | O | The scope (e.g., user-defined fault memories) for which fault entries are retrieved. The capability description defines which scopes are supported. |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 40	Response Status Codes - Read faults from an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 41 | The request was successful. |

Response Body:

Table 41	Response Body - Read faults from an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | Fault[] | M | Collection of faults set in the entity |
| schema | OpenAPI Schema | C | Schema for describing the fault response. Condition: Only provided if the query parameter include-schema is true. |

Table 42	Fault type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| code | string | M | Fault code in the native representation of the entity. |
| scope | string | O | Defines the scope (e.g., user-defined fault memories) for fault entries. The capability description defines which scopes are supported. |
| display_code | string | O | Display representation for the fault code. |
| fault_name | string | M | Name / description of the fault |
| fault_ translation_id | string | O | Optional identifier for translating the name |
| severity | integer | O | Severity defines the impact on the availability of the vehicle. Lower number means higher severity. For a classic ECU this resembles the level from ODX, for HPCs the SOVD API recommends 1 = FATAL, 2 = ERROR, 3 = WARN, and 4 = INFO. |
| status | object | C1 | Detailed status information for the fault as key value pairs. For a classic ECU this resembles the status byte of a DTC. The content of the keys and values is OEM-specific. The keys are defined in the capability description. See Notes below. |
| symptom | string | C1 | Detailed symptom / failure mode information for the fault. For classic ECU this resembles the symptom of a DTC. The content of the attribute is OEM-specific. |
| symptom_ translation_id | string | C1 | Translation identifier for the symptom name. |
| schema | OpenAPI Schema | C2 | Schema of the fault element. |

- C1:	This attribute is only present in case the fault provides the requested information.

- C2:	This attribute is only present in case the query attribute include-schema is true.

Note: The status can be aggregated in the key aggregatedStatus and is then available for HPCs and classic ECUs. The aggregation of the status is vendor specific.

Note: For UDS based ECUs, the nomenclature of ISO 14229-1[1] as defined in appendix

D.2.3 is used as keys for the status attribute. In addition, the status byte in hex is included in the key mask.

Note: For HPCs it is advised to follow the nomenclature of ISO 14229-1 for status keys if reasonable.

Example (Classic ECU): Request:

GET

{base_uri}/components/Camera/faults?status[aggregatedStatus]=active HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"code": "0012E3",

"scope": "Default", "display_code": "P102",

"fault_name": "No signal from sensor", "fault_translation_id": "CAMERA_0012E3_tid", "severity": 1,

"status":

{

"testFailed": "1",

"testFailedThisOperationCycle": "1",

"pendingDTC": "1",

"confirmedDTC": "1",

"testNotCompletedSinceLastClear": "0",

"testFailedSinceLastClear": "1",

"testNotCompletedThisOperationCycle": "0",

"warningIndicatorRequested": "0", "aggregatedStatus": "active"

}

},

{

"code": "003C01",

"scope": "Default", "display_code": "P03C",

"fault_name": "Battery voltage low", "fault_translation_id": "Low Vbat_tid", "severity": 2,

"status":

{

"testFailed": "1",

"testFailedThisOperationCycle": "1",

"pendingDTC": "1",

"confirmedDTC": "1",

"testNotCompletedSinceLastClear": "0",

"testFailedSinceLastClear": "1",

"testNotCompletedThisOperationCycle": "0",

"warningIndicatorRequested": "0", "aggregatedStatus": "active"

}

}

]

}

Example (HPC): Request:

GET {base_uri}/apps/AdvancedLaneKeeping/faults?severity=1 HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"code": "modelMissing", "scope": "Default",

"fault_name": "No Object Recognition Model available", "fault_translation_id": "ALK NoObjModel_tid", "severity": 1,

"status":

{

"aggregatedStatus": "active"

}

}

]

}

Further examples for requests:

- UDS Status byte mnemonic:

GET {base_uri}/{entity-path}/faults?status[pendingDTC]=1&status[confirmedDTC]=1 HTTP/1.1

- UDS Status mask:

GET {base_uri}/{entity-path}/faults?status[mask]=1A HTTP/1.1

- Interpreted UDS status byte:

GET {base_uri}/{entity-path}/faults?status[aggregatedStatus]=active HTTP/1.1

- Faults from crashed applications:

GET {base_uri}/{entity-path}/faults?status[state]=crashed HTTP/1.1

    - Read Details for a Fault

Method:

GET /{entity-path}/faults/{fault-code}

Method Description:

This method provides the details for a fault-code.

Path Parameters:

Table 43	Path Parameters - Read details for a fault

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| fault-code | string | M | The fault code for which the detailed information should be retrieved. The value shall be either returned by the request GET /{entity-path}/faults or defined in the capability description. |

Query Parameters:

Table 44	Query Parameters - Read details for a fault

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include the schema information for the response. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 45	Response Status Codes - Read details for a fault

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 46 | The request was successful. |

Response Body:

Table 46	Response Body - Read details for a fault

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| item | Fault | M | Fault description |
| environment_ data | AnyValue | C | Additional OEM-specific information describing the environment of the system at the time of setting the fault. Condition: Only set if the fault provides the requested information. |

| errors | DataError[] | C | Condition: Only set, if the value environment_data represents an error, and this attribute describes the error more precisely. |
| --- | --- | --- | --- |
| schema | OpenAPI Schema | C | Schema for describing the response. Condition: Only provided if the query parameter include-schema is true. |

Example:

Request:

GET {base_uri}/apps/AdvancedLaneKeeping/faults/0012E3 HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"item":

{

"code": "0012E3",

"scope": "Default", "display_code": "P102",

"fault_name": "No signal from sensor", "fault_translation_id": "CAMERA_0012E3_tid", "severity": 1,

"status":

{

"testFailed": "1",

"testFailedThisOperationCycle": "1",

"pendingDTC": "0",

"confirmedDTC": "1",

"testNotCompletedSinceLastClear": "1",

"testFailedSinceLastClear": "0",

"testNotCompletedThisOperationCycle": "0",

"warningIndicatorRequested": "0", "mask": "1E", "aggregatedStatus": "active"

}

},

"environment_data":

{

"id": "env-data", "data": {

"battery_voltage": 12.8,

"occurence_counter": 12, "first_occurence": "2021-06-22T11:47:22Z", "last_occurence": "2021-07-01T10:12:53Z"

}

}

}

    - Delete All Faults of an Entity

Method:

DELETE /{entity-path}/faults

Method Description:

This method deletes all fault entries of an entity.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Table 47	Query Parameters - Delete all faults of an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| scope | string | O | Defines the scope (e.g., user-defined fault memories) for which fault entries are deleted. The capability description defines which scopes are supported. |

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Status Codes:

Table 48	Response Status Codes - Delete all faults of an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | The request was successful. |

Response Body:

This method does not provide a response body.

Example:

Request:

DELETE {base_uri}/components/Camera/faults HTTP/1.1

Response:

HTTP/1.1 204 No Content

    - Delete Single Fault of an Entity

Method:

DELETE /{entity-path}/faults/{fault-code}

Method Description:

This method deletes the given fault entry from an entity.

Path Parameters:

Table 49	Path Parameters - Delete single fault of an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| fault-code | string | M | The fault code which should be deleted. The value shall be either returned by the request GET /{entity-path}/faults or defined in the capability description. |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Status Codes:

Table 50	Response Status Codes - Delete single fault of an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | Fault entry has been deleted successfully. |
| 404 | -- | The SOVD client tried to delete a fault entry which is not defined for the entity. |
| 409 | GenericError | The SOVD client does not fulfill the conditions for deleting a single DTC, e.g., the DTC is safety relevant. The SOVD server should supply information in the GenericError which condition is not met (e.g., safety conditions, DTC not set, …) |
| 501 | -- | The request for deleting single fault entries is not supported by the entity. |

Response Body:

This method does not provide a response body.

Example:

Request:

DELETE {base_uri}/apps/AdvancedLaneKeeping/faults/modelMissing HTTP/1.1

Response:

HTTP/1.1 204 No Content

  - API Methods for Data Resource Read / Write Access

    - Introduction

The SOVD API described in this subchapter allows an SOVD client to read and write various kinds of data values from and to an entity. SOVD does not differentiate between identification, measurements, and other data values. To allow an SOVD client to differ between various kinds of data values, the categories defined by SOVD can be used to distinguish between measurements etc.

Table 51	DataCategory type

| Category | Description |
| --- | --- |
| identData | Identifications Read access to fixed parameters that identify entities within a vehicle, e.g., part number or VIN. |
| currentData | Measurements Read access to dynamically changing values, e.g., battery voltage. |
| storedData | Parameters Read and write access to parameters in the vehicle. |
| sysInfo | System information Read access to dynamically changing system resources, e.g., CPU load. |

Note: OEM-specific categories can be included using alpha-numeric characters, hyphens, and underscores and have to start with x-<oem>. An implementor of the SOVD API may introduce additional categories. These categories are then not supported by an off-the-shelf SOVD client.

The SOVD server optionally supports the grouping of data values for data which belongs together. Groups are OEM specific.

SOVD clients having to read multiple values at once multiple times can define a temporary data list for this. By reading the data list, the SOVD client will get all defined values in one response.

    - Query of Data Resource Categories and Groups

GET /{entity-path}/data-categories

Retrieve Categories Supported by a Data Resource Collection Method:

Method Description:

This method returns the data categories provided by an entity.

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 52	Response Status Codes - Retrieve categories supported by a data resource collection

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 53 | The request was successful. |

Response Body:

Table 53	Response Body - Retrieve categories supported by a data resource collection

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | DataCategory [] | M | Supported categories. |

Example:

Request:

GET {base_uri}/apps/WindowControl/data-categories HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [ "currentData", "identData"

]

}

GET /{entity-path}/data-groups

Retrieve Groups Supported by a Data Resource Collection Method:

Method Description:

This method provides the groups defined for an entity. Groups are associated with a category.

The concept of groups is not intended to be used for partitioning data from different subcomponents, e.g., LIN slaves should be mapped to subcomponents of the master ECU and	are	thus	accessible	using	a	URI	like

{base_uri}/components/master/subcomponents/lin-slave.

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 54	Response Status Codes - Retrieve groups supported by a data resource collection

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 55 | The request was successful. |

Response Body:

Table 55	Response Body - Retrieve groups supported by a data resource collection

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | ValueGroup[] | M | Available groups. |

Table 56	ValueGroup type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Unique identifier for a group across all categories |
| category | DataCategory | M | Category for which the group is defined |
| category_ translation_id | string | O | Identifier for translating the category name |

| group | string | O | Name of the group |
| --- | --- | --- | --- |
| group_ translation_id | string | O | Identifier for translating the group name |

Example:

Request:

GET {base_uri}/apps/WindowControl/data-groups HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id": "front", "category": "currentData"

},

{

"id": "rear", "category": "currentData"

}

]

}

    - Retrieve List of All Data Provided by the Entity

Method:

GET /{entity-path}/data

Method Description:

This method provides the list of all data resources available for an entity. The returned data may be filtered by their category (e.g., to limit the list to current data values) or by their group identifiers.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Table 57	Query Parameters - Retrieve list of all data provided by the entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| groups | string[] | C | Filters the available elements based on the groups they belong to. If an element belongs to multiple groups, it is included if one of the groups is part of these multiple groups. If a category is defined, the categories parameter is ignored by the SOVD server. The groups |

|  |  |  | parameter takes precedence over the category if an SOVD client defines both as the groups is more restrictive. |
| --- | --- | --- | --- |
| categories | DataCategory [] | C | Filters the available elements based on their category. If groups are defined, the categories parameter is ignored by the SOVD server. |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

An SOVD client should either provide the groups or categories query parameter in the request, but not both.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 58	Response Status Codes - Retrieve list of all data provided by the entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 59 | The request was successful. |

Response Body:

Table 59	Response Body - Retrieve list of all data provided by the entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | ValueMetaData [] | M | Definition of the data. |
| schema | OpenAPI Schema | C | The schema definition of the response. Condition: Only provided if the query parameter include-schema is true. |

Table 60	ValueMetaData type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Unique identifier for a value |
| name | string | M | Name of the data value |

| translation_id | string | O | Identifier for translating the data’s name |
| --- | --- | --- | --- |
| category | DataCategory | M | Category of the data value |
| groups | string[] | O | The identifiers of the groups to which the resource belongs to. |

Example:

Request:

GET {base_uri}/apps/WindowControl/data HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id":	"DriverWindow",

"name":	"Position of driver window", "category": "currentData",

"groups": [ "front" ]

},

{

"id":	"PassengerWindow",

"name":	"Position of passenger window", "category": "currentData",

"groups": [ "front" ]

},

{

"id":	"RearWindows",

"name":	"Position of rear windows", "category": "currentData",

"groups": [ "rear" ]

},

{

"id":	"AppInfo",

"name":	"Window Control Version Numbers", "category": "identData"

}

]

}

    - Read Single Data Value from an Entity

Method:

GET /{entity-path}/data/{data-id}

Method Description:

This method retrieves the value of a single data resource, like VIN, battery voltage, or CPU load. The response includes all values associated with the resource and cannot be filtered down to individual attributes of the resource.

Path Parameters:

Table 61	Path Parameters - Read single data value from an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| data-id | string | M | The identifier for the value to be read |

Query Parameters:

Table 62	Query Parameters - Read single data value from an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 63	Response Status Codes - Read single data value from an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 64 | The request was successful. |

Response Body:

Table 64	ReadValue type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Unique identifier of a value. |
| data | AnyValue | M | The value of the data. The type of the value is defined by the schema. |
| errors | DataError[] | C | Condition: Only set, if the value data represents an error and this attribute describes the error more precisely. |
| schema | OpenAPI Schema | C | Schema for the data. Condition: Only provided if the query parameter include-schema is true. |

Note: The attribute errors is used if the method execution has been successful, but the underlying resource returned an erroneous value. A common example is that an ECU did not provide the actual sensor value, but an indication that the value from the sensor is not plausible. In this case a status code signaling an error would be inappropriate. This applies also to single parameters inside the response.

Note: Complex data structures (nested parameters, lists) will be described via the respective data type inside the schema.

Example:

Request:

GET {base_uri}/apps/WindowControl/data/RearWindows?include-schema=true HTTP/1.1

Response (Success):

HTTP/1.1 200 OK

{

"id": "RearWindows", "data": {

"PositionLeft": 100,

"PositionRight": 0

},

"schema": { "type": "object",

"id": "string", "data": {

"type": "object", "properties": {

"PositionLeft": { "type": "integer",

"format": "int32", "minimum": 0,

"maximum": 100, "x-sovd-unit": {

"display_name": "%"

}

},

"PositionRight": { "type": "integer",

"format": "int32", "minimum": 0,

"maximum": 100, "x-sovd-unit": {

"display_name": "%"

}

}

}

}

}

}

Response (Value returned by component could not be evaluated):

HTTP/1.1 200 OK

{

"id": "RearWindows", "data": {

"PositionLeft": 100,

"PositionRight": 0

},

"errors": [

{

"path": "/data/PositionRight", "error": {

"error_code": "vendor-specific", "vendor_code": "value-constraint-violated",

"message": "Value provided by component could not be evaluated", "parameters": {

"internal_value": "FF", "description": "Signal implausible"

}

}

}

]

}

Response (Negative response):

HTTP/1.1 200 OK

{

"id": "RearWindows", "data": {

},

"errors": [

{

"path": "/data", "error": {

"error_code": "error-response", "message": "Error response received", "parameters": {

"service": 34,

"nrc": 49

}

}

}

]

}

    - Read Multiple Data Values from an Entity

GET /{entity-path}/data-lists

Retrieve List of All Data-Lists Provided by the Entity Method:

Method Description:

Provides the list of all data-list resources available for an entity.

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 65	Response Status Codes - Retrieve list of all data-lists provided by the entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 66 | The request was successful. |

Response Body:

Table 66	Response Body - Retrieve list of all data-lists provided by the entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | DataListEntry [] | M | The list of data lists together with the individual data identifiers |

Table 67	DataListEntry type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Identifier of the data list |
| items | ValueMetaData [] | M | Definition of the data. |

Example:

Request:

GET {base_uri}/apps/WindowControl/data-lists HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id": "7ab52f10-5ea9-4610-a1de-c414c8265ce4", "items": [

{

"id":	"DriverWindow",

"name":	"Position of driver window", "category": "currentData",

"groups": [ "front" ]

},

{

"id":	"PassengerWindow",

"name":	"Position of passenger window", "category": "currentData",

"groups": [ "front" ]

},

{

"id":	"RearWindows",

"name":	"Position of rear windows", "category": "currentData",

"groups": [ "rear" ]

}

]

}

]

}

POST /{entity-path}/data-lists

Creating a Data List for Reading Multiple Data Values at Once from an Entity Method:

Method Description:

This method creates a temporary resource for reading multiple data resources with one request from an entity. The identifiers of the requested data resources of the given entity shall be listed in the request body. The SOVD API does not impose any restrictions on the category or group of the identifier, especially identifiers from different groups and categories maybe mixed.

The resource has to be deleted explicitly by the SOVD client. If the SOVD client does not delete the resource, the SOVD server can delete it after a certain period if the resource is not used anymore or upon SOVD server restart.

Note: This specification does neither forbid nor mandate that an SOVD server has to return a unique identifier if SOVD clients provide the same identifiers in a request.

Any SOVD client may access a data list if the data list identifier is known. If an SOVD client does not have access to some (or all) resources referenced in the data list, the creation of the data list will anyways succeed. However, an SOVD client with insufficient rights will get an error when reading the data list (for details see Table 17).

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: application/json

Request Body:

Table 68	Request Body - Creating a data list for reading multiple data values at once from an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| ids | string[] | M | The data resource identifiers which should be read as part of the data list. |

Response Header:

The response includes the Location header to redirect the SOVD client to reading the resource for requesting the values:

Location: {base_uri}/{entity-path}/data-lists/{id} Response Status Codes:

Table 69	Response Status Codes - Creating a data list for reading multiple data values at once from an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 201 | Table 70 | The SOVD server created the data list. |
| 501 | -- | The SOVD server does not support the creation of data list resources |
| 400 | GenericError | The SOVD client request included a resource identifier which does not exist. The identifiers are listed in the error. |

Response Body:

Table 70	Response Body - Creating a data list for reading multiple data values at once from an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | The identifier which has to be used for querying the values of the data list. |

Example:

Request:

POST {base_uri}/apps/WindowControl/data-lists HTTP/1.1 Content-Type: application/json

{

"ids": [ "DriverWindow", "PassengerWindow", "RearWindows"

]

}

Response:

HTTP/1.1 201 Created

Location: {base_uri}/apps/WindowControl/data-lists/7ab52f10-5ea9-4610-a1de-c414c8265ce4

{

"id": "7ab52f10-5ea9-4610-a1de-c414c8265ce4"

}

GET /{entity-path}/data-lists/{data-list-id}

Read Multiple Data Values at Once from an Entity Using a Data List Method:

Method Description:

This method returns the values defined by the given data list.

Path Parameters:

Table 71	Path Parameters - Read multiple data values at once from an entity using a data list

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| data-list-id | string | M | The identifier for the data list returned by the SOVD server upon creation. |

Query Parameters:

Table 72	Query Parameters - Read multiple data values at once from an entity using a data list

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 73	Response Status Codes - Read multiple data values at once from an entity using a data list

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 74 | The request was successful. |

Response Body:

Table 74	Response Body - Read multiple data values at once from an entity using a data list

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | ReadValue[] | M | The list of values for the data resources specified as part of the data list. |
| schema | OpenAPI Schema | C | The schema definition of the response. Condition: Only provided if the query parameter include-schema is true. |

If an SOVD client is not authorized to read a certain resource, the attribute error_code of the item shall be set to insufficient-access-rights and the path attribute defines the value which could not be read.

Note: The order of the elements is implementation specific and may not be identical to the order of the URIs when the data list has been defined.

Example:

Request:

GET {base_uri}/apps/WindowControl/data-lists/7ab52f10-5ea9-4610-a1de-c414c8265ce4 HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"id": "7ab52f10-5ea9-4610-a1de-c414c8265ce4", "items": [

{

"id": "DriverWindow", "data": {

"Position": 100

}

},

{

"id": "PassengerWindow",

"data": { "Position": 100

}

},

{

"id": "RearWindows", "data": {

"PositionLeft": 100,

"PositionRight": 0

}

}

]

}

DELETE /{entity-path}/data-lists/{data-list-id}

Delete an Existing Data List Method:

Method Description:

This method removes the temporarily defined data list.

Note: Subsequent calls for this data list from other SOVD clients will result in status code 404 afterwards.

Path Parameters:

Table 75	Path Parameters - Delete an existing data list

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| data-list-id | string | M | The identifier for the data list. |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Status Codes:

Table 76	Response Status Codes - Delete an existing data list

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | The request was successful. |

Response Body:

This method does not provide a response body.

Example:

Request:

DELETE {base_uri}/apps/WindowControl/data-lists/7ab52f10-5ea9-4610-a1de-c414c8265ce4 HTTP/1.1

Response:

HTTP/1.1 204 No Content

    - Write a Data Value to an Entity

Method:

PUT /<entity-path>/data/{data-id}

Method Description:

This method writes the value of a data resource of an entity.

Note: Only a single resource can be written. Path Parameters:

Table 77	Path Parameters - Write a data value data to an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| data-id | string | M | The ID for the value to be written |

Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: application/json

Request Body:

Table 78	Request Body - Write a data value data to an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| data | AnyValue | M | The value of the data. The type of the value is defined by the resource. |
| signature | string | O | Additional security artefact for the data itself. This is independent from the authentication in the method header. |

Response Status Codes:

Table 79	Response Status Codes - Write a data value data to an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | Data written |
| 400 | DataError | The request body does either not contain all required parameter values or the signature for the provided values is wrong. |

Response Body:

This method does not provide a response body.

Example:

Request:

PUT {base_uri}/apps/WindowControl/data/AppInfo Content-Type: application/json HTTP/1.1

{

"data":

{

"version": "1.15.0",

"vendor": "TheWindowLifterCompany"

}

}

Response:

HTTP/1.1 204 No Content

  - API Methods for Configuration

    - Introduction

Components and Apps need to be configured to a specific environment, e.g., vehicle equipment, country, customer demand, variant coding, etc.

To achieve this goal, the SOVD API supports methods for reading and writing of configuration data. A configuration resource can be expressed as a set of parameters or as a byte stream to support multiple types of configuration data e.g., manifest (e.g., ASCII) or binary files.

    - Retrieve List of All Configurations Provided by the Entity

Method:

GET /{entity-path}/configurations

Method Description:

Provides the configurations provided by an entity.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Table 80	Query Parameters - Retrieve list of all configurations provided by the entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 81	Response Status Codes - Retrieve list of all configurations provided by the entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 82 | The request was successful. |

Response Body:

Table 82	Response Body - Retrieve list of all configurations provided by the entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | ConfigurationMetaData [] | M | The list of all configurations provided by the entity. |
| schema | OpenAPI Schema | C | The schema definition of the response. Condition: Only provided if the query parameter include-schema is true. |

Table 83	ConfigurationMetaData type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Unique identifier of a configuration data |
| name | string | M | Name of the configuration data |
| translation_id | string | O | Identifier for translating the configurations data name |
| type | ConfigurationT ype | M | Configuration type definition |
| version | string | C1 | Version of the configuration data |
| content_type | string | C1 | MIME type with information for the SOVD client about the content type, e.g., XML etc. |

Table 84	ConfigurationType

| Value | Description |
| --- | --- |
| bulk | The configuration data is provided as bulk data and the attributes version and content_type define how it is interpreted. |
| parameter | The configuration is provided as single attributes of a JSON object. |

- C1: version and content_type are mandatory for bulk data configurations but are not present for parameter-based configurations.

Example

Request:

GET {base_uri}/apps/AdvancedLaneKeeping/configurations HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id": "ALKConfig",

"name": "Configuration for the Advanced Lane Keeping System", "type": "parameter",

},

{

"id": "ObjectRecognitionModel",

"name": "Model of objects to be recognized", "type": "bulk",

"version": "1.45.2107",

"content_type": "application/octet-stream"

}

]

}

    - Read Configuration

      - Introduction

The SOVD API provides a method for reading configuration data from an entity. A configuration can either be read as bulk data or as parameters. Either way, a configuration is always read as a whole, since it is a single resource. E.g., it is not possible to limit the access to certain configuration parameters of a resource. Available configuration-ids are requested with Retrieve List of All Configurations Provided by the Entity.

Method:

GET /{entity-path}/configurations/{configuration-id}

Method Description:

This method is used to read a configuration from an entity, which can either be

        - bulk data or

        - parameter data (MIME type: application/json)

The SOVD standard follows the server-driven content negotiation where the client defines its preferences using the Accept request header. If no Accept request header is provided, the SOVD server returns a default representation.

Path Parameters:

Table 85	Path Parameters - Read configuration

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| configuration-id | string | M | The identifier for the configuration to be read. |

      - Read Configuration as Bulk Data

This method only specifies the interface for reading bulk data, i.e., the requested MIME type is not application/json. It does not imply a certain implementation. Bulk data typically uses the Content-Type application/octet-stream. However, an SOVD client or SOVD server may always use a more specific MIME type than application/octet-stream if applicable.

The method can use the existing bulk data handling mechanism as described in subchapter 6.13.

Query Parameters:

This method does not support query parameters.

Request Headers:

Accept: application/octet-stream (or more specific)

Response Status Codes:

Table 86	Response Status Codes - Read Configuration as Bulk Data

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Response Body | The request was successful. |
| 406 | -- | The configuration cannot be provided in the requested MIME type. |

Response Body:

The content of the response body is OEM specific. Depending on the implementation, the transferred bulk data can use multipart/signed, multipart/form-data with security parameters in a JSON object, or simply application/octet-stream (if it is not confidential information).

Example

Request:

GET

{base_uri}/apps/AdvancedLaneKeeping/configurations/ObjectRecognitionMo del HTTP/1.1

Response:

HTTP/1.1 200 OK

Content-Type: multipart/form-data; boundary=formdataboundary

--formdataboundary

Content-Disposition: form-data; name="signature" Content-Type: application/json

{

"signature": "84e0982a082024fac464bc5eb69c0745",

}

--formdataboundary

Content-Disposition: form-data; name="ObjectRecognitionModel"; Content-Type: application/octet-stream [4D61792074686520666F726365206265207769746820796F75210D0A]

--formdataboundary

      - Read Configuration as Parameters

This method reads the configuration as parameter values from an entity, e.g., reading configuration of an entity as a list of key-value pairs (APP settings) or ECU config files.

Query Parameters:

Table 87	Query Parameters - Read configuration as parameters

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. |

|  |  |  | The default is false. |
| --- | --- | --- | --- |

Request Headers:

The SOVD server expects that the Accept header requests the Content-Type application/json.

Response Status Codes:

Table 88	Response Status Codes - Read configuration as parameters

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | ReadValue | The request was successful. |
| 406 | -- | The configuration cannot be provided as parameter data, i.e., the configuration cannot be provided as JSON. |

Response Body:

The Response Body contains the object of type ReadValue.

Example

Request:

GET {base_uri}/ apps/AdvancedLaneKeeping/configurations/ALKConfig?include-schema=true Accept: application/json HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"id": "ALKConfig", "data": {

"MaximumSpeed": 150,

"MinimumDistanceToLine": 15.0,

"EmergencyLaneDistanceToLine": 5.0

},

"schema": { "type": "object", "properties": {

"MaximumSpeed": { "type": "integer",

"format": "int32", "minimum": 0,

"maximum": 250, "x-sovd-unit": {

"display_name": "km/h", "physical_dimension":

{

"length": 1,

"time": -1

}

}

},

"MinimumDistanceToLine": {

"type": "number",

"format": "float",

"minimum": 10.0,

"maximum": 20.0,

"x-sovd-unit": { "display_name": "cm", "physical_dimension":

{

"length": 1

}

}

},

"EmergencyLaneDistanceToLine": { "type": "number",

"format": "float",

"minimum": 5.0,

"maximum": 10.0,

"x-sovd-unit": { "display_name": "cm", "physical_dimension":

{

"length": 1

}

}

}

}

}

}

    - Write Configuration

      - Introduction

The SOVD API provides a method for writing configuration data to an entity. A configuration can either be written as bulk data or as parameters. Either way, a configuration is always written as a whole, since it is a single resource. E.g., it is not possible to write certain configuration parameters individually without providing the values for the other configuration parameters of the resource as well.

Method:

PUT /{entity-path}/configurations/{configuration-id}

Method Description:

This method is used to write a configuration to an entity, which can either be

        - bulk data, or

        - parameter data (MIME type: application/json)

The SOVD client shall tell the SOVD server in the Content-Type header, which MIME type is used for the data in the request body.

Path Parameters:

Table 89	Path Parameters - Write configuration

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| configuration-id | string | M | The identifier for the configuration to be written |

      - Write Configuration as Bulk Data

This method only specifies the interface for writing bulk data, i.e., the Content-Type is not application/json. It does not imply a certain implementation. However, it can make use of the existing bulk data handling mechanism as described in subchapter 6.13.

Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: application/octet-stream (or more specific)

Request Body:

The content of the request body is OEM specific. Depending on the implementation, the transferred bulk data can use multipart/signed, multipart/form-data with security parameters in a JSON object, or simply application/octet-stream (if it is not confidential information).

Response Status Codes:

Table 90	Response Status Codes - Write Configuration as Bulk Data

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | Configuration written |
| 409 | GenericError | Unable to write configuration due to unsatisfied precondition. E.g., after satisfying precondition “vehicle not in motion” this should be possible. |

Response Body:

This method does not provide a response body.

Example

Request:

PUT {base_uri}/apps/AdvancedLaneKeeping/configurations/ ObjectRecognitionModel HTTP/1.1

Content-Type: multipart/form-data; boundary=formdataboundary

--formdataboundary

Content-Disposition: form-data; name="signature" Content-Type: application/json

{

"signature": "84e0982a082024fac464bc5eb69c0745",

}

--formdataboundary

Content-Disposition: form-data; name="ObjectRecognitionModel"; Content-Type: application/octet-stream [4CD563172A936520162652AA0CC6C9746D82020F7746865206A66F2726F7B96F7521]

--formdataboundary

Response:

HTTP/1.1 204 No Content

      - Write Configuration as Parameters

This method writes the configuration as parameter values to an entity. Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: application/json

Request Body:

Table 91	Request Body - Write configuration as parameters

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| data | AnyValue | M | The value of the data. The type of the value is defined by the resource. |
| signature | string | O | Binary string with signature information (if required by the SOVD server / entity) |

Examples:

- Writing configuration of an entity as a list of key-value pairs (APP settings)

- ECU config files

Response Status Codes:

Table 92	Response Status Codes - Write configuration as parameters

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | Configuration written |
| 400 | DataError | The request body does either not contain all required parameter values or the signature for the provided values is wrong. |
| 409 | GenericError | Unable to write configuration due to unsatisfied precondition. E.g., after satisfying precondition “vehicle not in motion” this should be possible |

Response Body:

This method does not provide a response body.

Example:

Request:

PUT {base_uri}/apps/AdvancedLaneKeeping/configurations/ALKConfig Content-Type: application/json HTTP/1.1

{

"data": { "MaximumSpeed": 100,

"MinimumDistanceToLine": 10.0,

"EmergencyLaneDistanceToLine": 5.0

},

"signature": "cz71a129fez26sw8"

}

Response:

HTTP/1.1 204 No Content

  - API Methods for Control of Operations

    - Introduction

An operation resource represents any executable diagnostic object, e.g., I/O controls, routines, and software functions. The SOVD API distinguishes between two different ways of executing an operation:

- Synchronous execution: The API method for executing the operation returns after the operation has been completed. The response includes the operation result.

- Asynchronous execution: The API method for executing the operation returns immediately and a client has to query the status of the execution which is provided as a temporary resource. In addition, the temporary resource can be used for updating the execution of the resource using a PUT method as long as the temporary resource exists. The temporary resource has to be removed explicitly using a DELETE method.

    - Ensuring Proximity Diagnostics

Certain operations should only be executed if their execution is supervised by a technician, e.g., to ensure that neither the vehicle nor a person takes damage. These operations require a proof of proximity which is performed by a challenge-response procedure. The implementation of the challenge-response mechanism is OEM-specific and not part of the SOVD specification.

There might be different levels of confirmation. E.g., for a remote control, the proximity confirmation only confirms that someone is within proximity. This does not necessarily have to be the one controlling the vehicle.

Examples for proximity confirmation could be:

- Request to press a specific button in the vehicle,

- Request to confirm on the screen in the vehicle,

- Scan a QR code displayed in the vehicle

    - Retrieve List of All Available Operations from an Entity

Method:

GET /{entity-path}/operations

Method Description:

This method returns all available operations defined for the given entity.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Table 93	Query Parameters - Retrieve list of all available operations from an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-proximity-proof | boolean | O | Specifies whether operations which require a proximity proof are returned or not. The default is true. |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 94	Response Status Codes - Retrieve list of all available operations from an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 95 | The request was successful. |

Response Body:

Table 95	Response Body - Retrieve list of all available operations from an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | OperationDescription[] | M | The list of all operations of the entity. |
| schema | OpenAPI Schema | C | The schema definition of the response. Condition: Only provided if the query parameter include-schema is true. |

Table 96	OperationDescription type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Unique identifier for an operation |
| name | string | O | Operation name |
| translation_id | string | O | Identifier for translating the operations name |
| proximity_ proof_required | boolean | M | If true, the execution of the operation requires a proof of proximity, and the offline capability description includes the attribute x-sovd-proximity-proof-required in the PathItem object. An SOVD client can use this information to filter operations in advance if a proximity proof cannot be provided. |
| asynchronous_ execution | boolean | M | If true, the execution of the operation is asynchronous, and the offline capability description includes the attribute x-sovd-asynchronous-execution with the value true in the PathItem object. |

Example:

Request:

GET {base_uri}/components/PowerSteering/operations HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id": "SteeringAngleControl",

"name": "Control the steering angle value", "translation_id": "tid1928653", "proximity_proof_required": false, "asynchronous_execution": true

}

]

}

    - Get Details of a Single Operation

Method:

GET /{entity-path}/operations/{operation-id}

Method Description:

This method returns all the details on the specified operation of the given entity.

Path Parameters:

Table 97	Path Parameters - Get details of a single operation

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| operation-id | string | M | Identifier for the operation |

Query Parameters:

Table 98	Query Parameters - Get details of a single operation

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 99	Response Status Codes - Get details of single operation

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 100 | The request was successful. |

Response Body:

Table 100	Response Body - Get details of single operation

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| item | OperationDescription | M | Description of the operation |
| proximity_ challenge | ProximityChallenge | C | If proximity_proof_requir ed is true, the challenge is provided. The challenge is used for proving proximity to the vehicle. |
| modes | Map of AnyValue[] (defined as additionalPrope rties see [25]) | C | The modes in which the entity shall be for executing the operation. An SOVD client shall set the entity into one of the listed modes before executing the operation. If the attribute is not present, the SOVD server does not provide the information. If it is empty, the operation can be executed in any mode. |
| schema | PathItem Object | C | Complete description of the capabilities including the operation parameters of the operation as an OpenAPI specification. Condition: Only provided if the query parameter include-schema is true. |

Table 101	ProximityChallenge type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| challenge | string | M | Challenge which has to be solved by the SOVD client to prove proximity (see 6.9.2). |
| valid_until | string:date -time | M | The time until which the challenge is valid |

The potential capabilities of an operation are defined as:

- execute: Starts the operation or adjusts an I/O control. The initial operation is executed using the POST method which creates an execution Identifier (see following subchapters). Subsequent adjustments are performed using a PUT method as the value should only be changed and no new execution should be created.

- stop: Stops the operation or returns the control over the operation to the entity.

- freeze: Freezes the current state of the executed I/O control (equivalent of UDS freeze current state).

- reset: Requests that the I/O control is reset to its initial state.

- status: Report the current status of the operation. The status includes if the operation is running or not, and additionally any values describing the current status of the operation in more detail (e.g., execution progress).

Table 102 shows how the mapping of capabilities to the I/O controls, ECU routines, and HPC operations is done.

Table 102	Operation capabilities relations to SOVD methods

| Capability | I/O control | ECU routine | HPC operation | Method |
| --- | --- | --- | --- | --- |
| execute | Short term adjustment | Start | Start | POST (see 6.9.5) |
| stop | Return control to ECU | Stop | Stop | DELETE (see 6.9.8) |
| execute | Short term adjustment | -- | -- | PUT (see 6.9.9) |
| freeze | Freeze current state | -- | -- | PUT (see 6.9.9) |
| reset | Reset to default | -- | -- | PUT (see 6.9.9) |
| status | Report current state | Request results | Status | GET (see 6.9.7) |

Note: All capabilities except for the POST method are only available for operations which have the attribute asynchronous_execution set to true.

An OEM may define additional capabilities to support specific use cases.

The different capabilities are represented by using a PathItem object from the OpenAPI specification, which defines all the possible methods (GET, POST, PUT, and DELETE) which can be executed. If a capability is not supported, the corresponding method is not included.

As the PUT method is used to express multiple capabilities the attribute x-sovd-capabilities is introduced for an operation (OpenAPI Operation object). This attribute specifies as an array which capabilities are supported. The type of x-sovd-capabilites is an enumeration with the values in the column “Capability” from the table above.

Note: Unlike UDS, measurements which represent the current state (e.g., window position) of an I/O control are not returned as part of the I/O control as there is no standardized way of how an SOVD server can identify this. Thus, the measurements are included in the

data collection. An OEM specific extension can be used to associate the I/O control with the corresponding data resource.

Example:

Request:

GET {base_uri}/components/PowerSteering/operations/ SteeringAngleControl HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"item": {

"id": "SteeringAngleControl",

"name": "Control the steering angle value", "translation_id": "tid1928653", "proximity_proof_required": false, "asynchronous_execution": true

},

"modes": {

"session": "EXTENDED"

}

}

    - Start Execution of an Operation

Method:

POST /{entity-path}/operations/{operation-id}/executions

Method Description:

An operation may support multiple executions in parallel. If this is not supported, this method will return an error if an operation with the same operation-id is already being executed.

A successfully started operation is added to the executions collection if it is executed asynchronously. After the asynchronous execution has been finished, the execution resource remains existent and an SOVD client can receive the status. However, the status of the execution cannot be queried indefinitely as an SOVD server implementation may remove the resource, e.g., upon SOVD server restart.

Path Parameters:

Table 103	Path Parameter - Start execution of an operation

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| operation-id | string | M | Identifier for the operation |

Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: application/json

Request Body:

Table 104	Request Body - Start execution of an operation

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| timeout | integer | O | Timeout after which the SOVD server should terminate the execution of the operation by using the capability stop. The time is specified in seconds. For extending the timeout a new control request for the operation has to be submitted by the SOVD client. |
| parameters | AnyValue | C | Parameters for executing the operation. |
| proximity_ response | string | C | Response to proximity challenge if required by the operation. |

Response Header:

The response for status code 202 includes the following header to redirect the SOVD client for requesting the status:

Location:	{base_uri}/{entity-path}/operations/{operation-id}/executions/{execution-id}

Response Status Codes

When an operation is started (capability execute) the status code indicates how the SOVD client should handle the operation.

Table 105	Response Status Codes - Start execution of an operation

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 106 | The operation returned immediately, and the response body contains the final result of the operation (“synchronous execution”). |
| 202 | Table 107 | The execution of the operation has been triggered by the SOVD server. The successful response code does not indicate that the operation has been started successfully. An SOVD client would then use the method for retrieving the status (see 6.9.7) to request the status of this particular execution instance of the operation (“asynchronous execution”). |
| 409 | GenericError | The method is currently not available, e.g., because the operation is still executing. |

Note: The SOVD server or its underlying components might have a timeout implemented after which the actuation is terminated (e.g., for component protection purposes). If necessary, the behavior can be monitored by requesting the execution status.

Response Body:

Table 106	Response Body - Start execution of an operation - synchronous execution

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| parameters | object | O | Response parameters of the operation depending on the capability. |
| error | GenericError | C | Condition: Set if an error occurred when the operation is executed. |

Table 107	Response Body - Start execution of an operation - asynchronous execution

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Identifier for monitoring the execution of the operation and performing additional operations on the execution instance (e.g., freeze). |
| status | ExecutionStatus | O | Status of the executed operation |

Table 108	ExecutionStatus type

| Value | Description |
| --- | --- |
| running | The execution of an operation is running. |
| completed | The execution of an operation has completed successfully. |
| failed | The execution of an operation has terminated with a fault. |

Example:

Request:

POST

{base_uri}/components/PowerSteering/operations/SteeringAngleControl/ex ecutions HTTP/1.1

{

"timeout": 120, "parameters": {

"control-type": "absolute", "angle": 180.0

}

}

Response:

HTTP/1.1 202 Accepted Location:

{base_uri}/components/PowerSteering/operations/SteeringAngleControl/ executions/fd34f39d-06e7-494b-af2d-8928e1458fb0

{

"id": "fd34f39d-06e7-494b-af2d-8928e1458fb0", "status": "running"

}

Note: For a complete operation sequence example, please refer to A.4.2.

    - Get Executions of an Operation

Method:

GET /{entity-path}/operations/{operation-id}/executions

Method Description:

This method returns currently existing executions of the given operation.

Path Parameters:

Table 109	Path Parameters - Get executions of an operation

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| operation-id | string | M | Identifier for the operation |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 110	Response Status Codes - Get executions of an operation

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 111 | The request was successful. |

Response Body:

Table 111	Response Body - Get executions of an operation

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | string[] | M | List of identifiers of all execution instances of the operation |

Example:

Request:

GET

{base_uri}/components/PowerSteering/operations/SteeringAngleControl/ex ecutions HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id": "fd34f39d-06e7-494b-af2d-8928e1458fb0"

}

]

}

    - Get the Status of an Operation Execution

Method:

GET /{entity-path}/operations/{operation-id}/executions/{execution-id}

Method Description:

This method returns the current status of the operation execution.

Path Parameters:

Table 112	Path Parameters - Get the status of an operation execution

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| operation-id | string | M | Identifier for the operation |
| execution-id | string | M | Identifier for monitoring the execution of the operation. |

Query Parameters:

Table 113	Query Parameters - Get the status of an operation execution

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 114	Response Status Codes - Get the status of an operation execution

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 115 | The request was successful. |

Response Body:

Table 115	Response Body - Get the status of an operation execution

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| status | ExecutionStatus | M | Status of the executed operation. |
| capability | Capability | M | Capability executed at the moment. This is either execute or the capability which has been used with a PUT request (see 6.9.5 and 6.9.9). |
| parameters | AnyValue | C | Response parameters of the operation depending on the capability. The type of the value is defined by the schema. Condition: If the capability provides response parameters. |
| schema | OpenAPI Schema | C | Schema of the operation executions. Condition: Only provided if the query parameter include-schema is true. |
| error | DataError[] | C | Array containing detailed descriptions about occurred errors during execution of the operation. Condition: Set if an error occurred when the operation is executed. The path attribute references erroneous parameters of the operation execution. If the path attribute is “/” then the operation itself failed and the error describes the reason for the failure. |

Example:

Request:

GET

{base_uri}/components/PowerSteering/operations/SteeringAngleControl/ executions/fd34f39d-06e7-494b-af2d-8928e1458fb0 HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"status": "running", "capability": "execute", "parameters": {

"control-type": "absolute", "angle": 120.3

}

}

    - Stop the Execution of an Operation

Method:

DELETE /{entity-path}/operations/{operation-id}/executions/

{execution-id}

Method Description:

This method terminates the execution of the operation and removes it. The status of the operation can no longer be requested.

Path Parameters:

Table 116	Path Parameters - Stop the execution of an operation

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| operation-id | string | M | Identifier for the operation |
| execution-id | string | M | Identifier for monitoring the execution of the operation. |

Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: application/json

Request Body:

Table 117	Request Body - Stop the execution of an operation

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| proximity_ response | string | C | Response to proximity challenge if required by the operation. |

Response Status Codes:

Table 118	Response Status Codes - Stop the execution of an operation

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | The request was successful. |

Response Body:

This method does not provide a response body.

Example:

Request:

DELETE

{base_uri}/components/PowerSteering/operations/SteeringAngleControl/ executions/fd34f39d-06e7-494b-af2d-8928e1458fb0 HTTP/1.1

Response:

HTTP/1.1 204 No Content

    - Support for Execute / Freeze / Reset and OEM-Specific Capabilities

Method:

PUT /{entity-path}/operations/{operation-id}/executions/{execution-id}

Method Description:

This method executes the given capability on the provided operation execution.

Path Parameters:

Table 119	Path Parameters - Support for execute / freeze / reset and OEM-specific capabilities

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| operation-id | string | M | Identifier for the operation |
| execution-id | string | M | Execution Identifier of the operation |

Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: application/json

Request Body:

Table 120	Request Body - Support for execute / freeze / reset and OEM-specific capabilities

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| capability | Capability | M | Capability to be executed (one of execute, freeze, or reset as well as the OEM specific capabilities. |
| timeout | integer | O | Timeout after which the SOVD server should terminate the execution of the operation by using the capability stop. The time is specified in seconds. For extending the timeout a new control request for the operation has to be submitted by the SOVD client. |
| parameters | AnyValue | C | Parameters for executing the operation. |
| proximity_ response | string | C | Response to proximity challenge if required by the operation. |

Response Header:

The response for status code 202 includes the Location header to redirect the SOVD client for requesting the status:

Location:	{base_uri}/{entity-path}/operations/{operation-id}/executions/{execution-id}

Response Status Codes:

Table 121	Response Status Codes - Support for execute / freeze / reset and OEM-specific capabilities

| Status Code | Response Body | Description |
| --- | --- | --- |
| 202 | Table 107 | The execution of the capability has been triggered by the SOVD server. The successful response code does not indicate that the operation has executed the capability successfully. An SOVD client would then use the method for retrieving the status (see 6.9.7) to request the |

|  |  | status of this particular execution instance of the operation. |
| --- | --- | --- |

Response Body:

See Table 107.

Example:

Request:

PUT

{base_uri}/components/PowerSteering/operations/SteeringAngleControl/ executions/fd34f39d-06e7-494b-af2d-8928e1458fb0 HTTP/1.1

{

"capability": "reset", "timeout": 60

}

Response:

HTTP/1.1 202 Accepted Location:

{base_uri}/components/PowerSteering/operations/SteeringAngleControl/ executions/fd34f39d-06e7-494b-af2d-8928e1458fb0

{

"id": "fd34f39d-06e7-494b-af2d-8928e1458fb0", "status": "running"

}

  - API Methods for Support of Target Modes

    - Introduction

As a prerequisite for specific interactions with an entity, e.g., the execution of operations or changing data/configurations of an entity, the entity may have to be in a specific state in order to successfully process related requests. To enable SOVD clients to explicitly control and prepare such states of target entities (i.e., Components, Apps), the SOVD API provides information about current and possible states of target entities and supports any triggering required for state transitions. The state is maintained on the vehicle side. A concrete example in the CDA context is setting an ECU to a specific diagnostic session as a prerequisite to execute its operations. By explicitly setting the state, the SOVD client can control stateful hardware which is required by scenarios where a mode cannot be determined automatically, or performance is crucial. In the following, controllable states of target entities are called “modes”. A common basis is specified on how such modes can be served and controlled through an SOVD server via the SOVD API.

In an HPC context modes might be useful, e.g., for controlling that an input receives its value from a different source such as a simulation environment.

OEMs can use this as a basis to expose and interact with their introduced modes via the SOVD API.

The SOVD API supports two different ways of specifying which mode is required for the communication with an entity:

- Explicit mode control: The SOVD client defines which modes shall be activated prior to the API method call by the entity.

- Hint-based mode control: The SOVD client includes the modes which should be activated by the entity in the API method call.

    - Retrieve List of All Supported Modes of an Entity

Method:

GET /{entity-path}/modes

Method Description:

This method returns all available modes defined for the given entity.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Table 122	Query Parameters - Retrieve list of all supported modes of an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 123	Response Status Codes - Retrieve list of all supported modes of an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 124 | The request was successful. |

Response Body:

Table 124	Response Body - Retrieve list of all supported modes of an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | ModeCollectionItem[] | M | The list of available modes of an entity. |
| schema | OpenAPI Schema | C | The schema definition of the response. Condition: Only provided if the query parameter include-schema is true. |

Table 125	ModeCollectionItem type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | The resource identifier of a mode of an entity. |
| name | string | O | The name of a mode of an entity. |
| translat ion_id | string | O | Translation identifier for the name |

Example:

Request:

GET {base_uri}/components/PowerSteering/modes HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id": "session",

"name": "Diagnostic session"

}

]

}

    - Get Details of a Single Mode of an Entity

Method:

GET /{entity-path}/modes/{mode-id}

Method Description:

This method returns all the details on the specified mode of the given entity.

Path Parameters:

Table 126	Path Parameters - Get details of a single mode of an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| mode-id | string | M | The identifier of a mode defined for an entity. |

Query Parameters:

Table 127	Query Parameters - Get details of a single mode of an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 128	Response Status Codes - Get details of a single mode of an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 129 | The request was successful. |

Response Body:

Table 129	Response Body - Get details of a single mode of an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| name | string | O | The name of the mode. |
| translation_ id | string | O | The identifier for translating the name. |
| value | AnyValue | M | The value of the mode. |
| schema | OpenAPI Schema | O | The schema of the mode resource. The default value for the mode is defined by the attribute default. Condition: Only provided if the query parameter include-schema is true. |

Example:

Request:

GET {base_uri}/components/PowerSteering/modes/session?include-schema=true HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"name": "Diagnostic session",

"value": "DEFAULT",

"schema": { "type": "object", "required": [

"id",

"name", "value"

],

"properties": { "id": {

"type": "string"

},

"name": {

"type": "string"

},

"value": {

"type": "string", "enum": [

"DEFAULT", "EXTENDED", "PROGRAMMING"

],

"default": "DEFAULT"

}

}

}

}

    - Explicit Control of Entity States via Their Defined Modes

Method:

PUT /{entity-path}/modes/{mode-id}

Method Description:

The method allows to control the state of the specified mode of the given entity explicitly.

Path Parameters:

Table 130	Path Parameters - Explicit control of entity states via their defined modes

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| mode-id | string | M | The resource identifier of the mode of an entity. |

Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: application/json

Request Body:

Table 131	Request Body - Explicit control of entity states via their defined modes

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| value | AnyValue | M | The value to be set for the specified mode. |
| mode_ expiration | integer | M | Defines after how many seconds the mode expires and should therefore be automatically reset to the mode’s default value. The capability description shall define the maximum expiration time and the default value of the mode, using the attribute default. |

Response Status Codes

Table 132	Response Status Codes - Explicit control of entity states via their defined modes

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 133 | Updated mode resource. |
| 500 | GenericError | The SOVD server received an error/negative response from the target entity when trying to set the mode to the provided value. |

Response Body:

Table 133	Response Body - Explicit control of entity states via their defined modes

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | The resource identifier of the mode. |
| value | AnyValue | M | The value of the mode. |

Example:

Request:

PUT {base_uri}/components/PowerSteering/modes/session HTTP/1.1

{

"value": "EXTENDED",

"mode_expiration": 1200

}

Response:

HTTP/1.1 200 OK

{

"id": "session", "value": "EXTENDED"

}

    - Hint-Based Control of Entity States via Their Defined Modes

An SOVD client may provide a hint to the SOVD server by providing a mode with any request as a request header:

X-SOVD-Mode: {mode-id};{mode-value}

Multiple modes are set in the header by using a comma. While processing the request the SOVD server shall set the entity to the mode given in the header.

  - API Methods for Locking

    - Introduction

The execution of some operations of an entity may require a lock so that the execution of this operation does not interfere with the execution of other operations, or the same operation performed by a different SOVD client. For example, the execution of an I/O control of a classic ECU would interfere with the execution of the same I/O control by a different SOVD client.

The standard provides the support for locking an entity by an SOVD client. The SOVD client then has exclusive access to all resources of the entity which require a lock. Only one lock per entity is allowed.

If one entity is locked, the sub entities below are also in the scope of this lock. It is not possible to lock those sub-entities individually in this case. It is also not possible to lock the entities above in this case. This means that there can be only one lock along the path to the entity.

The online and offline capability description describe that a lock is required for accessing a particular resource by introducing the attribute x-sovd-lock-required. This attribute is defined in the scope of an Operation or Path Item Object of the OpenAPI specification:

paths:

/components/PowerSteering/operations/SteeringAngleControl/executions post:

operationId: "SteeringAngleControl" x-sovd-lock-required: true

The attribute is specified for the Path Item if any action (HTTP method) on the resource requires a lock. If the attribute is specified for a specific action (e. g. POST) on the resource, then the lock is only required for performing this particular method. For example, for the execution of an operation the temporary resource /{entity-path}/operations/{operation-id}/executions/{execution-id} is used. For a given execution of an operation, querying the status with the GET method may not require a lock but changing the execution with a PUT method could require a lock.

Table 134	x-sovd-lock-required attribute

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| x-sovd-lock-required | boolean | O | If true performing an action on the resource or a particular HTTP method requires a lock of the entity. By default, the attribute has the value false. |

The SOVD server may associate the acquired lock with the authorization token of an SOVD client in order to identify the ownership of the lock.

All resources which require a lock can only be accessed by an SOVD client if this SOVD client has acquired a lock before. If an SOVD client tries to access a resource which requires a lock without having locked the entity previously, the HTTP status code 409 (Conflicted) will be sent as a response. This status code is also used when an SOVD client tries to acquire a lock and the request cannot be fulfilled because the lock is already acquired by a different SOVD client. Resources which can be accessed without requiring a lock shall still be accessible by SOVD clients when a lock is set for an entity. For example, if the faults resource collection defines that the GET method can be accessed without a lock, it can still be accessed by another client when the whole entity is locked.

The SOVD Client defines an expiration time for each lock it acquires and is able to extend that time if required. However, the operation – e. g. an I/O control execution or a software update – for which a lock has been acquired may run longer than the time for which the lock has been acquired. Once the lock times out, the SOVD server will terminate all resources – for example an operation – which is associated with the expired lock. This implies, that the lock resource as well as temporary resources – for example an execution resource – are deleted and further access to them by an SOVD client results in 404 (not found) status code. An SOVD client can only acquire the lock after the associated resources have been terminated.

    - Acquire a Lock on an Entity

Method:

POST /{entity-path}/locks

Method description:

This method acquires a lock from the SOVD server for the given entity to reserve its resources for exclusive access of a single SOVD client, i.e., prevent other SOVD clients from operating on the resources in parallel. Any further requests against the SOVD API towards controlling an operation (actuators, routines, and input control), e.g., start the execution of an operation, is rejected by the SOVD server if the required lock is not acquired by the same SOVD client.

Once the lock is not required anymore, the SOVD client can delete the created lock resource by a corresponding DELETE request.

In addition, an SOVD client shall provide a relative expiration time after which the SOVD server should release the lock automatically. As a result, the SOVD server deletes the corresponding lock resource. The SOVD server might reject the requested expiration time if it is too long.

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Body:

Table 135	Request Body - Acquire a lock on an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| lock_ expiration | integer | M | Defines after how many seconds the lock expires and should therefore be automatically released by the SOVD server. The expiration time starts when the request is processed by the server. The capability description shall define the maximum expiration time. |

Response Header:

The Location header redirects the SOVD client for requesting the status:

Location: {base_uri}/{entity-path}/locks/{lock-id} Response Status Codes

Table 136	Response Status Codes - Acquire a lock on an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 201 | Table 137 | The SOVD server has successfully created a new lock resource and acquired a lock on all relevant resources. |
| 409 | GenericError | The SOVD server rejected the creation of the lock resource. |

Response Body:

Table 137	Response Body - Acquire a lock on an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | The identifier of the created lock resource. |

Example:

Request:

POST {base_uri}/components/PowerSteering/locks HTTP/1.1

{

"lock_expiration": 3600

}

Response:

HTTP/1.1 201 Created

Location: {base_uri}/components/PowerSteering/locks/15c3e824-8278-44e1-b0b2-

3bc16e4522f2

{

"id": "15c3e824-8278-44e1-b0b2-3bc16e4522f2"

}

    - Get All Acquired Locks of an Entity

Method:

GET /{entity-path}/locks

Method description:

The method returns the acquired locks for the given entity.

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Response Headers:

The method has no response headers.

Response Status Codes:

Table 138	Response Status Codes - Get all acquired locks of an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 139 | The request was successful. |

Response Body:

Table 139	Response Body - Get all acquired locks of an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | LockInfo[] | M | Collection of Locks acquired for the entity. Note: In this version of the standard only one lock is |

|  |  |  | possible for the whole entity. |
| --- | --- | --- | --- |

Table 140	LockInfo type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Identifier of the lock. |
| owned | boolean | M | If true, the SOVD client which performed the request owns the lock. The value is always false if the entity is not locked. |

Example:

Request:

GET {base_uri}/components/PowerSteering/locks HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items":[

{

"id": "15c3e824-8278-44e1-b0b2-3bc16e4522f2",

"owned": true

}

]

}

    - Get a Single Active Lock of an Entity

Method:

GET /{entity-path}/locks/{lock-id}

Method description:

The method returns all the details on the specified lock of the given entity. For extending the expiration of a lock the method described in 6.11.5 has to be invoked.

Path Parameters:

Table 141	Path Parameters - Get a single active lock of an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| lock-id | string | M | The resource identifier of the lock resource to be requested. |

Query Parameters:

This method does not support query parameters.

Response Status Codes:

Table 142	Response Status Codes - Get a single active lock of an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 143 | The request was successful. |

Response Body:

Table 143	Response Body - Get a single active lock of an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| lock_ expiration | string:date -time | M | A date-time string ([21], [19]), that defines at which point in time the lock expires and would therefore be automatically released by the SOVD server. The SOVD server always returns an absolute date and time in UTC based on the relative time provided by the SOVD client upon lock creation. |

Example:

Request:

GET {base_uri}/components/PowerSteering/locks/15c3e824-8278-44e1-b0b2-3bc16e4522f2 HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"lock_expiration": "2021-06-22T11:47:22Z"

}

    - Modify the Expiration Time of an Acquired Lock on an Entity

Method:

PUT /{entity-path}/locks/{lock-id}

Method description:

This method modifies an acquired lock by providing a new expiration time. The modification is only valid for the SOVD client that has already acquired the corresponding lock.

Path Parameters:

Table 144	Path Parameters - Modify the expiration time of an acquired lock on an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| lock-id | string | M | Identifier of a previous lock owned by the SOVD client. |

Query Parameters:

This method does not support query parameters.

Request Headers:

The method has no special request headers.

Request Body:

Table 145	Request Body - Modify the expiration time of an acquired lock on an entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| lock_ expiration | integer | M | Defines after how many seconds the lock expires and should therefore be automatically released by the SOVD server. The expiration time starts when the request is processed by the server and is not added to the original expiration time. |

Response Status Codes

Table 146	Response Status Codes - Modify the expiration time of an acquired lock on an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | The request was successful. |
| 409 | GenericError | The SOVD server rejected the extension of the lock resource. |

Response Body:

The method has no response body.

Example:

Request:

PUT {base_uri}/components/PowerSteering/locks/15c3e824-8278-44e1-b0b2-3bc16e4522f2 HTTP/1.1

Content-Type: application/json

{

"lock_expiration": 1800

}

Response:

HTTP/1.1 204 No Content

    - Release an Acquired Lock on an Entity

Method:

DELETE /{entity-path}/locks/{lock-id}

Method description:

This method releases an acquired lock by deleting the created lock resource.

Request Headers:

The method has no special request headers.

Path Parameters:

Table 147	Path Parameters - Release an acquired lock on an entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| lock-id | string | M | The resource identifier of the lock resource to be deleted. |

Query Parameters:

This method does not support query parameters.

Response Status Codes

Table 148	Response Status Codes - Release an acquired lock on an entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | The request was successful. |
| 409 | GenericError | The SOVD server rejected the deletion of the lock resource for some reason. |

Response Body:

The method has no response body.

Example:

Request:

DELETE {base_uri}/components/PowerSteering/locks/15c3e824-8278-44e1-b0b2-3bc16e4522f2 HTTP/1.1

Response:

HTTP/1.1 204 No Content

  - API Methods for Software Update

    - Introduction

In the classic approaches the diagnostic tester (as an SOVD client outside of the vehicle) has control over the flash process and the vehicle simply executes the prescribed steps. In contrast to this approach, the SOVD API puts the control into the vehicle and leverages the OTA capabilities of a connected vehicle to retrieve and install the available software updates.

The SOVD API distinguishes between the two update procedures

- stepwise software update and

- automated software update

to accommodate the different software update scenarios. With the stepwise software update the SOVD client can control which update is installed and when the update is prepared and executed. The automated software update puts the SOVD server in control of the complete update process for the update package selected by the client.

The possible update packages are provided by the SOVD API described in 6.12.2. An update package is always self-contained, i.e., if an update requires the update of multiple ECUs (e.g., engine and gear shift) these updates are included in the update package and are installed in one atomic step. If an update package includes multiple entities, these entities shall be located below the given entity path. E.g., if different top-level Components of the vehicle are updated, the common entity path will be the SOVD server entity. As the SOVD API does not restrict which actions can be performed as part of a software update, the update routine may also include parameterization and other activities. An additional consequence of an atomic update is that an update is rolled back completely in case of an error.

Although the SOVD API only defines the resource /updates on an SOVD server, which is used for updating all entities under the control of that SOVD server, an entity may provide this resource as well. If an entity provides the updates resource, specific use cases like updating individual entities in engineering or production can be supported.

      - Update Origins

The retrieval of update packages supports the definition of an origin to specify in which environment or by whom the software update is triggered by the SOVD client.

Table 149	UpdateOrigins type

| Value | Description |
| --- | --- |
| remote | The update is performed over-the-air and no technician intervention is possible. Based on the requested version, authorization, network connection, etc. the SOVD server (or OEM backend) defines which updates are applicable. |
| proximity | The update is performed in a workshop and potentially technician intervention is possible. |

An OEM may define additional origins. These origins shall be prefixed with x-<oem> and can contain alphanumeric characters, hyphens, and underscores.

      - Stepwise Software Update

The software update is divided into the following steps:

        - Step 1: Get applicable updates (see 6.12.2):

The vehicle provides the applicable updates to the SOVD client. The SOVD client then selects which update should be installed. The applicable updates may either by queried from an OTA backend when the command is executed, or the vehicle provides a list of updates which have been pushed to the vehicle by the OTA backend before.

        - Step 2: Prepare update (see 6.12.5):

The update selected by the SOVD client is prepared for installation, by downloading it from the OTA backend, validating its applicability for the vehicle, installing it in B-memory, etc. After this step the update is ready to be installed.

        - Step 3: Execute update (see 6.12.6):

The prepared update is installed and activated. If multiple ECUs are flashed as part of the update, an SOVD server may install these updates in parallel if possible. As part of this step also necessary configurations, setup procedures, … etc. are performed.

Step 1 may be executed independently. This step can be omitted, if the identifier for the update package – which is required in step 2 – can be obtained from a different source. However, steps 2 and 3 are closely related to each other as the preparation step defines which update is installed during the execution step.

      - Automated Software Update

The SOVD API provides a method for initiating the update and monitoring its execution status. An SOVD server is then responsible for ensuring the following aspects:

        - Downloading the update package

        - Installing and activating the update package

      - Autonomous Update Package

The SOVD API defines the special update package identifier autonomous. Autonomous update packages are used in scenarios where a selection of the update package by an SOVD client is not required, e.g., for installing the latest security fixes. This is also known as self-governed update.

The special update package identifier autonomous can only be used to get the concrete update package identifier for installing the update. The update package identifier autonomous cannot be used to install an update.

      - Example

This subchapter provides an overview of the idea behind the mechanisms. It is not intended to be a comprehensive list of the provided methods which are described in detail in the following subchapters.

As depicted in Figure 7, below the collection resource “/updates”, two different kinds of resources can exist: “autonomous” and update resources representing available update

packages, i.e., “update-1” and “update-4”. An update package can contain e.g., one or more updates for ECUs or other entities, files, containers, or any possible combination of them.

Figure 7:	SOVD API sub-tree related to SW updates

As reflected by the HTTP methods attached to the resources shown in Figure 7, the usage of these resources can be as follows:

        - GET {base_uri}/updates delivers a list containing “autonomous”, “update-1” and “update-4”.

        - GET {base_uri}/updates/autonomous reports the update-id of an update package which is identified by the SOVD server to be an installable update at that point in time. With respect to the example shown in Figure 7, the SOVD server may return “update-1” as response.

The update package represented by the returned ID can then be installed either using:

        - PUT	{base_uri}/updates/update-1/prepare,	followed	by	PUT

{base_uri}/updates/update-1/execute (for the stepwise update) or by using

        - PUT {base_uri}/updates/update-4/automated

With this structure, automated updates as well as stepwise updates can be handled using the same resource structure. If the SOVD server respectively the update package supports this, the SOVD client can even choose which method it wants to apply.

    - Retrieve List of All Updates

Method:

GET /updates

Method Description:

The method returns the available updates for the entity.

Path Parameters:

The method does not support path parameters.

Query Parameters:

Table 150	Query Parameters - Retrieve list of all updates provided by the entity

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| target-version | string | O | Defines the version which is requested by the SOVD client. This version can be a specific version or a version pattern. |
| origin | UpdateOrigins | O | Defines from where the update is requested, e.g., inside a workshop or by an OTA update request. Default: remote |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 151	Response Status Codes - Retrieve list of all updates provided by the entity

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 152 | The request was successful. |

Response Body:

Table 152	Response Body - Retrieve list of all updates provided by the entity

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | string[] | M | Update package identifier |

Note: The SOVD standard predefines the special update package id autonomous, which is used for autonomous updates. This update package is included in the response if the autonomous update of the entity is supported.

Example (Remote): Request:

GET {base_uri}/updates HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [ "autonomous", "ADAS-v2.03.2154",

"CAM-v1.18.8643"

]

}

Example (Workshop): Request:

GET {base_uri}/updates?origin=proximity HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

"WC-v4.03.8413"

]

}

    - Get Details of Update

Method:

GET /updates/{update-package-id}

Method Description:

The method returns detailed information for a given update package.

Path Parameters:

Table 153	Path Parameters - Get details of update

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| update-package-id | string | M | Identifier for the update package. |

Query Parameters:

Table 154	Query Parameters - Get details of update

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 155	Response Status Codes - Get details of update

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 156 | The request was successful. |

Response Body:

Table 156	Response Body - Get details of update

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Identifier of the update package. |
| update_name | string | M | Name of the update |
| update_ translation_id | string | O | Translation identifier for the update_name. |
| automated | boolean | O | Specifies whether the update package can be installed automatedly or not. If missing, the attribute defaults to false. |
| origin | UpdateOrigins [] | O | List of origins for which the update package is applicable. If the attribute is left out or empty, any origin is applicable. |
| notes | string | O | Release notes for the updates, may also be a URI. |
| notes_ translation_id | string | O | Translation identifier for the notes. |
| user_activity | string | O | Activities which have to be performed by the user after the update. |
| user_activity_ translation_id | string | O | Translation identifier for the user_activity. |
| preconditions | string | O | Preconditions which shall be fulfilled for installing the update (e.g., doors locked, parking, …). In general, not being able to fulfill the preconditions would block the preparation of the update. |
| preconditions_ translation_id | string | O | Translation identifier for the preconditions. |
| execution_ | string | O | Description of the conditions which are required for executing |

| conditions |  |  | the update. May only be set if status is completed. |
| --- | --- | --- | --- |
| duration | integer | C | Time for which the vehicle will not be available when the update is installed and activated. The time is defined in seconds. If not set, the availability of the vehicle would not be affected. |
| size | integer | M | The download size of the update is defined in kilo bytes. |
| updated_ components | string[]: uri-reference | O | A list of Components to be updated. The URIs are in the form of {entity-path}. |
| affected_ components | string[]: uri-reference | O | The list documents which Components may have a different behavior etc. The focus lies on user-perceived functions and Components. The URIs are in the form of {entity-path}. |
| schema | OpenAPI Schema | C | Schema of the update package. Condition: Only provided if the query parameter include- schema is true. |

Example:

Request:

GET {base_uri}/updates/autonomous HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"id":	"ADAS-v2.03.2154",

"update_name":	"ADAS feature update to version 2.03.2154", "automated":	"true",

"origins":	[ "remote", "proximity" ], "preconditions": "parking",

"duration":	900,

"size":	247000,

"updated_components": [ "{base_uri}/components/PowerSteering", "{base_uri}/apps/AdvancedLaneKeeping"

],

"affected_components": [ "{base_uri}/apps/AdvancedLaneKeeping"

]

}

    - Automated Installation of an Update

Method:

PUT /updates/{update-package-id}/automated

Method Description:

The method starts the automated installation of the update package. For this scenario, the SOVD server and the OEM’s backend have full control over the update process and decide which action can be taken when.

The method returns immediately and redirects the SOVD client to a method for requesting the status and progress of the installation.

Path Parameters:

Table 157	Path Parameters - Automated installation of an update

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| update-package-id | string | M | Identifier for the update package. |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Header:

The response includes the following header to redirect the SOVD client for requesting the status:

Location: {base_uri}/updates/{update-package-id}/status Response Status Codes:

Table 158	Response Status Codes - Automated installation of an update

| Status Code | Response Body | Description |
| --- | --- | --- |
| 202 | --- | If no error is detected immediately (e.g., because the update package does not exist) |
| 409 | GenericError | An update is already in installation and not yet done or aborted. ErrorCode: update-process-in-progress message: ‘An update is already in preparation’ |

|  |  | parameters: - key: id value: ‘Identifier of update’ |
| --- | --- | --- |
| 409 | GenericError | Automated installation of the update package is not supported ErrorCode: update-automated-not-supported message: ‘The package cannot be installed automatically’ |

Response Body:

This method does not provide a response body.

Example:

Request:

PUT {base_uri}/updates/ADAS-v2.03.2154/automated HTTP/1.1

Response:

HTTP/1.1 202 Accepted

Location: {base_uri}/updates/ADAS-v2.03.2154/status

    - Prepare Installation of an Update

Method:

PUT /updates/{update-package-id}/prepare

Method Description:

The method starts the preparation of the update package for installation. As part of this step the following tasks are performed for example:

- Validates if the update package is still installable (e.g., checking the software versions of the affected entities)

- Download of update package from OTA backend

- Validation of the integrity of update package

- Applying deltas

- Installing the update in B-memory

The method returns immediately and redirects the SOVD client to a method for requesting the status and progress of the preparation.

An SOVD server may remove prepared updates after an SOVD server specific amount of time. After a successful installation, the SOVD server will delete the update package automatically. If an update fails, it is in general not implicitly removed as the prepared package may be used for another attempt. An SOVD server may however remove the package if free space is required.

Path Parameters:

Table 159	Path Parameters - Prepare installation of an update

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| update-package-id | string | M | Identifier for the update package. |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Header:

The response includes the Location header to redirect the SOVD client for requesting the status:

Location: {base_uri}/updates/{update-package-id}/status Response Status Codes:

Table 160	Response Status Codes - Prepare installation of an update

| Status Code | Response Body | Description |
| --- | --- | --- |
| 202 | --- | If no error is detected immediately (e.g., because the update package identifier does not exist). |
| 409 | GenericError | An update is already in preparation and not yet done or aborted |

Response Body:

This method does not provide a response body.

Note: The SOVD server may execute no operation at all, and the status of this step is “completed” as the OTA backend already prepared the update.

Example:

Request:

PUT {base_uri}/updates/WC-v4.03.8413/prepare HTTP/1.1

Response (Success):

HTTP/1.1 202 Accepted

Location: {base_uri}/updates/WC-v4.03.8413/status

Response (Error):

HTTP/1.1 409 Conflicted

{

"error_code": "update-preparation-in-progress", "message": "An update is already in preparation", "parameters": {

"id": "Identifier of update"

}

}

    - Execute Installation of an Update

Method:

PUT /updates/{update-package-id}/execute

Method Description:

The method starts the installation and activation of the update package. As part of this step the following tasks are performed for example:

- Validation if the update package is still installable (e.g., checking the software versions of the updated entities)

- Installation of the update on all updated entities

- Configuration of the entities

- Execution of specific post install actions

- Activation of the update (e.g., by switching to B-memory)

- Rollback of update in case of an error

The method returns immediately and redirects the SOVD client to a method for requesting the status and progress of the installation.

After the update has been finished, the resource for the update package (and its sub-resources like status) remains existent and an SOVD client can receive the status of an update package. However, the status of the update package cannot be queried indefinitely as an SOVD server implementation may remove the update package resource, e.g., upon SOVD server restart or the retrieval of new update packages.

Path Parameters:

Table 161	Path Parameters - Execute installation of an update

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| update-package-id | string | M | Identifier for the update package. |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Header:

The response includes the Location header to redirect the SOVD client for requesting the status:

Location: {base_uri}/updates/{update-package-id}/status Response Status Codes:

Table 162	Response Status Codes - Execute installation of an update

| Status Code | Response Body | Description |
| --- | --- | --- |
| 202 | --- | If no error is detected immediately (e.g., because the identifier does not exist). |
| 409 | GenericError | Another update is already executed. ErrorCode: update-execution-in-progess message: ‘An update is already executed’ |

Response Body:

This method does not provide a response body.

Example:

Request:

PUT {base_uri}/updates/WC-v4.03.8413/execute HTTP/1.1

Response:

HTTP/1.1 202 Accepted

Location: {base_uri}/updates/WC-v4.03.8413/status

    - Get Status of an Update

Method:

GET /updates/{update-package-id}/status

Method Description:

The method returns the status of the preparation or execution of the update.

Path Parameters:

Table 163	Path Parameters - Get status of an update

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| update-package-id | string | M | Identifier for tracking the preparation or execution of the update. |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 164	Response Status Codes - Get status of an update

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 165 | The request was successful. |

Response Body:

Table 165	Response Body - Get status of an update

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| phase | Phase | M | Current phase of the update process. |
| status | Status | M | Status of the preparation or execution. |
| progress | integer | O | Progress of the preparation or execution in percent. |
| subprogress | Progress[] | O | Progress of the updated entities. |
| step | string | O | Description of the current step. |
| step_ translation_id | string | O | Translation identifier for the step. |
| error | GenericError | O | Only set if status is failed. |

Table 166	Phase type

| Value | Description |
| --- | --- |
| prepare | Update preparation |
| execute | Update execution |

Table 167	Progress type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| entity | string: uri-reference | M | URI of the entity. |

| status | Status | M | Status of the preparation or execution. |
| --- | --- | --- | --- |
| progress | integer | O | Progress of the preparation or execution in percent. |
| error | GenericError | O | Only set if status is failed. |

Table 168	Status types

| Status | Description |
| --- | --- |
| phase: prepare |  |
| pending | The preparation of the update has not yet started. |
| inProgress | The preparation of the update has started. |
| failed | The preparation of the update has failed. Details on the reason are specified in the error attribute. |
| completed | The preparation of the update has been completed successfully, but the execution of the update has not yet been requested. |
| phase: execute |  |
| pending | The preparation of the update has been completed, but the update itself has not yet been started. |
| inProgress | The installation of the update has started. |
| failed | The installation of the update has failed. Details on the reason are specified in the error attribute. |
| completed | The installation of the update has been completed successfully. |

Note: If the preparation of an update fails, the request for the status would still use the response status code 200, as the status request was answered successfully. The attribute error, however, will contain information about the error that occurred. A response status code of 5xx would only be used if the status request could not be answered by the SOVD server.

Example:

Request:

GET {base_uri}/updates/ADAS-v2.03.2154/status HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"phase": "execute", "status": "inProgress", "progress": 87, "subprogress": [

{

"entity": "{base_uri}/components/PowerSteering", "status": "completed",

"progress": 100

},

{

"entity": "{base_uri}/apps/AdvancedLaneKeeping", "status": "inProgress",

"progress": 75

}

]

}

    - Delete Update Package from an SOVD Server (Optional)

Method:

DELETE /updates/{update-package-id}

Method Description:

The method deletes an update package, which is not in the phase execute. The resource is afterwards not available anymore.

Path Parameters:

Table 169	Path Parameters - Delete update package from an SOVD server

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| update-package-id | string | M | Identifier for the update package created by the SOVD server. |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Status Codes:

Table 170	Response Status Codes - Delete update package from an SOVD server

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | The request was successful. |
| 405 | -- | Deleting is not foreseen for an autonomous update-package-id or update-package is in phase execute. |

Response Body:

This method does not provide a response body.

Example:

Request:

DELETE {base_uri}/updates/WC-v4.03.8413 HTTP/1.1

Response:

HTTP/1.1 204 No Content

    - Register an Update at the SOVD Server

Method:

POST /updates

Method Description:

The method registers a new update at the SOVD server which can be installed afterwards. After creating the update, the SOVD client has to use the methods described before for installing the update (automated or prepare & execute).

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Headers:

Content-Type: OEM-specific

Request Body:

The content of the request body is OEM and SOVD server implementation specific as it provides the necessary information to the SOVD server to create a new update package including the required files.

Response Header:

The response includes the Location header to redirect the SOVD client for interacting with the package:

Location: {base_uri}/updates/{update-package-id} Response Status Codes:

Table 171	Response Status Codes - Register an update at the SOVD server

| Status Code | Response Body | Description |
| --- | --- | --- |
| 201 | Table 172 | The request was successful. |

Response Body:

Table 172	Response Body - Register an update at the SOVD server

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Identifier for the update package created by the SOVD server. |

Example:

Request:

POST {base_uri}/updates HTTP/1.1 Content-Type: application/json

{

"id":	"SW-v2.1.9525",

"source":	"USB",

"filename": "UpdatePackage-V2.1.9525.pkg", "signature": "d9df94cf1f5f5413fa325f8b52bd28a4"

}

Response:

HTTP/1.1 201 Created

Location: {base_uri}/updates/SW-v2.1.9525

{

"id": "SW-v2.1.9525"

}

  - API Methods for Handling of Bulk Data

    - Introduction

The methods defined in this subchapter enable an SOVD client to manage bulk data such as kernel-dump files, temporarily generated log-files, etc. An SOVD client can download, and upload bulk data including proof-of-origins (security artefacts) and delete the data. The categories of the bulk data are defined by an OEM-specific BulkDataCategory type.

For transferring bulk data standard HTTP mechanism such as multipart request and response bodies are used as defined by RFC 1341 [27] and RFC 1847 [15]. There might be use cases, in which due to technical limitations a special API shall be used, e.g., to prepare and transfer the bulk data using a proprietary cloud data provider. How this is realized is not part of the SOVD API.

One example may be that log files increase over time, especially with higher log-levels activated. It might happen that a specific bulk data resource exceeds the limits of a single response. As a consequence, a resource that was accessible earlier may no longer be accessible by the same SOVD client with the same access rights. In this case, the SOVD API should still allow an SOVD client with the proper access rights to retrieve this bulk data resource.

      - Resource Organization

Figure 8 gives an overview how the bulk data resources of an entity are organized. Similar to data, the BulkDataCategory type is used for logical grouping of bulk data types. For example, “maps” and “pois” are different categories of bulk data which are typically handled differently by the navigation unit.

Using the methods described here a user can access the meta information of a bulk data category, can upload, download, and delete the corresponding resource. The access to the resources is based on the SOVD client’s authorization.

Figure 8	Resource organization based on categories

In addition, Figure 8 also shows the relationship between the standard resource logs and the log file from which the log resource retrieved the log entries. The underlying log file is accessible as a bulk data resource if required.

      - Encoding of Bulk Data

The bulk data transferred between the SOVD server and SOVD client is based on MIME (RFC 1341) [27] and MIME security (RFC 1847) [15] as a mandatory Content-Type header of the request or response. With this header the content of the body is defined and the SOVD server and SOVD client know how the content has to be interpreted.

Bulk data typically uses the Content-Type application/octet-stream. However, an SOVD client or SOVD server may always use a more specific MIME type than application/octet-stream if applicable. As an SOVD client may require additional information – e.g., the name of a file – additional information is provided as a JSON object. Thus, the typical body uses the Content-Type multipart/form-data.

For signing bulk data, the mechanisms defined by RFC 1847 [15] can be used, e.g., the Content-Type multipart/signed is used to store the signature for the bulk data together with the bulk data itself.

    - Retrieve List of all Bulk Data Categories

Method:

GET /{entity-path}/bulk-data/

Method Description:

This method provides the list of bulk data categories supported by an entity. The result contains all available and based on the authorization accesible bulk data categories. Certain bulk data categories are hidden based on the user’s authorization.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Table 173	Query Parameters - Retrieve list of all bulk data categories

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 174	Response Status Codes - Retrieve list of all bulk data categories

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 175 | The request was successful. |

Response Body:

Table 175	Response Body - Retrieve list of all bulk data categories

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | BulkDataCategory [] | M | The bulk data categories supported by the entity. |
| schema | OpenAPI Schema | C | The schema definition of the response. Condition: Only provided if the query parameter include-schema is true. |

Table 176	Examples for BulkDataCategory type

| Value | Description |
| --- | --- |
| maps | OEM specific category 1 |
| pois | OEM specific category 2 |
| logs | OEM specific category N |

Note: These enumeration values are an example and not normative. Example:

Request:

GET {base_uri}/apps/Navi/bulk-data HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

"maps",

"pois", "logs"

]

}

    - Read Bulk Data Meta Data

Method:

GET /{entity-path}/bulk-data/{category}

Method Description:

This method provides the list of BulkDataDescriptors for a specific category available and accessible for an SOVD client. The possible categories can be retrieved either using the interactive capability description method Retrieve List of all Bulk Data Categories or the offline capability description.

Note: Filtering can be done with proprietary mechanisms on the SOVD server-side or on the SOVD client-side based on the provided descriptors.

Path Parameters:

Table 177	Path Parameters - Read bulk data meta data

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| category | BulkData Category | M | The category represents a logical grouping of bulk data exposed by the SOVD server implementation, e.g., POIs stored on a navigation unit. |

Query Parameters:

Table 178	Query Parameters - Read bulk data meta data

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 179	Response Status Codes - Read bulk data meta data

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 180 | The request was successful. |

Response Body:

Table 180	Response Body - Read bulk data meta data

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | BulkDataDescriptor[] | M | List of BulkDataDescriptors |
| schema | OpenAPI Schema | O | The schema of the response. Condition: Only provided if the query parameter include-schema is true. |

Table 181	BulkDataDescriptor type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | A unique identifier for the bulk data managed by the SOVD server to identify the bulk data. |
| mimetype | string | M | Specifies the MIME type of the bulk data. |
| name | string | C | Name / description of the bulk data. |
| translation _id | string | O | Identifier for translating the name. |
| size | integer | C | Size of the bulk data in bytes. |
| creation_ date | string: datetime | C | Time and date of the creation of the bulk data. |

| last_ modified | string: datetime | C | Time and date of the last modification of the bulk data. |
| --- | --- | --- | --- |
| hash | string | C | Hash value of the bulk data for verifying the integrity of the bulk data after transfer. |
| hash_ algorithm | string | C | Algorithm used to generate the hash. |

- C:	Only provided if the information is available for the bulk data.

Example:

Request:

GET {base_uri}/apps/Navi/bulk-data/maps HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

"id":	"EU",

"name":	"OSM data for Europe", "mimetype": "application/vnd.osm+xml"

},

{

"id":	"US",

"name":	"OSM data for the United States of America", "mimetype": "application/vnd.osm+xml"

}

]

}

    - Download and Upload Bulk Data

This subchapter defines the SOVD API methods for downloading and uploading bulk data from and to the SOVD server respectively. Transfer of bulk data, as described in this subchapter, is limited to a size that can be transferred within one shot by the underlying technology. For transfers that exceed the limit, other methods, such as an indirect synchronization with third-party services (like cloud storage providers) shall be used.

The security parameters to check the integrity (e.g., SHA256, MD5, etc.) of the bulk data or verification of proof-of-origin of the bulk data either through digital signatures (e.g., asymmetric keys) or any other method can also be optionally sent along with the bulk data during the transfer in a multi-part message.

GET /{entity-path}/bulk-data/{category}/{bulk-data-id}

Download Bulk Data Method:

Method Description:

This method downloads the bulk data identified by bulk-data-id and category from the SOVD server. The SOVD server manages how bulk-data-id and category are mapped to the underlying data storage and how the retrieval of the bulk data based on the identifier is performed.

The method allows the transfer of a single bulk data.

Path Parameters:

Table 182	Path Parameters - Download bulk data

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| category | BulkData Category | M | The category the requested bulk data belongs to. |
| bulk-data-id | string | M | Bulk data identifier |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not require any request headers. However, if an SOVD client requires a proof-of-origin, the Accept header has to be set with the value multipart/signed. The SOVD server will then sign the transferred data and set Content-Type header to multipart/signed instead of multipart/form-data. OEMs may, alternatively, decide to use multipart/form-data and include the security artefacts within the application/json part using OEM specific formats.

Response Status Codes:

Table 183	Response Status Codes - Download bulk data

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | OEM specific | Provides content of the bulk data as part of the response. |
| 202 | OEM specific | The SOVD server accepted the request, but the data is not yet ready for retrieval. The SOVD client has to retry the request later on. An SOVD server may use the response header field Retry-After to inform the SOVD client after how many seconds a retry is reasonable. |
| 307 | -- | Temporary Redirect The SOVD server does not provide access to the resource under the requested URI, but the SOVD client has to use a different URI for downloading the bulk data. The URI is provided using the Location response header. |

Note: An SOVD server may technically not be able to provide a download for large bulk data or may have to do some processing before the download (e.g., compress the bulk data). By using the status codes 202 or 307, the SOVD server can separate the processing from the download step. For example, 202 is returned whilst the bulk data is uploaded to an external system like a cloud and 307 redirects the SOVD client to download the information from there.

Response Body:

The content of the response body is OEM specific. Depending on the implementation, the transferred bulk data can use multipart/signed, multipart/form-data with security parameters in a JSON object, or simply application/octet-stream (if it is not confidential information). The example below uses multipart/form-data with a JSON object for storing the signature of the bulk data.

Example:

Request:

GET {base_uri}/apps/Navi/bulk-data/pois/GasStations HTTP/1.1

Response:

HTTP/1.1 200 OK

Content-Type: multipart/form-data; boundary=formdataboundary

--formdataboundary

Content-Disposition: form-data; name="signature" Content-Type: application/json

{

"signature": "925b36e5bce4966f6ede24f0f4ec7a56",

}

--formdataboundary

Content-Disposition: form-data; name="GasStations"; filename="gas-stations.kml"

Content-Type: application/octet-stream [File content]

--formdataboundary

POST /{entity-path}/bulk-data/{category}

Upload Bulk Data Method:

Method Description:

This method uploads bulk data to the SOVD server and creates a resource for the bulk data. The request provides the content as well as any additional information, e.g., security artifacts, which are required to validate the content.

Path Parameters:

Table 184	Path Parameters - Upload bulk data

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| category | BulkData Category | M | The category of the bulk data to be uploaded. |

Query Parameters:

This method does not support query parameters.

Request Headers:

The following request headers for bulk data upload shall be used to specify the Content-Type, its size etc.

- Content-Type: Depends on Request Body

- Content-Length: Size of the content

- Content-Disposition: form-data; name="<field>"; filename="<file>”: Provides more information on the content and shall at least have the attribute name to give the SOVD server a hint on what is meant specifically. In addition, the attribute filename may be set to propose a file name in case the bulk data is stored on the file system. However, an SOVD server may also discard the proposed file name.

Request Body:

The content of the request body is OEM specific. Depending on the implementation, the transferred bulk data can use multipart/signed, multipart/form-data with security parameters in a JSON object, or simply application/octet-stream (if it is not confidential information).

Response Header:

The response includes the Location header to redirect the SOVD client to the created bulk data resource:

Location: {base_uri}/{entity-path}/bulk-data/{category}/{bulk-data-id}

Response Status Codes:

Table 185	Response Status Codes - Upload bulk data

| Status Code | Response Body | Description |
| --- | --- | --- |
| 201 | Table 186 | The request was successful. |

Response Body:

Table 186	Response Body - Upload bulk data

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Bulk data identifier created by the SOVD server to identify the bulk data. |

Example:

Request:

POST {base_uri}/app/navigation/bulk-data/pois HTTP/1.1 Content-Length: 2740

Content-Type: multipart/form-data; boundary=formdataboundary

--formdataboundary

Content-Disposition: form-data; name="signature" Content-Type: application/json

{

"signature": "925b36e5bce4966f6ede24f0f4ec7a56",

}

--formdataboundary

Content-Disposition: form-data;name="GasStations";file-name="gas-stations.kml"

Content-Type: application/octet-stream [File content]

--formdataboundary

Response:

HTTP/1.1 Created 201

Location: {base_uri}/app/navigation/bulk-data/pois/GasStations

{

"id": "GasStations"

}

    - Delete Bulk Data

There are two mechanisms to delete the bulk data present in a bulk data node

- Delete All Bulk Data Defined by Category

- Delete Specific Bulk Data Resource

DELETE /{entity-path}/bulk-data/{category}

Delete All Bulk Data Defined by Category Method:

Method Description:

This method requests the deletion of all bulk data for the given category.

Path Parameters:

Table 187	Path Parameters - Delete all bulk data defined by category

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| category | BulkData Category | M | The category of which all bulk data shall be deleted from. |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Status Codes:

Table 188	Response Status Codes - Delete all bulk data defined by category

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 189 | The request was successful. |
| 409 | GenericError | Request cannot be completed due to a general conflict, e.g., none of the bulk data resource could be deleted. |

Response Body:

Table 189	Response Body - Delete all bulk data defined by category

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| deleted _ids | string[] | M | List of deleted bulk data identifiers belonging to the given category |
| errors | DeletionError[] | M | Bulk data identifiers which could not be deleted and why. |

Table 190	DeletionError type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| id | string | M | Bulk data identifier, that could not be deleted. |
| error | GenericError | M | The reason why the bulk data resource could not be deleted. |

Example:

Request:

DELETE {base_uri}/apps/Navi/bulk-data/maps HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"deleted_ids": [ "US"

],

"errors": [

{

"id": "EU",

"error":

{

"error_code": "vendor-specific", "vendor_code": "unable-to-delete",

"message":	"Could not delete map currently in use."

}

}

]

}

DELETE /{entity-path}/bulk-data/{category}/{bulk-data-id}

Delete Specific Bulk Data Resource Method:

Method Description:

This method requests the deletion of a specific bulk data resource of the given category.

Path Parameters:

Table 191	Path Parameters - Delete bulk data

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| category | BulkData Category | M | The category of the bulk data to be deleted. |
| bulk-data-id | string | M | Bulk data identifier |

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Status Codes:

Table 192	Response Status Codes - Delete bulk data

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | The request was successful. |
| 409 | GenericError | Request cannot be completed because the deletion of the bulk data resource failed, e.g., the bulk data resource cannot be deleted as it is still in use. |

Response Body:

This method does not provide a response body.

Example:

Request:

DELETE {base_uri}/apps/Navi/bulk-data/maps/US HTTP/1.1

Response:

HTTP/1.1 204 No Content

  - API Methods for Logging

    - Introduction

The SOVD logging methods provide access to aggregated log information. Critical messages should always be included.

SOVD clients are able to specify by log configurations what is aggregated from different contexts, e.g., Components or Apps.

Note: Tracing is explicitly excluded.

The log information provided via SOVD is not intended for evaluation in the workshop. It is intended to be used by software experts typically located in the OEMs back office. For workshop technicians, critical messages will be reported via the faults interface including appropriate additional information.

Note: The online capability description shall deliver both logs/entries and logs/config

when querying the entity, if supported.

    - Retrieve List of All log Information

Method:

GET /{entity-path}/logs/entries

Method Description:

This method provides the log information aggregated by this entity.

Path Parameters:

This method does not support path parameters.

Query Parameters:

An SOVD client can optionally add query parameters to retrieve logging information only for a specific severity.

Table 193	Query Parameters - Retrieve list of all log information

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| severity | Severity | O | SOVD specific enumeration defining log-levels |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

Table 194	Severity type in relation to ContextType

| Severity | ContextType |  |
| --- | --- | --- |
|  | RFC5424 [36] | AUTOSAR_DLT [6] |
| fatal | Emergency Alert Critical | DLT_Fatal |
| error | Error | DLT_ERROR |
| warn | Warning | DLT_WARN |
| info | Notice Informational | DLT_INFO |
| debug | Debug | DLT_DEBUG DLT_VERBOSE |

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 195	Response Status Codes - Retrieve list of all log information

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 196 | The request was successful. |

Response Body:

Table 196	Response Body - Retrieve list of all log information

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | LogEntry[] | M | An array of log entries |
| schema | OpenAPI Schema | C | The schema definition of the response. Condition: Only provided if the query parameter include-schema is true. |

Table 197	LogEntry type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| timestamp | string:date -time | M | Time and date when the log entry has been created. |
| context | Context | M | Definition of the context of the log message identifying the source of the entry. |
| severity | Severity | M | SOVD specific datatype defining log-levels |
| msg | string | M | The actual log message (text) |
| href | string:uri-reference | O | Reference to a URI for retrieving the underlying original log file in an application specific format |

Table 198	Context attributes

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| type | ContextType | M | Enumeration identifying the logformat used by the respective log source. |
| … |  |  | Depending on the value of the type attribute, further attributes will be present (see Table 199). |

Table 199	ContextType dependent attributes

| ContextType | Additional ContextType dependent Attributes | Type |
| --- | --- | --- |
| RFC5424 |  |  |
|  | host | string |
|  | process | string |
|  | pid | integer |
| AUTOSAR_DLT |  |  |
|  | session | string |
|  | session_id | string |
|  | application_id | string |
|  | context_id | string |
|  | message_id | string |

Note: OEM-specific context types can be included using alpha-numeric characters, hyphens, and underscores and have to start with x-. An implementor of the SOVD API may introduce additional context types. These context types will not be supported by an off-the-shelf SOVD client.

Example:

Request:

GET {base_uri}/components/DrivingComputer/logs/entries HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"items": [

{

…

},

{

"timestamp": "2021-07-20T00:00:04.387819Z",

"context":

{

"type": "RFC5424",

"host": "Linux", "process": "systemd", "pid": 1

},

"severity": "info",

"msg": "Closed D-Bus User Message Bus Socket", "href": "{base_uri}/components/DrivingComputer/bulk-data/logs/server.log"

},

{

…

}

]

}

    - Configure SOVD Logging

Method:

PUT /{entity-path}/logs/config

Method Description:

This method configures the log aggregation.

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

Due to its dynamic nature, the context information is not provided by the online or offline capability description. The information has to be exchanged by other means.

Table 200	Request Body - Configure SOVD logging

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | LogConfiguration[] | M | Array of LogConfiguration elements |

Table 201	LogConfiguration type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| context | Context | M | Definition of the context of the log message. Parts not specified in the context are regarded as not set. |
| severity | Severity | M | The severity specifies the log-level for the given context. |

Response Status Codes:

Table 202	Response Status Codes - Configure SOVD logging

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | The configuration was accepted. |
| 400 | GenericError | Invalid configuration provided |

Note: In general, changes to the Log configuration triggered via the SOVD client shall be persistent (i.e., still be available after a system restart to support enhanced startup logging). Implementations could deviate from this rule for specific reasons.

Response Body:

This method does not provide a response body.

Example:

Request:

PUT {base_uri}/components/DrivingComputer/logs/config HTTP/1.1 Content-Type: application/json

{

"items": [

{

"context":

{

"type": "RFC5424",

"host": "Linux", "process": "systemd"

},

"severity": "warn"

}

]

}

Response:

HTTP/1.1 204 No Content

    - Retrieve the Current SOVD Logging Configuration

Method:

GET {entity-path}/logs/config

Method Description:

This method returns an array of configured SOVD LogConfiguration elements, specifying the different contexts for which a specific log configuration was created with the method Configure SOVD Logging.

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Response Status Codes:

Table 203	Response Status Codes - Retrieve the current SOVD logging configuration

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 204 | The request was successful. |

Response Body:

Table 204	Response Body - Retrieve the current SOVD logging configuration

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| contexts | LogConfiguration[] | M | An array of LogConfiguration elements |

Example:

Request:

GET {base_uri}/components/DrivingComputer/logs/config HTTP/1.1

Response:

HTTP/1.1 200 OK

{

"contexts": [

{

"context":

{

"type": "RFC5424",

"host": "Linux", "process": "systemd", "pid": 1

},

"severity": "info"

},

{

"context":

{

"type": "AUTOSAR_DLT",

"session": "SES01", "session_id": "0x0001",

"application_id": "WindowControl", "context_id": "STD0", "message_id": "msg000012"

},

"severity": "info"

}

]

}

    - Reset SOVD Logging Configuration to Default

Method:

DELETE /{entity-path}/logs/config

Method Description:

This method resets the current configuration to its default.

Path Parameters:

This method does not support path parameters.

Query Parameters:

This method does not support query parameters.

Request Headers:

This method does not use any specific header parameters.

Request Body:

No request body required.

Response Status Codes:

Table 205	Response Status Codes - Reset SOVD logging configuration to default

| Status Code | Response Body | Description |
| --- | --- | --- |
| 204 | -- | Configuration has been reset successfully. |
| 404 | GenericError | The SOVD client tried to reset a configuration which does not exist. |

Response Body:

This method does not provide a response body.

Example:

Request:

DELETE {base_uri}/components/DrivingComputer/logs/config HTTP/1.1

Response:

HTTP/1.1 204 No Content

  - Authentication of SOVD Clients (Informative)

    - Introduction

The goal of authentication is to enable an authorization and validation process to protect the resources of any accessible entity in a vehicle against misuses of features while accessing an SOVD server.

Another aspect of security might be end-to-end security, e.g., signatures for coding and programming data. This is part of the respective payload data and is not described in this SOVD API subchapter.

This subchapter defines requirements for the SOVD API. However, the process using the SOVD API features might differ depending on the security architecture as well as authorization protocols and is thus not standardized here. Only examples for the process are given.

A trust anchor needs to be established inside the vehicle. This trust anchor knows the SOVD servers and can give out tokens to a requestor to access those SOVD servers or their functionalities. Tokens may be implemented e.g., using JSON Web Tokens (see [11]).

The Vehicle AuthorizationServer provides the possibility to validate the issued tokens, which on the other hand enables the SOVD servers to decide how to deal with a request by the SOVD client.

The SOVD servers shall protect all resources against unauthorized access. The Vehicle AuthorizationServer does not directly prevent the SOVD server from being accessed by the SOVD client, but it does provide information, with what kind of access an SOVD client is authenticated. Only if the SOVD client has the appropriate access rights, the SOVD server shall grant the access to the requested resources.

Therefore, a receiving SOVD server may only process further requests with a valid token. If no token or an invalid token is provided with a request, the SOVD server needs to deny the request or redirect the SOVD client to the Vehicle AuthorizationServer. Therefore, a token provides the legitimacy to the SOVD client to communicate with the SOVD server.

The SOVD client should always possess a valid token from the /token URI before performing requests at an SOVD server. The AuthorizationServer creates a token after successfully validating the request.

The AuthorizationServer signs each token it creates with a private key so the token can be validated by the requested SOVD client.

With each request, the SOVD client shall send a valid token to the receiving SOVD server. Only if the token validation was successful, the SOVD server may execute the request.

SOVD offers basically two options related to authentication and authorization:

- Online: shown in Figure 9 (AuthorizationServer in backend and vehicle):

  - AuthorizationServer in backend and vehicle established trust and are synchronized

  - An SOVD client authenticates against the backend.

  - The backend authenticates against the vehicle

  - Backend issues the token to the SOVD client

- Offline: shown in Figure 10 (AuthorizationServer in the vehicle):

  - An SOVD client can authenticate itself at the vehicle using its stored credentials (e.g., certificates)

  - A token is negotiated between SOVD client and vehicle

Note: Both figures only illustrate the interaction between the components underlying to the SOVD security concept. For this purpose, only one HPC which hosts the SOVD API implementation, and the Vehicle AuthorizationServer are shown here. This might differ depending on the concrete vehicle architecture and deployment of SOVD servers across several HPCs.

In the following, the overall SOVD security concept based on these two options is introduced in more detail.

The concept is based on various components interacting together:

- A vehicle containing

- One or multiple SOVD servers,

- a classic mechatronic architecture with multiple ECUs,

- an AuthorizationServer, e.g., contained in a security module

- A diagnostics SOVD client

- A Backend User Administration

      - Online Authentication and Authorization

An example for a typical sequence can be as shown in Figure 9:

        - The SOVD client authenticates at the Backend User Administration, e.g., via a log-in.

        - The SOVD client requests a token from the Backend AuthorizationServer. If allowed, the Backend AuthorizationServer issues a token to the SOVD client.

        - The Backend AuthorizationServer has a trusted communication with the Backend User Administration and is synchronized with the Vehicle AuthorizationServer(this communication is not scope of SOVD standardization). The Vehicle AuthorizationServer validates the SOVD client’s authorization.

        - The SOVD client uses this token when issuing requests to the SOVD API.

        - The addressed SOVD server validates the token by asking the Vehicle AuthorizationServer for the validation key (public key fitting the private key which was used to create the signature for the token). This public key is then used to validate the signature with the public key and the access can be granted.

Figure 9:	Online authentication and authorization

      - Offline Authentication and Authorization

An example for a typical sequence can be as shown in Figure 10:

        - The SOVD client needs to be authorized prior to be able to request a token at the vehicle by providing credentials (e.g., a certificate) to the Vehicle AuthorizationServer via the SOVD API. The Vehicle AuthorizationServer responds with an authorization code.

        - The SOVD client requests a token from the Vehicle AuthorizationServer and provides the authorization code as part of the token request. The SOVD client also provides its credentials in this step.

        - If allowed, the Vehicle AuthorizationServer issues a token to the SOVD client.

        - The SOVD client uses this token when issuing requests to the SOVD API.

        - The addressed SOVD server validates the token from the Vehicle AuthorizationServer (public key fitting the private key which was used to create the signature for the token) and the access can be granted.

Figure 10:	Offline authentication and authorization

      - Implementation Hints

Since the SOVD API shall be independent and non-restrictive regarding applicable authentication mechanisms and protocols for implementing SOVD client authorization as well as token-based authentication of requests, in the following, a basic set of resources is described with respective examples. An implementation of the SOVD API may provide additional resources (which are not standardized by the SOVD API) based on the selected authentication protocol and token format/implementation.

To model complex chained API calls (application calls API 1, and this API 1 calls another API 2 to fulfil a certain function), for both online and offline, the Vehicle AuthorizationServer could offer a mechanism called token exchange, to allow API 1 to act on behalf of the original calling application in regard to API 2. Effectively, API 1 can call API 2 with the permissions granted to the application. The technical specifics of this flow are defined in [12].

    - Secure Connection using TLS

The communication between the SOVD client and the AuthorizationServer and to any SOVD server as well as the communication inside the vehicle to the Vehicle AuthorizationServer or between each SOVD server requires a secure end-to-end channel, so that the communication cannot be intercepted or sniffed by any third party. For this, current consensus standards should be employed (e.g., at least TLS 1.2).

    - Verifying SOVD Client Credentials and Requesting a Token at the Backend

In the online authentication and authorization shown in Figure 9, an SOVD client which requires access to any SOVD server needs to be known and administrated in the backend. After successful authentication at the backend, the SOVD client can request a token at the Backend AuthorizationServer. If allowed, the Backend AuthorizationServer issues a token to the SOVD client. The Backend AuthorizationServer and the Vehicle AuthorizationServer build on established trust, i.e., the backend authenticates against the vehicle, and both AuthorizationServer instances are synchronized. Therefore, the Backend AuthorizationServer synchronizes the issued token created for the SOVD client with the Vehicle AuthorizationServer to enable token validation within the vehicle.

    - Verifying SOVD Client Credentials at the Vehicle

Method:

POST /authorize

Method Description:

This method initiates an authorization flow to authenticate and authorize an SOVD client to act upon a human user. Depending on the actual flow type, the result of this flow is an authorization code, that the client can use to obtain a token using a subsequent call to the

/token API method, or an access token. This method is only used in the context of human user interaction.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Potential query parameters are defined by the selected authorization protocol.

Request Headers:

Potential request headers are defined by the selected authorization protocol.

Request Body:

The request body depends on the selected authorization protocol and its implementation by the SOVD API.

Response Status Codes:

Table 206	Response Status Codes - Verifying SOVD client credentials at the vehicle

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Response Body | The request was successful. |
| 400 | GenericError | If the SOVD client credentials are not provided as part of the request. The error code complies to the errors defined in [13]. |

Response Body:

The response body depends on the selected authorization protocol and its implementation by the SOVD API.

Example:

As the request and response behavior is highly dependent on the utilized, protocol, no example is given here.

    - Requesting a Token

Method:

POST /token

Method Description:

The SOVD client can request a token via this method. The AuthorizationServer shall accept requests to create tokens from authenticated SOVD clients only. Therefore, an SOVD client shall authenticate against the AuthorizationServer or have some other credentials to proof its identity prior a token is created and returned via the SOVD API. An SOVD client, which cannot be authorized shall not be able to request a token through the SOVD API. Traditional methods like client-id / client-secret combinations can be used, as well as advanced methods for instance, based on client certificates.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Potential query parameters are defined by the selected authorization protocol.

Request Headers:

Potential request headers are defined by the selected authorization protocol.

Request Body:

The request body depends on the selected authorization protocol and token implementation.

Response Status Codes:

Table 207	Response Status Codes - Requesting a token

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Response Body | The request was successful. |
| 400 | GenericError | If the challenge was not responded to correctly or something else is missing or is invalid (request, client-id, etc.) For example, if using OAuth 2.0 the following list of errors might be replied in the response body: [13] chapter 5.2 |

Response Body:

The response body depends on the selected authorization protocol and token implementation.

Example:

As for example for a token request, in the following, OAuth 2.0 [13] is assumed as underlying authorization protocol and “client credentials” are used as authorization grant. Example is taken from [14], chapter 4. Cache control for HTTP/2 should be deactivated in order not to cache secrets.

Request:

POST {base_uri}/token HTTP/1.1 Host: server.example.com

Authorization: Basic czZCaGRSa3F0MzpnWDFmQmF0M2JW Content-Type: application/x-www-form-urlencoded

grant_type=client_credentials

Response:

HTTP/1.1 200 OK

Content-Type: application/json;charset=UTF-8 Cache-Control: no-store

Pragma: no-cache

{

"access_token":"eyJ0eXAiOiJKV1QiLA0KICJhbGciOiJIUzI1NiJ9. eyJpc3MiOiJqb2UiLA0KICJleHAiOjEzMDA4MTkzODAsDQogImh0dHA6Ly9leGFtcGxlLm NvbS9pc19yb290Ijp0cnVlfQ.

dBjftJeZ4CVP-mB92K27uhbUJU1p1r_wW1gFWFOEjXk", "token_type":"Bearer",

"expires_in":3600

}

    - Request Header for Access-Restricted Resources

When trying to access any access-restricted resource at the SOVD API, the SOVD client has to authorize against the server by providing a corresponding token in the request header.

Whenever an SOVD client requests data, a functionality or method from an SOVD server by sending an HTTP request to a URI of the SOVD API, a valid token shall be provided within the HTTP Authorization header of the request, e.g., by providing a Bearer [14] token [11]. If a token is valid or not is validated by the SOVD server with the help of the Vehicle AuthorizationServer as described in 6.15.7. This includes for example to check if the SOVD client sending the request is allowed to/has the required permissions to execute an HTTP method on the protected resource specified through the request.

The following response status codes are foreseen to provide information about authorization errors.

Table 208   Response Status Codes - Request header for access-restricted resources

| Status Code | Response Body | Description |
| --- | --- | --- |
| 401 | -- | If no token is provided in the Authorization header of the request. |
| 403 | GenericError | If the token validation done by the SOVD server responsible for the target resource has failed. For example, the SOVD client identified by the token does not have the required access rights or role associated and therefore the token is not sufficient to interact with the protected resource using the selected HTTP method. For example, the SOVD client has not the access rights to delete the resource. If for security reasons the presence of a protected resource shall not be unveiled, the response code „404 Not Found“ can be sent. This principle is also described in the HTTP standard. |

    - Validating a Token

If the SOVD API receives a request by an SOVD client, the accompanying token will be validated by the SOVD API. The validation itself is conducted by the underlying SOVD server with the help of the Vehicle AuthorizationServer. The SOVD server has to request the public key from the Vehicle AuthorizationServer to validate the signature of the token as well as any other limiting content in the token. Based on that, the SOVD server will determine whether the SOVD client is authorized to execute the request. Depending on the result of the validation, the requested resource can be accessed or the SOVD server rejects the request with a respective HTTP response status code within the response to indicate that the validation was not successful as described in 6.15.6.

- Classic Diagnostic Adapter

  - Introduction

The introduction of SOVD will not eliminate classic diagnostic protocols such as UDS in the vehicles. Furthermore, it is assumed that a considerable number of devices will still be based on UDS diagnostics. Therefore, the SOVD API can also provide access to components which are based on classic diagnostic protocols.

Typically, a component in the SOVD Server will perform the mapping of the SOVD HTTP requests to the classic diagnostic protocol. It is not foreseen that the classic components themselves will provide an SOVD interface.

Currently the standard only describes the mapping of SOVD to UDS, however, other mappings can be implemented.

It is not mandatory for an SOVD server to expose classic diagnostic services as resources. If it doesn’t, it is assumed that the server handles these classic diagnostic services internally (e.g., ensuring correct session/security levels) and an SOVD client does not need access to them. If the SOVD API provides individual access to the classic diagnostic services, it shall be done as specified in this chapter.

  - Access to UDS Based Entities

There is an overlap between the API methods supported by SOVD and the diagnostic services provided via UDS. Therefore, the approach of SOVD is to include the UDS based content into the defined API methods. There are no special API methods defined for accessing UDS content.

Table 209	Overview on how to map UDS based contents to SOVD

| Service-Id | REST resource | See |
| --- | --- | --- |
| 0x10 | <entity>/modes/session | 7.3.1 |
| 0x11 | <entity>/operations/ecureset | 7.6.2 |
| 0x27, 0x29 | <entity>/modes/security | 7.3.2 |
| 0x28 | <entity>/modes/commctrl | 7.3.4 |
| 0x3E | Will be handled by the Classic Diagnostic adapter |  |
| 0x85 | <entity>/modes/dtcsetting | 7.3.5 |
| 0x22, 0x2E | <entity>/data/ <entity>/configurations/ | 7.4 |
| 0x14, 0x19 | <entity>/faults/ | 7.5 |
| 0x2F | <entity>/operations/ | 7.6.1 |
| 0x31 | <entity>/operations/ | 7.6.1 |

| Service-Id | REST resource | See |
| --- | --- | --- |
| 0x23, 0x24, 0x2A, 0x2C, 0x34, 0x35, 0x36, 0x37, 0x38, 0x84, 0x86 | Will not be represented on SOVD API level |  |

  - Specific Mapping of UDS Services to SOVD Modes

    - Mapping of SessionControl ($10) Subfunctions to SOVD Modes

There shall be a mode session representing the available UDS diagnostic session in SOVD. The PUT method (see 6.10.4) shall support an ECU-specific enumeration representing the subfunctions of service $10 supported by the UDS ECU. This method may also control the sending of TesterPresent commands by switching to a non-default session.

    - Mapping of SecurityAccess ($27)

This subchapter provides advice how UDS security using service $27 could be implemented in an SOVD server.

UDS Service $27 requires a security access sequence. This functionality can be

- performed as a sequence implicitly by the SOVD server when an SOVD mode

security is established,

- performed as a sequence implicitly by the SOVD server when a UDS ECU requiring

$27 security is accessed or

- provided by exposing the individual $27 subservices as operations at the SOVD API (the sequence has to be implemented in the client in this case).

If option c) is used, a mode security representing the available UDS security access modes in SOVD can be implemented.

The GET method (see 6.10.3) supports an ECU-specific enumeration representing the RequestSeed subfunctions of service $27 supported by the UDS ECU.

The PUT method (see 6.10.4) supports an ECU-specific enumeration representing the SendKey subfunctions of service $27 supported by the UDS ECU.

    - Mapping of $29 Authentication to SOVD Modes (Informative)

This subchapter provides advice how UDS security using service $29 could be implemented in an SOVD server.

UDS Service $29 requires an authentication sequence. This functionality can be

- performed as a sequence implicitly by the SOVD server when an SOVD mode

authentication is established,

- performed as a sequence implicitly by the SOVD server when a UDS ECU requiring authentication is accessed or

- provided by exposing the individual $29 subservices as operations at the SOVD API (the sequence has to be implemented in the client in this case).

Options a) and b) provide the possibility to handle the authentication within the SOVD server, while option c) exposes the subservices as defined in ISO14229-1 as used in the respective vehicle.

    - Mapping of Communication Control ($28) Subfunctions to SOVD Modes

There shall be a mode comm-ctrl representing the communication control service.

The PUT method (see 6.10.4) shall support an ECU-specific enumeration representing subfunctions of service $28 supported by the UDS ECU.

    - Mapping of Control DTC Settings ($85) Subfunctions to SOVD Modes

There shall be a mode dtcsetting representing the control DTC setting service.

The PUT method (see 6.10.4) shall support an enumeration (“on/off”) representing UDS subfunctions of service $85 supported by the UDS ECU.

  - Mapping of UDS Services to Data Resources

UDS data identifiers available via ReadDataByIdentifier ($22) shall be available as data resources in SOVD. An SOVD server could determine the category of data based on its input data (e.g., ODX semantics). It is recommended to provide an individual data resource for each UDS data identifier.

In case a data identifier is also writable (using WriteDataByIdentifier $2E) it shall be available via the same data resource using the PUT method.

Alternatively, data identifiers for writing can also be provided as configurations resources.

  - Mapping of UDS Services to Fault Resources

The DTC information available via UDS Services $19 and $14 shall be available as individual faults in the faults resource collection representing the UDS entity.

See Table 46 for details on which fields in the SOVD API resemble fault information provided for classic ECUs.

  - Mapping of UDS Services to Operation Resources

    - Mapping of Routine and IOControl Services to Operation Resources

IOControl Identifiers (available via Service $2F and $22) and Routine Identifiers (available via Service $31) shall be available as operations resources.

For the relation of IOControl and RoutineControl UDS subfunctions to API methods see Table 102.

Note: Several UDS services (with different UDS subfunctions) referring to the same Routine/IOControl Identifier shall be mapped to the same operations resource.

    - Mapping of EcuReset Service ($11) to Operation Resources

In case the UDS entity supports ECUReset, there shall be a resource ecureset in the resource collection for operations. The resource shall support a start parameter “ResetType” being an ECU-specific enumeration of the different UDS subfunctions supported by the ECU.

- Terms and Definitions

For the purposes of this ASAM standard the following terms and definitions apply (in alphabetical order).

| Authentication | It is the mechanism employed to verify an identity towards a software system. Within SOVD authentication is used to verify the identity of the SOVD client (user). Remark: Examples for authentication method are ISO 14229-1 Service $29 [1], TLS 1.3. [3], IEEE 1609.2[7] or X.509 [8]-Attribute Certificates |
| --- | --- |
| Authorization | Is the mechanism to verify whether an authenticated SOVD client (user) has sufficient privileges to execute a specific SOVD method. It is also used to verify, whether an authenticated SOVD client (user) has sufficient privileges to access specific data or capabilities of the vehicle that can be obtained or invoked by executing a specific SOVD method (e.g., it is possible a user has access to SOVD method A, but is only allowed to retrieve a subset of the data that can potentially be obtained by calling SOVD method A) |
| Bulk Data | Specifies any larger set of structured or unstructured digital data. The volume of the data is high and / or in proprietary or binary format, so that it is not possible or desirable to standardize its format/structure specifically as part of a request/response structure of a SOVD method. |
| Capability | Describes a possibility of interaction with an entity via SOVD. A capability can be a specific method, a sensor value, a routine, a coding parameter, an access to log files etc. |
| Capability Description | Is a human- and machine-readable record of all capabilities of a specific vehicle (instance-specific CD) or a set of similar vehicles, e.g., a vehicle platform (general CD). Remark: As such, it resembles an ODX-file for a classic ECU. Note: A capability description does not have to be statically written to a vehicle or entity. Rather, they can be composed at runtime from various data sources. Information that may later contribute to an in-vehicle capability description generation, can be transferred into the vehicle (or into entities) via existing methods like bulk data writing, vehicle / entity configuration, vehicle / entity reprogramming. It may be specific to OEMs processes, vehicle architecture and update process considerations, which way to handle the capability description may be appropriate. |

| Credentials | Are used by software modules to control access to information or other resources. The classic combination of a user's account number or name and a secret password is a widely used example of IT credentials. An increasing number of information systems use other forms of documentation of credentials, such as biometrics (fingerprints), public key certificates, and so on. |
| --- | --- |
| Diagnostic Trouble Code | Fault or event that was stored by an ECU as part of its self-diagnostic capabilities. |
| ECU | Electronic Control Unit – an embedded computing device within the vehicle that is mainly used for control functions and is based on realtime oriented operating systems like OSEK [4], AUTOSAR classic [5]. |
| HPC | High Performance Computer – a powerful computing device within the vehicle that is capable of hosting multiple virtualized guest systems based on operating systems. |
| In-Vehicle Diagnostics | Diagnostic applications that are self-sustained and executed within the vehicle (potentially running on an HPC), which means they do not require a connection to the Internet to be executed. |
| Offline Capability Description | Is an offline-version of a capability description that exists outside of the context of a particular vehicle (e.g., as a file, database data). |
| Online Capability Description | Is a version of a capability description that is obtained directly from a vehicle instance or an HPC through SOVD capability discovery mechanisms. |
| Operation | An operation is an internal function, which shall be controlled and parameterized by the SOVD API. Operations are used additionally as synonym for actuators (I/O control) and routines. |
| Proximity Diagnostics | Diagnostic applications that are executed on an external device (i.e., device that is not part of the vehicle) that is connected to an SOVD server of a vehicle in a wired or wireless way using common LAN or NFC technology. |
| Remote Diagnostics | Diagnostic applications that are executed in a remote external environment (e.g., backend, cloud) that is connected to the vehicle using common WAN technology. |
| Resource | The key abstraction of information in REST is a resource. Any information that can be named can be a resource: a document or image, a temporal service, a collection of other resources, a non-virtual object (e.g., a person), and so on. REST uses a resource identifier to identify the resource involved in an interaction between components. |

- Symbols and Abbreviated Terms

| API | Application Programming Interface |
| --- | --- |
| APP | APPlication |
| C | Conditional |
| CD | Capability Description |
| CDA | Classic Diagnostic Adapter |
| CPU | Central Processing Unit |
| DHCP | Dynamic Host Configuration Protocol |
| DID | Datal IDentifier |
| DLT | Diagnostic Log and Trace |
| DNS | Domain Name System |
| DTC | Diagnostic Trouble Code |
| ECU | Electronic Control Unit |
| E/E | Electrical / Electronic architecture |
| FOTA | Firmware Over The Air |
| HPC | High Performance Computer |
| HTTP | HyperText Transfer Protocol |
| ID | IDentifier |
| IETF | Internet Engineering Task Force |
| IT | Information Technology |
| JSON | Java Script Object Notation |
| LAN | Local Area Network |
| M | Mandatory |
| MD5 | Message-Digest Algorithm 5 |
| MIME | Multipurpose Internet Mail Extensions |
| NFC | Near Field Communication |
| O | Optional |
| ODX | Open Diagnostic data eXchange |
| OEM | Original Equipment Manufacturer |
| OS | Operating System |
| OSEK | Offene Systeme und deren Schnittstellen für die Elektronik im Kraftfahrzeug |
| OTA | Over The Air |

| REST | REpresentational State Transfer |
| --- | --- |
| RFC | Request For Comments |
| SD | Service Discovery |
| SOVD | Service-Oriented Vehicle Diagnostics |
| TLS | Transport Layer Security |
| UDS | Unified Diagnostic Services |
| UML | Unified Modeling Language |
| URI | Uniform Resource Identifier |
| URL | Uniform Resource Locator |
| UUID | Universally Unique IDentifier |
| VDX | Vehicle Distributed eXecutive |
| VIN | Vehicle Identification Number |
| WAN | Wide Area Network |
| XML | eXtensible Markup Language |
| YAML | Yet Another Markup Language |

- Bibliography

- ISO 14229-1:2020 Road vehicles — Unified diagnostic services (UDS) — Part 1:

Application layer

- IETF RFC 7231: Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content; 2014

- IETF RFC 8446: The Transport Layer Security (TLS) Protocol Version 1.3; 2018

- ISO 17356-3:2005 Road vehicles — Open interface for embedded automotive applications — Part 3: OSEK/VDX Operating System (OS)

- AUTOSAR; Classic Platform Release Overview

- AUTOSAR; Specification of Diagnostic Log and Trace

- IEEE 1609.2 - IEEE Standard for Wireless Access in Vehicular Environments--

Security Services for Applications and Management Messages

- ISO /IEC 9594-08:2017 Information technology — Open Systems Interconnection —

The Directory — Part 8: Public-key and attribute certificate frameworks

- ISO 22901-1:2008 - Open diagnostic data exchange (ODX) - Part 1: Data model specification

- Open API Initiative; OpenAPI Specification v3.1.0; 2021

- IETF RFC 7519; JSON Web Token (JWT); 2015

- IETF RFC 8693; OAuth 2.0 Token Exchange; 2020

- IETF RFC 6749; The OAuth 2.0 Authorization Framework; 2012

- IETF RFC 6750; The OAuth 2.0 Authorization Framework: Bearer Token Usage; 2012

- IETF RFC 1847; Security Multiparts for MIME: Multipart/Signed and Multipart/Encrypted; 1995

- R. T. Fielding; “REST: Architectural Styles and the Design of Network-based Software Architectures,” PhD dissertation, University of California, Irvine, 2000

- Ecma international; Standard ECMA 262 ECMA Script 2021 Language Specification; 2021

- ISO 22900-2; Road vehicles — Modular vehicle communication interface (MVCI) — Part 2: Diagnostic protocol data unit application programmer interface (D-PDU API); 2009

- IETF RFC 3339; Date and Time on the Internet: Timestamps; 2002

- IETF RFC 6762; Multicast DNS; 2013

- ISO 8601-1; Date and Time Representations for information exchange – Part 1: Basic rules: 2019

- IETF RFC 7540: Hypertext Transfer Protocol Version 2 (HTTP/2); 2015

- IETF RFC 2616; Hypertext Transfer Protocol Version 2 (HTTP/1.1); 1999

- IETF RFC 6763; DNS-Based Service Discovery; 2013

- JSON; JSON schema specification; 2020

- ASAM e.V.; ODX Authoring Guidelines; 2011

- IETF RFC 1341; MIME (Multipurpose Internet Mail Extensions); 1992

- IETF RFC 8259; The JavaScript Object Notation (JSON) Data Interchange Format; 2017

- ISO 22900-3; Road vehicles — Modular vehicle communication interface (MVCI) —

Part 2: Diagnostic server application programming interface (D-Server API); 2012

- IETF RFC 5322; Internet Message Format; 2008

- IETF RFC 3986; Uniform Resource Identifier (URI): Generic Syntax; 2005

- IETF RFC 6570; URI Template; 2012

- IETF RFC 6901; JavaScript Object Notation (JSON) Pointer; 2013

- IETF RFC 4122; A Universally Unique IDentifier (UUID) URN Namespace; 2005

- ASAM e.V.; ASAM Data Types; 2005

- IETF RFC 5424; The Syslog Protocol; 2009

Appendix: A. SOVD API Common Example

Informative

  - Introduction

In this appendix, an example is given for the SOVD API. It is not intended to show a complete example here, but with enough details to explain the intended usage of the SOVD API. This example is used as the base for the method examples given in the standard.

For this example, object diagrams are used to describe the entity hierarchy and available resources for each entity. For more complex sequences of methods defined in the SOVD API, sequence diagrams are provided.

The diagrams in this example use the UML notation. The object diagrams show the relation between the entities themselves, and between entities and resources. These diagrams are not to be understood as a model of the underlying data in the vehicle.

Furthermore, this appendix provides an example of an OEM specific realization of a Function

entity and how this can be documented.

  - Entity Hierarchy

Figure 11 shows the entity hierarchy of the example for an SOVD server named “Root”. The SOVD server includes one top-level Area “Driving”, which contains the two subareas “Perception” and “Controlling”. Each of these Areas link to top-level Components under “Root”.

Figure 11	Example hierarchy for “Root”

The Component “DrivingComputer” is an HPC running a hypervisor on which two different operating systems are executed. Both OSs are integrated as subcomponents of “DrivingComputer” into the SOVD API hierarchy and link to Apps. The other Components “Camera”, and “PowerSteering” are defined as embedded ECUs in this example.

  - Entities and Their Resources

The following object diagrams illustrate the resource collections and resources used in the example HTTP snippets in the SOVD API standard.

    - App AdvancedLaneKeeping

Figure 12	Resources of the App “AdvancedLaneKeeping”

    - App WindowControl

Figure 13	Resources of the App “WindowControl”

    - App Navi

Figure 14	Resources of the App “Navi”

    - Component Camera

Figure 15	Resources for the Component “Camera”

    - Component PowerSteering

Figure 16	Resources and variants for the Component “PowerSteering”

  - Complex Sequence Examples

The following subchapters show the usage of the SOVD API in situations where a sequence of API methods has to be called. These examples are in line with the HTTP snippets at the respective API methods in chapter SOVD REST API.

    - Creation and Usage of a Temporary Data-List Resource

Figure 17	UML sequence diagram for a data-list resource

    - SteeringAngleControl Operation incl. Modes and Locks

The following subchapter gives an overview how the execution of an operation via the SOVD API may look like for an “I/O-control” operation (SteeringAngleControl) of a classic ECU (PowerSteering) as an example. Therefore, also required prerequisites to execute this operation are described in the following. This comprises locking the underlying Component to prevent any interference with the execution of other operations or parallel access of other SOVD clients as well as manually setting the ECU to a specific mode which is required to execute the operation. For each of the steps, the methods introduced within this document are used as a basis. Respective examples for the underlying HTTP requests and responses are given in the API method descriptions.

Figure 18 depicts an excerpt of the topology, i.e., the SOVD API resources, relevant for the execution of the example operation “SteeringAngleControl” at the respective ECU “PowerSteering”. The operation allows to control the steering angle in two different ways. With the parameter “control-type” set to “absolute”, the PowerSteering controls the steering angle

to the position specified in parameter “angle”. With the “control-type” set to “relative”, the PowerSteering will adjust the current angle by the value provided by the parameter “angle”. We assume that the “SteeringAngleControl” operation requires a lock and specifies that the “PowerSteering” has a “session” mode which has to be set to “EXTENDED” in order to execute the operation. The SOVD server support “Explicit” mode control as well as “Hint-based”. It is not capable of implicitly identifying the required mode setting based on the executed operation.

Figure 18:	Topology for the “SteeringAngleControl” operation of “PowerSteering”

The overall sequence of steps for controlling an operation of an ECU are shown in Figure 19. For the sake of understandability, no exceptional or erroneous paths are presented within the sequence diagram nor discussed as part of the example in the following. Furthermore, the proximity challenge is not explicitly reflected as part of the sequence diagram.

Figure 19:	UML sequence diagram for the “SteeringAngleControl” operation

The following list summarizes the steps depicted in the sequence diagram above:

- Acquire Lock: The SOVD client acquires a lock of the ECU which allows the operation to be executed.

- Start Operation [Explicit]: The SOVD client explicitly sets the required mode “session” to “EXTENDED”, which is a prerequisite for executing the selected operation. The SOVD client requests the execution of the selected operation with a POST request containing all required input parameters and further information. The SOVD server then triggers the execution of the respective operation in the “PowerSteering” Component and creates a new execution resource. In this alternative, the SOVD server has identified that the operation can be executed and responds to the SOVD client before the ECU sends a positive response. The SOVD client has to request the status to learn that the ECU may have sent a negative response.

- Start Operation [Hint-based]: As an alternative to the two previous steps, the SOVD client requests the execution of the selected operation with a POST request and provides the “hint” to the SOVD server to set the mode “session” to “EXTENDED”. This request also contains all required input parameters. The SOVD server sets the requested mode and triggers the execution of the respective operation in the “PowerSteering” Component and creates a new execution resource. Here, the SOVD server waits for a positive response from the ECU before sending the answer to the SOVD client. This behavior is not specified and left to the SOVD server’s implementation.

- Get execution status: This temporary execution resource is then used by the SOVD client to poll the status of the operation execution via a GET request. As soon as the SOVD server receives a result from the ECU “PowerSteering” for the operation execution, the result data is returned to the SOVD client.

- Reset value: Before the operation is stopped, the SOVD client requests to reset the value of the operation via a PUT request and the capability set to “reset”.

- Stop execution: Afterwards, the SOVD client stops the operation via a DELETE

API method.

- Reset Mode: The SOVD client deactivates the previously activated mode of the ECU, e.g., by setting the mode “session” to “DEFAULT”.

- Release Lock: Finally, the SOVD client releases the acquired lock on the ECU to enable others SOVD clients to interact with the previously locked resources again.

  - Function VehicleHealth

    - Introduction

In this part, an example is provided for a Function entity. It shows how an OEM can define

Functions and describe the API methods available.

Since the SOVD API does not permit the retrieval of aggregated faults for an Area and thus all related Components, this example realizes this functionality in an SOVD server using an entity type Function. Figure 20 shows the Function VehicleHealth beneath an SOVD server VehicleDiag.

Figure 20	Function VehicleHealth

The Function described in this annex collects the faults – either just the fault count or all the details – as well as the hardware and software versions of all Components in the vehicle.

For the sake of simplicity, the following assumptions are made:

- Each Component is able to provide the collected information upon request in a timely manner. Thus, timeouts due to long running request or asynchronous responses are not considered.

- In a multiple SOVD server environment the method collects the vehicle health from other SOVD servers.

    - Retrieval of VehicleHealth

Method:

GET /functions/VehicleHealth

Method description:

The Function gathers the health of a vehicle based on the SOVD API as defined in the introduction.

Path Parameters:

This method does not support path parameters.

Query Parameters:

Table 210	Query Parameters - Retrieval of VehicleHealth

| Parameter Name | Type | Convention | Description |
| --- | --- | --- | --- |
| include-schema | boolean | O | Specifies whether the response should include schema information or not. The default is false. |

| fault-details | boolean | O | Specifies whether the response should include details on the fault or just the number of faults. The default is false. |
| --- | --- | --- | --- |

Response Status Codes

Table 211	Response Status Codes – Retrieval of VehicleHealth

| Status Code | Response Body | Description |
| --- | --- | --- |
| 200 | Table 212 | The request was successful. |
| 502 | GenericError | The SOVD server was unable to retrieve vehicle health from any Component, but the error was not in the SOVD server itself. |

Response Body:

Table 212	Response Body - Retrieval of VehicleHealth

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| items | VehicleHealth [] | M | Array with the vehicle health of each Component. |
| schema | Schema | C | The schema definition of the response. Condition: Only provided if the query parameter include-schema is true. |

Table 213	VehicleHealth type

| Attribute | Type | Convention | Description |
| --- | --- | --- | --- |
| component | string:uri-reference | M | URI to the Component providing the information. |
| numberOfFaults | number | C | Number of faults stored for the Component. Only returned if fault-details is false. |
| faults | Fault[] | C | Faults as returned by GET /{entity-path}/faults without setting any of the query parameters. Only returned if fault-details is true. |
| swVersions | AnyValue | C | Value of the software version(s). The type AnyValue allows to store multiple version information, e. g. the version of bootloader, application, and |

|  |  |  | calibration data depending on the ECU. |
| --- | --- | --- | --- |
| hwVersions | AnyValue | C | Value of the hardware version(s). |
| errors | DataError[] | C | Error describing why a particular information of the vehicle health of a Component could not be retrieved. Only set if information could not be retrieved from a Component. |

If an error occurs, the errors attribute of the VehicleHealth is set. For each erroneous attribute, the field will contain an entry. The attribute path of an error uses a JSON pointer to reference the erroneous attribute with a more detailed error description. In case the path points to Component, the vehicle health for the Component could not be retrieved.

Appendix: B. Specific Variant Applicability

Informative

  - Introduction

The author can define which response or request schema is applicable for specific variants using the x-sovd-applicability extension. This is achieved by first using oneof to specify only one of a set of schemas would match. And in each schema, the x-sovd-applicability specifies the variant(s) for which the corresponding schema is applicable.

In the below example, /components/PowerSteering/data/SteeringMonitors resource is defined to retrieve some of the parameters of PowerSteering ECU. Based on the variant of PowerSteering mounted, two different response schemas are possible –VariantHighResponse and VariantLowResponse. The variant “Pow_Str_Variant_High” provides information of three data elements (VehicleSpeed, SteeringWheelRotationSpeed, SteeringWheelAngle) while the variant “Pow_Str_Variant_Low” provides information of only one data element (SteeringWheelAngle).

  - Example

"paths": { "/components/PowerSteering/data/SteeringMonitors": {

"get": {

"summary": "Gets the diagnostic information about power steering",

"responses": {

"200": {

"description": "description of successful response", "content": {

"application/json": { "schema": {

"$ref": "#/components/schemas/inline_response_200"

}

}

}

},

"400": {

"description": "bad input parameter"

}

}

}

},

"components": { "schemas": {

"VariantHighResponse": { "type": "object", "properties": {

"VechileSpeed": {

"$ref": "#/components/schemas/VehicleSpeed"

},

"SteeringWheelRotationSpeed": {

"$ref": "#/components/schemas/SteeringWheelRotationSpeed"

},

"SteeringWheelAngle": {

"$ref": "#/components/schemas/SteeringWheelAngle"

}

},

"description": "Response Schema Pow_Str_Variant_High", "x-sovd-applicability": {

"description": "Applicability Extension to specify which variant supports this schema",

"variant_id_list": { "type": "array", "items": [

"Pow_Str_Variant_High"

]

}

}

},

"VariantLowResponse": { "type": "object", "properties": {

"SteeringWheelAngle": {

"$ref": "#/components/schemas/SteeringWheelAngle"

}

},

"description": "Response Schema Pow_Str_Variant_Low", "x-sovd-applicability": {

"description": "Applicability Extension to specify which variant supports this schema",

"variant_id_list": { "type": "array", "items": [

"Pow_Str_Variant_Low"

]

}

}

},

"VehicleSpeed": { "maximum": 200,

"minimum": 0, "type": "integer", "example": 40

},

"SteeringWheelRotationSpeed": { "maximum": 30,

"minimum": 0, "type": "integer", "example": 15

},

"SteeringWheelAngle": { "maximum": 270,

"minimum": 0, "type": "integer", "example": 45

},

Figure Directory

Figure 1	WindowControl example	14

Figure 2	Entity types overview	15

Figure 3	Public SOVD server	17

Figure 4	Private SOVD server	17

Figure 5	Entity hierarchy	18

Figure 6	Example response structure	29

Figure 7:	SOVD API sub-tree related to SW updates	114

Figure 8	Resource organization based on categories	129

Figure 9:	Online authentication and authorization	150

Figure 10:	Offline authentication and authorization	151

Figure 11	Example hierarchy for “Root”	166

Figure 12	Resources of the App “AdvancedLaneKeeping”	167

Figure 13	Resources of the App “WindowControl”	168

Figure 14	Resources of the App “Navi”	168

Figure 15	Resources for the Component “Camera”	169

Figure 16	Resources and variants for the Component “PowerSteering”	169

Figure 17	UML sequence diagram for a data-list resource	170

Figure 18:	Topology for the “SteeringAngleControl” operation of “PowerSteering”171 Figure 19:	UML sequence diagram for the “SteeringAngleControl” operation	172

Figure 20	Function VehicleHealth	174

Table Directory

Table 1	HTTP methods used by SOVD	14

Table 2	Details of entity types	16

Table 3	Standardized resource collections	19

Table 4	Relation between entities and resource collections	20

Table 5	Standardized resource names	20

Table 6	Relation between entities and resources	21

Table 7	VersionInfo type	22

Table 8	SOVDInfo type	22

Table 9	VendorInfo type	22

Table 10	x-sovd-unit attributes	23

Table 11	PhysicalDimension type	23

Table 12:	Common SOVD server-side supported status codes on success	24

Table 13:	Common SOVD server-side supported status codes on SOVD client error	25

Table 14:	Common SOVD server-side supported status codes on SOVD server error	25

Table 15	GenericError type	25

Table 16	DataError type	26

Table 17	SOVD error codes for common cases	26

Table 18	SOVD primitive data types	27

Table 19	Mapping of ASAM data types	28

Table 20	Mapping of example response structure to JSON	29

Table 21	SOVD API required fields	32

Table 22	Extension Property Info	33

Table 23	Extension Property Path Item	33

Table 24	Extension Property Operation	34

Table 25	Extension Property Schema	34

Table 26	Timeout Definition Header	39

Table 27	Path Parameters - Discover contained entities	42

Table 28	Query Parameters - Discover contained entities	42

Table 29	Response Status Codes - Discover contained entities	42

Table 30	Response Body - Discover contained entities of entity collections	42

Table 31	EntityReference type	43

Table 32	Query Parameters - Query sub-entities of an entity	44

Table 33	Response Status Codes - Query sub-entities of an entity	44

Table 34	Response Body - Query sub-entities of an entity	44

Table 35	Response Status Codes - Query related entities of an entity	46

Table 36	Response Body - Query related entities of an entity	46

Table 37	Response Status Codes - Query of entity capabilities	47

Table 38	Response Body - Query of entity capabilities	47

Table 39	Query Parameters - Read faults from an entity	49

Table 40	Response Status Codes - Read faults from an entity	50

Table 41	Response Body - Read faults from an entity	50

Table 42	Fault type	51

Table 43	Path Parameters - Read details for a fault	54

Table 44	Query Parameters - Read details for a fault	54

Table 45	Response Status Codes - Read details for a fault	54

Table 46	Response Body - Read details for a fault	54

Table 47	Query Parameters - Delete all faults of an entity	56

Table 48	Response Status Codes - Delete all faults of an entity	56

Table 49	Path Parameters - Delete single fault of an entity	57

Table 50	Response Status Codes - Delete single fault of an entity	57

Table 51	DataCategory type	58

Table 52	Response Status Codes - Retrieve categories supported by a data resource collection	59

Table 53	Response Body - Retrieve categories supported by a data resource collection	59

Table 54	Response Status Codes - Retrieve groups supported by a data resource collection	60

Table 55	Response Body - Retrieve groups supported by a data resource collection	60

Table 56	ValueGroup type	60

Table 57	Query Parameters - Retrieve list of all data provided by the entity	61

Table 58	Response Status Codes - Retrieve list of all data provided by the entity

.	62

Table 59	Response Body - Retrieve list of all data provided by the entity	62

Table 60	ValueMetaData type	62

Table 61	Path Parameters - Read single data value from an entity	64

Table 62	Query Parameters - Read single data value from an entity	64

Table 63	Response Status Codes - Read single data value from an entity	64

Table 64	ReadValue type	64

Table 65	Response Status Codes - Retrieve list of all data-lists provided by the entity	67

Table 66	Response Body - Retrieve list of all data-lists provided by the entity	67

Table 67	DataListEntry type	67

Table 68	Request Body - Creating a data list for reading multiple data values at once from an entity	69

Table 69	Response Status Codes - Creating a data list for reading multiple data values at once from an entity	69

Table 70	Response Body - Creating a data list for reading multiple data values

at once from an entity	69

Table 71	Path Parameters - Read multiple data values at once from an entity using a data list	70

Table 72	Query Parameters - Read multiple data values at once from an entity using a data list	70

Table 73	Response Status Codes - Read multiple data values at once from an entity using a data list	71

Table 74	Response Body - Read multiple data values at once from an entity using a data list	71

Table 75	Path Parameters - Delete an existing data list	72

Table 76	Response Status Codes - Delete an existing data list	72

Table 77	Path Parameters - Write a data value data to an entity	73

Table 78	Request Body - Write a data value data to an entity	73

Table 79	Response Status Codes - Write a data value data to an entity	74

Table 80	Query Parameters - Retrieve list of all configurations provided by the entity	75

Table 81	Response Status Codes - Retrieve list of all configurations provided by the entity	75

Table 82	Response Body - Retrieve list of all configurations provided by the entity	75

Table 83	ConfigurationMetaData type	76

Table 84	ConfigurationType	76

Table 85	Path Parameters - Read configuration	77

Table 86	Response Status Codes - Read Configuration as Bulk Data	78

Table 87	Query Parameters - Read configuration as parameters	78

Table 88	Response Status Codes - Read configuration as parameters	79

Table 89	Path Parameters - Write configuration	81

Table 90	Response Status Codes - Write Configuration as Bulk Data	81

Table 91	Request Body - Write configuration as parameters	82

Table 92	Response Status Codes - Write configuration as parameters	83

Table 93	Query Parameters - Retrieve list of all available operations from an entity	84

Table 94	Response Status Codes - Retrieve list of all available operations from an entity	85

Table 95	Response Body - Retrieve list of all available operations from an entity

.	85

Table 96	OperationDescription type	85

Table 97	Path Parameters - Get details of a single operation	86

Table 98	Query Parameters - Get details of a single operation	86

Table 99	Response Status Codes - Get details of single operation	87

Table 100	Response Body - Get details of single operation	87

Table 101	ProximityChallenge type	87

Table 102	Operation capabilities relations to SOVD methods	88

Table 103	Path Parameter - Start execution of an operation	89

Table 104	Request Body - Start execution of an operation	90

Table 105	Response Status Codes - Start execution of an operation	90

Table 106	Response Body - Start execution of an operation - synchronous execution	91

Table 107	Response Body - Start execution of an operation - asynchronous execution	91

Table 108	ExecutionStatus type	91

Table 109	Path Parameters - Get executions of an operation	92

Table 110	Response Status Codes - Get executions of an operation	92

Table 111	Response Body - Get executions of an operation	92

Table 112	Path Parameters - Get the status of an operation execution	93

Table 113	Query Parameters - Get the status of an operation execution	93

Table 114	Response Status Codes - Get the status of an operation execution	94

Table 115	Response Body - Get the status of an operation execution	94

Table 116	Path Parameters - Stop the execution of an operation	95

Table 117	Request Body - Stop the execution of an operation	96

Table 118	Response Status Codes - Stop the execution of an operation	96

Table 119	Path Parameters - Support for execute / freeze / reset and OEM-specific capabilities	96

Table 120	Request Body - Support for execute / freeze / reset and OEM-specific capabilities	97

Table 121	Response Status Codes - Support for execute / freeze / reset and OEM-specific capabilities	97

Table 122	Query Parameters - Retrieve list of all supported modes of an entity	99

Table 123	Response Status Codes - Retrieve list of all supported modes of an entity	99

Table 124	Response Body - Retrieve list of all supported modes of an entity	99

Table 125	ModeCollectionItem type	100

Table 126	Path Parameters - Get details of a single mode of an entity	100

Table 127	Query Parameters - Get details of a single mode of an entity	101

Table 128	Response Status Codes - Get details of a single mode of an entity	101

Table 129	Response Body - Get details of a single mode of an entity	101

Table 130	Path Parameters - Explicit control of entity states via their defined modes	102

Table 131	Request Body - Explicit control of entity states via their defined modes

.	103

Table 132	Response Status Codes - Explicit control of entity states via their defined modes	103

Table 133	Response Body - Explicit control of entity states via their defined modes	103

Table 134	x-sovd-lock-required attribute	105

Table 135	Request Body - Acquire a lock on an entity	106

Table 136	Response Status Codes - Acquire a lock on an entity	106

Table 137	Response Body - Acquire a lock on an entity	106

Table 138	Response Status Codes - Get all acquired locks of an entity	107

Table 139	Response Body - Get all acquired locks of an entity	107

Table 140	LockInfo type	108

Table 141	Path Parameters - Get a single active lock of an entity	108

Table 142	Response Status Codes - Get a single active lock of an entity	109

Table 143	Response Body - Get a single active lock of an entity	109

Table 144	Path Parameters - Modify the expiration time of an acquired lock on an entity	110

Table 145	Request Body - Modify the expiration time of an acquired lock on an entity	110

Table 146	Response Status Codes - Modify the expiration time of an acquired lock on an entity	110

Table 147	Path Parameters - Release an acquired lock on an entity	111

Table 148	Response Status Codes - Release an acquired lock on an entity	111

Table 149	UpdateOrigins type	112

Table 150	Query Parameters - Retrieve list of all updates provided by the entity 115 Table 151	Response Status Codes - Retrieve list of all updates provided by the

entity	115

Table 152	Response Body - Retrieve list of all updates provided by the entity	115

Table 153	Path Parameters - Get details of update	116

Table 154	Query Parameters - Get details of update	116

Table 155	Response Status Codes - Get details of update	117

Table 156	Response Body - Get details of update	117

Table 157	Path Parameters - Automated installation of an update	119

Table 158	Response Status Codes - Automated installation of an update	119

Table 159	Path Parameters - Prepare installation of an update	121

Table 160	Response Status Codes - Prepare installation of an update	121

Table 161	Path Parameters - Execute installation of an update	122

Table 162	Response Status Codes - Execute installation of an update	123

Table 163	Path Parameters - Get status of an update	123

Table 164	Response Status Codes - Get status of an update	124

Table 165	Response Body - Get status of an update	124

Table 166	Phase type	124

Table 167	Progress type	124

Table 168	Status types	125

Table 169	Path Parameters - Delete update package from an SOVD server	126

Table 170	Response Status Codes - Delete update package from an SOVD server	126

Table 171	Response Status Codes - Register an update at the SOVD server	127

Table 172	Response Body - Register an update at the SOVD server	128

Table 173	Query Parameters - Retrieve list of all bulk data categories	130

Table 174	Response Status Codes - Retrieve list of all bulk data categories	130

Table 175	Response Body - Retrieve list of all bulk data categories	130

Table 176	Examples for BulkDataCategory type	131

Table 177	Path Parameters - Read bulk data meta data	131

Table 178	Query Parameters - Read bulk data meta data	132

Table 179	Response Status Codes - Read bulk data meta data	132

Table 180	Response Body - Read bulk data meta data	132

Table 181	BulkDataDescriptor type	132

Table 182	Path Parameters - Download bulk data	134

Table 183	Response Status Codes - Download bulk data	134

Table 184	Path Parameters - Upload bulk data	136

Table 185	Response Status Codes - Upload bulk data	136

Table 186	Response Body - Upload bulk data	137

Table 187	Path Parameters - Delete all bulk data defined by category	138

Table 188	Response Status Codes - Delete all bulk data defined by category	138

Table 189	Response Body - Delete all bulk data defined by category	138

Table 190	DeletionError type	138

Table 191	Path Parameters - Delete bulk data	139

Table 192	Response Status Codes - Delete bulk data	140

Table 193	Query Parameters - Retrieve list of all log information	141

Table 194	Severity type in relation to ContextType	141

Table 195	Response Status Codes - Retrieve list of all log information	141

Table 196	Response Body - Retrieve list of all log information	142

Table 197	LogEntry type	142

Table 198	Context attributes	142

Table 199	ContextType dependent attributes	143

Table 200	Request Body - Configure SOVD logging	144

Table 201	LogConfiguration type	144

Table 202	Response Status Codes - Configure SOVD logging	145

Table 203	Response Status Codes - Retrieve the current SOVD logging configuration	146

Table 204	Response Body - Retrieve the current SOVD logging configuration	146

Table 205	Response Status Codes - Reset SOVD logging configuration to default

.	147

Table 206	Response Status Codes - Verifying SOVD client credentials at the vehicle	153

Table 207	Response Status Codes - Requesting a token	154

Table 208	Response Status Codes - Request header for access-restricted resources	155

Table 209	Overview on how to map UDS based contents to SOVD	156

Table 210	Query Parameters - Retrieval of VehicleHealth	174

Table 211	Response Status Codes – Retrieval of VehicleHealth	175

Table 212	Response Body - Retrieval of VehicleHealth	175

Table 213	VehicleHealth type	175

Email:	support@asam.net Web:	www.asam.net

© by ASAM e.V., 2022